3GPP TSG-RAN WG3 Meeting #129-bis	R3-257184

Prague, Czech Republic, 13th - 17th Oct, 2025

Title: 	Discussion on the IETF Network Slice Application

Source: 	Huawei

Agenda item:	8.1

Document Type:	Discussion

# 1. Introduction

The IETF TEAS WG sent a LS on IETF Network Slice Application in 3GPP 5G End-to-End Network Slice” as follows [1].

- Description As a follow-up to the liaison statement [1] that was posted on 2023-10-09, the IETF TEAS WG [2] would like to inform 3GPP about the progress of "IETF Network Slice Application in 3GPP 5G End-to-End Network Slice" [3], an informational document that is almost ready for Working Group Last Call. This document describes the application of the IETF network slice framework in providing 5G end-to-end network slices, including network slice mapping in the management, control, and data planes. The following is the mailing list where discussions related to these documents take place [4]. 2. Actions The TEAS WG would like to request 3GPP to review the document and verify if the description of the 3GPP 5G End-to-End Network Slice is accurate, and if not, to kindly provide suggested clarifications. The TEAS WG would like to request that feedback on this be provided in an LS by the deadline. The TEAS WG also encourages the use of the TEAS WG mailing list for individuals to send comments, raise concerns, and seek clarification on the document.

This contribution provides our views towards the informational document contained in the incoming LS.

# 2. Discussion

## 2.1 Past LSes related to IETF TEAS

The LS in [1] is a subsequent LS from IETF on the framework for network slices, which was discussed at the RAN3-122 meeting, and simply noted as follows.

| - R3-235754 | - LS on a Framework for Network Slices in Networks Built from IETF Technologies Submission (IETF_LS_231009; to: RAN3, SA2, SA3, SA5; cc: -; contact: Cisco) | - LS in noted |
| --- | --- | --- |

In the previous RAN3-125 meeting, the IETF TEAS sent a related LS on the Realization of Network Slices for 5G Networks Using Current IP/MPLS Technologies. RAN3 agreed the LS in R3-244785 as follows.

| R3-244501 | Discussion on the Realization of Network Slices for 5G Networks Using Current IP/MPLS Technologies (Huawei) | Discussion E///: Would like to go for a more complete reply on transport layer, there has the risk to have two different architectures in 3GPP and IETF Nok: The main point is to remove RU, BBU split, it seems fine that they show their own figure # 1_ReplyLStoIETF - Work on the reply LS, taking R3-244502 and R3-244614 into account (moderator - HW) |
| --- | --- | --- |
| R3-244502 | [draft] Reply LS on the Realization of Network Slices for 5G Networks Using Current IP/MPLS Technologies (Huawei) | LS out To: IETF TEAS CC: SA2, SA3, SA5 Rev in R3-244660 - Remove WI code and release Rev in R3-244785 Agreed unseen |

In the reply LS R3-244785, RAN3 clearly indicated several aspects requesting IETF TEAS consideration. Especially:

- RAN3 would like to point out that RAN3 is not responsible for the design and specification of any aspect of IP/MPLS transport networks.  However, RAN3´s Terms of Reference includes requirements concerning transport network layer´s protocols to be used for communication over 3GPP defined interfaces terminating at the NG-RAN.

- To reflect the 5G NG-RAN architecture agreed in 3GPP, RAN3 kindly suggests IETF TEAS references in their document to TS 38.300, describing the NG-RAN architecture, and TS 38.401, describing the split NG-RAN architecture, to avoid possible misalignments.

- Observation 1: During the previous LS with IETF, RAN3 already indicated the following aspects requesting IETF TEAS consideration. Especially:

- RAN3 would like to point out that RAN3 is not responsible for the design and specification of any aspect of IP/MPLS transport networks.  However, RAN3´s Terms of Reference includes requirements concerning transport network layer´s protocols to be used for communication over 3GPP defined interfaces terminating at the NG-RAN.

- To reflect the 5G NG-RAN architecture agreed in 3GPP, RAN3 suggests IETF TEAS references in their document to TS 38.300, describing the NG-RAN architecture, and TS 38.401, describing the split NG-RAN architecture, to avoid possible misalignments.

## 2.2 Document analysis

- After reviewing the informational document, we found that the above aspects mentioned in the previous reply LS is still worthwhile for IETF TEAS consideration. It is suggested to reply to IETF that the aspects above could be considered when necessary.

RAN3 to reply that the points mentioned in R3-244785 (e.g., referring to the 3GPP specifications on 5G NG-RAN architecture) are worthwhile to be considered when necessary.

- In the document, the section 3 provides the 5G End-to-End Network Slice. Two pictures are copied below, specifically,

- In section 3.2.  IETF Network Slices in Centralized RAN Deployment, in Centralized RAN, network functions NFs1 and NFs2 are separated by a transport network TN3 called fronthaul network (FH).

- In section 3.3 IETF Network Slices in Cloud RAN deployment (C-RAN), not only the TN3 (FH) but also the Midhaul network (MH) between the CU (Central Unit) and DU (Distributed Unit) are provided.

- Also, in section 3.3 IETF Network Slices in Cloud RAN deployment (C-RAN), it is described that in the document, TN slice between RAN and CN is considered, while other IETF network slice cases are similar.

- Figure 2: IETF network slices in distributed RAN deployment - Figure 3: IETF network slices in centralized RAN deployment

- Clearly, there are no FH, and MH terminologies in 3GPP specifications. Especially for the FH, the detailed handlings are totally up to the implementation. In the document, multiple methods for Mapping Between 3GPP E2E Network Slice and IETF are provided:

- Mapping based on VLAN ID  .

- Mapping based on MPLS Label or SR-MPLS SID

- Mapping based on SRv6 SID . .

- Mapping based on Policy Based Routing (PBR)

- Mapping based on UDP Source Port

- For the INS3 used for the (TN3) FH, it is an open question whether and how the above mapping methods could be applied between IETF network slice and 3GPP E2E Network Slice works. However, this is totally up to IEFT consideration.

RAN3 to reply that the FH, and MH are not 3GPP defined terminologies. And for the IETF network slices used for FH, it is up to IETF to consider the methods for mapping between 3GPP E2E Network Slice and these IETF network slices.

- In the document, there are many places where the O-RAN and 3GPP are couple used. First, the O-RAN system is totally agnostic in 3GPP system. Second, any ambiguities should be avoided when IEFT refers the RAN functions defined in 3GPP. Typically, in section 7 IETF Network Slice request through IETF Network Slice NBI, as shown below in Figure 20 and Figure 24, the F1-U interface is described according to O-RAN. Though the 3GPP TS 38.470 is correctly referred, it should be corrected to refer to 3GPP specifications instead.

- Figure 20: CE-mode slice realization example between DU and CU-UP OPTION - Figure 24: PE-mode slice realization OPTION 2

RAN3 to reply to IETF TEAS that in Figure 20 and Figure 24, the DU/CU-UP as well as the F1-U interface are defined in 3GPP specification, as indicated in the TS 38.401.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

- Observation 1: During the previous LS with IETF, RAN3 already indicated the following aspects requesting IETF TEAS consideration. Especially:

- RAN3 would like to point out that RAN3 is not responsible for the design and specification of any aspect of IP/MPLS transport networks.  However, RAN3´s Terms of Reference includes requirements concerning transport network layer´s protocols to be used for communication over 3GPP defined interfaces terminating at the NG-RAN.

- To reflect the 5G NG-RAN architecture agreed in 3GPP, RAN3 suggests IETF TEAS references in their document to TS 38.300, describing the NG-RAN architecture, and TS 38.401, describing the split NG-RAN architecture, to avoid possible misalignments.

- RAN3 to reply that the points mentioned in R3-244785 (e.g., referring to the 3GPP specifications on 5G NG-RAN architecture) are worthwhile to be considered when necessary.

RAN3 to reply that the FH, and MH are not 3GPP defined terminologies. And for the IETF network slices used for FH, it is up to IETF to consider the methods for mapping between 3GPP E2E Network Slice and these IETF network slices.

RAN3 to reply to IETF TEAS that in Figure 20 and Figure 24, the DU/CU-UP as well as the F1-U interface are defined in 3GPP specification, as indicated in the TS 38.401.

- The reply LS can be found in [2].

# 4. Reference

- R3-256526	LS on “IETF Network Slice Application in 3GPP 5G End-to-End Network Slice”	, IETF teas

- R3-257185	[draft] Reply LS on “IETF Network Slice Application in 3GPP 5G End-to-End Network Slice”, Huawei