3GPP TSG-RAN WG3 Meeting #129-bis	R3-257193

Prague, CZ, Oct 13-17, 2025

Agenda Item:	10.4

Source:	THALES, Echostar

Title:	6G RAN internal functional split and interfaces for NTN

Document for:	Discussion

# 1	Introduction

The 6G Radio Study Item was approved at the RAN#108 meeting and updated at RAN#109 [1]. It states that internal function split is considered for RAN architecture.

(6) Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] a) Overall RAN architecture aspects [RAN3, RAN2] b) RAN-CN functional split, interface, protocol stack and procedures [RAN3] c) RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

6GR also aims to harmonize TN and NTN radio interface :

j) Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

In this contribution, we share our view for supporting a flexible internal functional split for NTN.

# 2	Discussion

The split architecture is described for 5G in [7]. In this architecture, the gNB distributed unit (DU) provides the radio access with low layers, and the gNB centralized unit concentrates the higher layer and the connectivity with the core. A gNB-DU can be connected to one gNB-CU but multiple gNB-DUs can be handled by a gNB-CU. This architecture offers a certain flexibility for deployment in remote areas in terrestrial networks. The F1 interface is used as a transport network layer between gNB-CU and gNB-DU for both the user plane and control plane. Several options are specified as illustrated below, with some that get more interest than other according to latency requirements and other gains.

Figure 1 - functional split options

The support of the split architecture for NTN was studied in Rel-16 [5], the split option 2 (F1 interface between RLC and PDCP layers) was considered as a baseline. The split architecture was not specified in the stage 2 specifications alongside the transparent architecture (introduced in Rel-17) and regenerative architecture with full gNB embarked (introduced in Rel-19). However, it can be supported by implementation with the appropriate adaptations to handle long delays on the feeder link, e.g. the timers for RRC and L3 and buffering on the F1 user-plane interface.

For 6G, supporting base station split architecture for NTN offers several significant benefits and deployment options. Some deployment scenarios are in [6]. The scenario (3) shows some benefits for NTN in limiting the computation and memory footprint on the satellite while limiting the feeder link throughput and latency constraints.

|  | Scenario On the satellite On the ground |  | Benefits | Constraints |
| --- | --- | --- | --- | --- |
| (3) | RU+DU | CU + Core network | Limited computation and memory footprint on the satellite while limiting the feeder link throughput and latency constraints | High mobility between DU and CU and inter-satellite handover relies on the ground segment |

Reducing network function footprint on satellite payload computation and memory is particularly critical for deployments. For instance, for each UE and for each data radio bearer (signalling radio bearer for control plane), an entity at each protocol layer is instantiated. For the access node (i.e. gNB in 5G), the number of instantiated entities can be quite huge with a cost in terms of memory and computation. For a satellite where the gNB is fully embarked, the satellite payload and the related gNB(s) can handle hundreds of cells (illustrated below with a LEO-600 view field with 1058 beams that can be generated), with terminals in each cell, inducing a large memory and computational footprint on the satellite payload.

Figure 1 : View field of a LEO-600 satellite with 1058 beams

The satellite constrained SWaP (Size, Weight and Power) has to be used at its best for the user service and, in particular, on the service availability and throughput.

By leveraging the functional split between, NTN deployments can achieve greater flexibility and efficiency in network design and resource allocation. This separation allows part of the baseband processing to be centralized on ground stations or cloud infrastructure while distributing real-time radio functions closer to the satellite or airborne platform. As a result, it reduces latency, optimizes bandwidth usage over satellite links, and enhances scalability by enabling dynamic allocation of processing resources depending on the traffic demand. Moreover, the split architecture facilitates easier upgrades and maintenance by isolating components, thus improving the overall operational resilience and adaptability of NTN systems to varying mission requirements and environments.

Proposal	The 6G RAN architecture should support various base station internal functional splits able to accommodate the NTN characteristics and allow the selection of the minimum set of RAN functions to be embarked on board for the mission purpose.

# 3	Conclusion

Proposal	The 6G RAN architecture should support various base station internal functional splits able to accommodate the NTN characteristics and allow the selection of the minimum set of RAN functions to be embarked on board for the mission purpose.

# References

RP-252912, “Revised SID: Study on 6G Radio”, NTT Docomo, CMCC, AT&T, Vodafone, Beijing, CN, September 2025.

RP-250810, “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT Docomo, Deutsche Telekom, Incheon, KR, March 2025.

3GPP TR 38.914, “	Study on 6G Scenarios and requirements”, v0.1.1, Beijing, CN, September 2025.

6GWS-250040, “Vision and priorities for 6G RAT to support NTN”, Thales, Incheon, KR, March 2025.

3GPP TR 38.821, “Solutions for NR to support non-terrestrial networks (NTN)”, Rel-16, May 2021.

R3-257192, “6G RAN overall architecture to support NTN”, Prague, CZ, October 2025.

3GPP TS 38.401, “5G; NG-RAN; Architecture description”, Rel-18, May 2025.