3GPP TSG-RAN WG3 Meeting #130	R3-258038

# Dallas, USA, 17 – 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1560 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of Xn-related issues for WAB-gNB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Ericsson, Nokia, Nokia Shanghai Bell, CATT, Qualcomm, Canon, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_WAB_5GFemto-Core |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In TS 38.401, it is specified that the establishment of Xn connections between two WAB-gNBs can be avoided by WAB-gNB reject the Xn setup initiated by another WAB-gNB. However, this is missing in the stage 3 specification. It is suggested to capture the Xn setup failure case in clause 8.4.1.3. TS 38.401: Establishment of Xn connections between two WAB-gNBs can be avoided. To achieve this, the WAB-gNB may reject the Xn setup initiated by another WAB-gNB, e.g., based on the presence of the WAB-MT ID received in the XN SETUP REQUEST message.. - In clause 8.4.1.2 and 8.4.2.2, the second occurrence of the NG-RAN node is actually a WAB-gNB, clarify that the NG-RAN node is a WAB-gNB. |
| Summary of change: |  | - In clause 8.4.1.3, capture the Xn setup failure case for WAB-gNBs. - In clause 8.4.1.2 and 8.4.2.2, clarify the NG-RAN node is a WAB-gNB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Xn setup failure case for WAB-gNBs is missing in the specification. Specification is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1.2, 8.4.1.3, 8.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 0: R3-256951 Rev 1: Revision of R3-256951 by removing some changes not needed and merging the first and third change in R3-257191 with rewording. Rev 2: Revision of R3-257260, change “WAB-gNB” to “WAB-node” in clauses 8.4.1.2 and 8.4.2.2, and update coversheet. Rev 3: Revision of R3-257306, resubmit to RAN3#130. |  |  |  |  |  |  |  |  |

----------------Start of Change ---------------

#### 8.4.1.2	Successful Operation

Figure 8.4.1.2: Xn Setup, successful operation

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

If the WAB-MT Identifier IE is included in the XN SETUP REQUEST message or in the XN SETUP RESPONSE message, the receiving NG-RAN node shall, if supported, consider that the transmitting NG-RAN node is a WAB-node. If the receiving NG-RAN node serves the WAB-MT identified by the WAB-MT Identifier IE, the receiving NG-RAN node, shall, if supported, conclude that the WAB-MT is co-located with the WAB-gNB functionality of the transmitting NG-RAN node.

----------------Next Change---------------

8.4.1.3	Unsuccessful Operation

Figure 8.4.1.3-1: Xn Setup, unsuccessful operation

If the candidate NG-RAN node2 cannot accept the setup it shall respond with the XN SETUP FAILURE message with appropriate cause value.

If the XN SETUP FAILURE message includes the Time To Wait IE, the initiating NG-RAN node1 shall wait at least for the indicated time before reinitiating the Xn Setup procedure towards the same NG-RAN node2.

If case of network sharing with multiple Cell ID broadcast with shared Xn-C signalling transport, as specified in TS 38.300 [9], the XN SETUP REQUEST message and the XN SETUP REQUEST FAILURE message shall include the Interface Instance Indication IE to identify the corresponding interface instance.

If the Message Oversize Notification IE is included in the XN SETUP FAILURE, the initiating node shall, if supported, deduce that the failure is due to a too large XN SETUP REQUEST message and ensure that the total number of served cells in following XN SETUP REQUEST message is equal to or lower than the value of the Maximum Cell List Size IE.

If the WAB-MT Identifier IE is included in the XN SETUP REQUEST message, and if the NG-RAN node2 determines that it should avoid Xn establishment with a WAB-gNB (based on, e.g. configuration or implementation), the NG-RAN node2 shall reject the Xn setup and send the XN SETUP FAILURE message to the NG-RAN node1 with an appropriate cause value.

----------------Next Change---------------

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: NG-RAN node Configuration Update, successful operation

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

If the WAB-MT Identifier IE is included in the NG-RAN NODE CONFIGURATION UPDATE message or in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message, the receiving NG-RAN node shall, if supported, consider that the transmitting NG-RAN node is a WAB-node. If the receiving NG-RAN node serves the WAB-MT identified by the WAB-MT Identifier IE, the receiving NG-RAN node, shall, if supported, conclude that the WAB-MT is co-located with the WAB-gNB functionality of the transmitting NG-RAN node.

-----------------End of Changes ---------------