3GPP TSG-RAN WG3 Meeting #130	R3-258142

Dallas, USA, 17-21, November, 2025

Agenda Item:	9.2.11

Source:	ZTE Corporation, Ericsson, Xiaomi, CATT

Title:		Discussion on the reply LS on AI/ML assisted positioning

Document for:	Discussions & Approval

- Introduction

SA2 replied to the LS [1] from RAN3 on AI/ML positioning case 3a and informed RAN3 that the procedure for data collection for NG-RAN-sided models for UEs under positioning has been captured in TS 23.273 [2]. However, we identified an discrepancy between SA2 and RAN3 regarding user consent. Here, we provide the discussion and corresponding CR [3] to correct this issue.

- Discussion

As described in the LS [1], the procedure for data collection for NG-RAN-sided models for UEs undergoing positioning has been captured in Section 6.23 of TS 23.273 [2].

Precondition: There is an ongoing positioning session for the UE/PRU, and the network assisted or network based positioning is used.

1.	The procedure in Figure 6.11.2-1 can be used for data collection by the NG RAN for input data to support NG-RAN node assisted positioning with gNB-side model.

The steps 1-6 of network assisted positioning procedure are used as defined in clause 6.11.2 and the information to indicate that data collection is needed for the UE being positioned is included in the network positioning message in steps 5-6 as defined in NRPPa signalling in TS 38.455 [15].

2.	To provide the data for the UE being positioned, before LMF provides data to the NG-RAN, the LMF performs the user consent check by interaction with UDM as step 4 of data collection to train models for LMF-based AI/ML positioning based on NG RAN measurements as defined in clause 6.22.3. If user consent is not granted, LMF sends to the NG-RAN an appropriate indication as defined in TS 38.455 [15].

3-4.	The LMF determines the data requested by NG-RAN based on the ground truth data of the UE. The LMF transfers a Network Positioning message to the NG-RAN node the requested data as defined in TS 38.455 [15]. If the LMF is unable to provide the positioning information, LMF sends to the NG-RAN an appropriate indication as defined in TS 38.455 [15].

According to step 2, the LMF performs the user consent check before it provides the requested data to the NG-RAN node. If user consent is not granted, the LMF sends an appropriate indication to the NG-RAN as defined in TS 38.455. Therefore, SA2 clearly indicates that a lack of user consent is one reason the LMF may be unable to provide the requested data to the NG-RAN. Furthermore, in a subsequent step, if the LMF is unable to provide the positioning information, the LMF sends an appropriate indication to the NG-RAN as defined in TS 38.455. In this case, a specific reason is not explicitly mentioned.

- The lack of user consent is one reason why the LMF may be unable to provide the requested data to the NG-RAN, which is clearly specified in the SA2 specification.

Continuing our review of the current specified NRPPa specification, the Positioning Data Unavailable IE within the POSITIONING DATA COLLECTION REPORT message only indicates 'not supported' or 'not available'. This does not clearly specify that the inability to provide the data is due to user consent.

9.1.5.1	POSITIONING DATA COLLECTION REPORT

This message is sent to report positioning information for data collection.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| Positioning Data Information | O |  | 9.2.107 |  | YES | ignore |
| Positioning Data Unavailable | O |  | ENUMERATED(not supported, not available, …) |  | YES | ignore |

- Current Positioning Data Unavailable IE can not indicate the inability to provide the data is due to user consent.

Given the above observations, we believe it is necessary to extend the Positioning Data Unavailable IE by introducing a new value in the IE enumeration: "no user consent".

Introduce the new codepoint "no user consent" in the Positioning Data Unavailable IE.

- Conclusion

Following is our observations and proposals:

- The lack of user consent is one reason why the LMF may be unable to provide the requested data to the NG-RAN, which is clearly specified in the SA2 specification.

- Current Positioning Data Unavailable IE can not indicate the inability to provide the data is due to user consent.

- Introduce the new codepoint "no user consent" in the Positioning Data Unavailable IE.

- Reference

[1] R3-258004, Reply LS on AI/ML Positioning Case 3a, SA2(CATT)

[2] TS23.273

[3] R3-258143, Correction on Positioning Data Unavailable IE, ZTE Corporation