3GPP TSG-RAN WG3 Meeting #130	R3-258155

Dallas, US, 17 - 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1606 | Rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the CSI-RS Coordination procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, LG Electronics, ZTE, Offinno, NTT Docomo, Jio Platforms, CMCC, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are some redundant IEs in the CSI-RS Coordination procedure. The CSI-RS Resources for L1 measurements and for CSI acquisition are indicated separately. This is not needed from function perspective as the CSI resource configuration indicated by the CSI resource configuration ID can already distinguish the purpose or the CSI-RS resources. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are made to the XnAP for inter-CU LTM. - Remove the redundant IEs existing in the CSI-RS Coordination procedure (align with F1AP) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignments exit between XnAP and F1AP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.7, 9.1.5.8, 9.3.4, 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

#### 9.1.5.7	CSI-RS COORDINATION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to coordinate the activation and deactivation of CSI-RS transmission for a UE at NG-RAN node2.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 UE XnAP ID | M |  | NG-RAN node1 UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node. | YES | reject |
| NG-RAN node2 UE XnAP ID | M |  | NG-RAN node2 UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | YES | reject |
| CSI-RS Coordination Request List |  | 1 |  |  | YES | reject |
| >CSI-RS Coordination Request Item |  | 1 .. < maxnoofLTMCSResourceConfig > |  |  | – |  |
| >>Transmission Request | M |  | ENUMERATED(activate, deactivate, …) |  | – |  |
| >>CSI Resource Configuration ID | M |  | INTEGER (0..111, …) | Corresponds to information provided in the LTM-CSI-ResourceConfigId as defined in TS 38.331 [10]. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCSIResourceConfigurations | Maximum number of CSI Resource Configurations . Value is 112. |

<< Next Change >>

#### 9.1.5.8	CSI-RS COORDINATION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to coordinate the activation and deactivation of CSI-RS transmission for a UE at NG-RAN node2.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 UE XnAP ID | M |  | NG-RAN node1 UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node. | YES | reject |
| NG-RAN node2 UE XnAP ID | M |  | NG-RAN node2 UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | YES | reject |
| CSI-RS Coordination Result List |  | 1 |  |  | YES | reject |
| > CSI-RS Coordination Result Item |  | 1 .. <maxnoofCSIResourceConfigurations> |  |  | – |  |
| >>Transmission Status | M |  | ENUMERATED(activated, deactivated, …) |  | – |  |
| >>CSI Resource Configuration ID | M |  | INTEGER (0..111, …) | Corresponds to information provided in the CSI-ResourceConfigId as defined in TS 38.331 [10]. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCSIResourceConfigurations | Maximum number of CSI Resource Configurations . Value is 112. |

<< Next Change >>

-- **************************************************************

--

#### -- CSI-RS COORDINATION REQUEST

--

-- **************************************************************

CSIRSCoordinationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{CSIRSCoordinationRequest-IEs}},

...

}

CSIRSCoordinationRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NGRAN-Node1-ID				CRITICALITY reject	TYPE NG-RANnodeUEXnAPID						PRESENCE mandatory}|

{ ID id-NGRAN-Node2-ID				CRITICALITY reject	TYPE NG-RANnodeUEXnAPID						PRESENCE mandatory}|

{ ID id-CSI-RSCoordinationRequest	CRITICALITY reject	TYPE CSI-RSCoordinationRequestList				PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- CSI-RS COORDINATION RESPONSE

--

-- **************************************************************

CSIRSCoordinationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{CSIRSCoordinationResponse-IEs}},

...

}

CSIRSCoordinationResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NGRAN-Node1-ID				CRITICALITY reject	TYPE NG-RANnodeUEXnAPID						PRESENCE mandatory}|

{ ID id-NGRAN-Node2-ID				CRITICALITY reject	TYPE NG-RANnodeUEXnAPID						PRESENCE mandatory}|

{ ID id-CSI-RSCoordinationResponse	CRITICALITY reject	TYPE CSI-RSCoordinationResultList				PRESENCE mandatory},

...

}

<< Next Change >>

CSI-RSCoordinationRequestList ::= SEQUENCE (SIZE(1.. maxnoofLTMCSI-RSResourceConfig)) OF CSI-RSCoordinationRequest-Item

CSI-RSCoordinationRequest-Item	::= SEQUENCE {

transmissionRequest					ENUMERATED{activate, deactivate},

cSIResourceConfigurationID			INTEGER (0..111),

iE-Extensions						ProtocolExtensionContainer { { CSI-RSCoordinationRequest-Item-ExtIEs} }	OPTIONAL,

...

}

CSI-RSCoordinationRequest-Item-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

CSI-RSCoordinationResultList ::= SEQUENCE (SIZE(1.. maxnoofLTMCSI-RSResourceConfig)) OF CSI-RSCoordinationResult-Item

CSI-RSCoordinationResult-Item	::= SEQUENCE {

transmissionStatus					ENUMERATED{activated, deactivated},

cSIResourceConfigurationID			INTEGER (0..111),

iE-Extensions						ProtocolExtensionContainer { { CSI-RSCoordinationResult-Item-ExtIEs} }	OPTIONAL,

...

}

CSI-RSCoordinationResult-Item-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

<< Next Change >>

<< End of Changes >>