3GPP TSG-RAN3 Meeting #130	R3-258161

Dallas, USA, 17 – 21 Nov 2025

Agenda Item: 	10.4

Source: 	Qualcomm Incorporated

Title: 	[pCR to TR 38.760-3] 6G RAN internal functional split and interfaces

Document for:	pCR

- Background

Interim results shall be delivered as per the milestones below, in coordination with the Rel-20 RAN Plenary 6G Study (FS_6G_RAN_Scen_Req).

TSG#112 (June/2026):

RAN3 to provide interim study results to allow TSGs to make a decision on:

- RAN-CN interface: P2P vs SBI

- RAN internal interfaces: CU-DU split, CP-UP split.

NOTE: It is planned to decide on Release-21 timeline in June/2026.

6G RAN WG SID, RP-252912, [1], has the below guidelines on Interim study results, which indicates that RAN3 has to study CU-DU split and CP-UP split of the RAN internal functional split and provide study results by June 2026 for RAN-P to make a decision.

Additionally, in RAN3#129bis meeting, RAN3 chair notes captured the below

Capture pain points and benefits (and potentially derived requirements) of 5G HLS

In this paper, we discuss our views on the benefits and concerns of 5G HLS for both CU-DU split and CU-CP, CU-UP split. We also propose a pCR for TR 38.760-3 to capture the analysis.

- Discussion

    - 5G HLS (CU-DU split) benefits

5G RAN architecture has a clear functional and Higher Layer protocol layer split (CU-DU split) defined, along with standardized interfaces for inter-node communication. 5G HLS (CU-DU split) has many benefits as described below.

Deployment flexibility

5G technology provided operators the flexibility to choose from an integrated gNB, where the CU/DU/RU are deployed together, or a disaggregated/split gNB, where the CU (handling RRC/PDCP), DU (handling RLC/MAC/High PHY) and RU are deployed at separate sites based on the needs of the different deployment scenarios as shown in below figures.

- Deployment Scenario 1: Integrated gNB, where CU/DU/RU are deployed together at the cell site edge to provide low latency scheduling and performance.

- Deployment Scenario 2: CU can be deployed in telco edge cloud to manage multiple DUs resources and DU can be deployed at the cell site edge to provide better scheduling latency and RF components at the top of the cell tower.

- Deployment Scenario 3: CU can be deployed in the central cloud, DU can be deployed in telco edge cloud to make use of cloud computing power by sharing common pool of hardware resources across multiple DUs and RF components at the top of the cell tower.

- Deployment Scenario 4: Both CU and DU can be deployed in telco edge cloud as CNF to make use of cloud computing power by sharing common pool of hardware resources across multiple CUs/DUs and RF components at the cell site.

Observation 1: 5G provided flexibility to deploy RAN functions as an integrated gNB(i.e. CU/DU/RU at cell site) or as a disaggregated gNB where the CU/DU/RU can be deployed at different sites based on the Operator needs for the different deployment scenarios, to provide optimal system performance (e.g. reduced scheduling latency, efficient resource management) and efficient usage of shared or distributed hardware resources across multiple CUs/DUs.

Observation 2: 5G HLS (CU-DU split) allows flexibility to choose CU and DU from either a single vendor or multiple vendors.

In current 5G rollouts worldwide, we see a mix of both approaches—some networks use single-vendor RAN solutions, while others adopt multi-vendor setups with disaggregated CU-DU components.

Observation 3: Commercial gNB deployments today include both single vendor and multi-vendor split gNB as well as integrated gNB.

Simplified mobility

With centralized CU handling many DUs, mobility of UE across larger area is simplified for both L3 handovers and LTM cell switch (including subsequent LTM) as light-weight intra-gNB signaling procedures are used. For integrated gNBs, handovers and (subsequent) LTM cell switch becomes signaling-heavier as the inter-gNB procedures have to be used between the gNBs as shown in the following figure.

Observation 4: 5G HLS allows centralized CU to manage many DUs, which simplifies UE mobility handling for both L3 handovers and LTM cell switch (including subsequent LTM) as light-weight intra-gNB signaling procedures can be used.

Centralized coordination

With centralized CU handling many DUs, it is easier to support the NES, SON (CCO, MRO, etc) features as the central node has visibility to the data for multiple DUs.

Observation 5: 5G HLS allows centralized CU manage many DUs, which simplifies coordination for inter-DU NES, SON and CCO optimizations at central node.

    - 5G HLS (CP-UP split) benefits

RAN#109 plenary discussion included the below requirement in RP-252870, pCR for 38.914 on requirements for architecture and migration for 6G. So, RAN3 should also study the CP-UP split:

-	The 6G RAN architecture shall allow for control plane and user plane separation.

5G HLS supports a further functional split within the CU dividing it into CU-CP (Control Plane) and CU-UP (User Plane). This design allows operators to scale control and user plane resources independently. It enabled independent evolution of control plane and user plane nodes.  It also enables flexible deployment strategies: for instance, CU-UP can be placed closer to the edge or tower to meet ultra-low latency requirements for URLLC, while a centralized CU-UP may be more suitable for eMBB services.

Observation 6: 5G HLS provides flexibility to support a functional split within the CU dividing it into CU-CP and CU-UP to scale control and user plane resources independently and to evolve the CP and UP nodes independently.

Observation 7: CU-UP and CU-CP split allows the operator to deploy multiple CU-UPs based on use case, e.g.CU-UP closer to edge/tower for URLLC or centralized CU-UP for eMBB.

As 6G aims to support additional performance-centric services like immersive communications, integrated sensing and communications, and AI-native communications, the user data volumes and service diversity will be significantly higher compared to 5G. If large amounts of 6G user plane traffic are to be routed through a combined CP/UP node i.e. CU, it can lead to overload and performance bottlenecks, affecting service quality. Thus, keeping 6G CU-CP and CU-UP separate like 5G, helps avoid this issue by allowing each to scale and operate independently based on service demands.

Observation 8: Keeping CU-CP and CU-UP separate helps avoid overload and performance bottlenecks, when high data rate user plane traffic is routed through the RAN node.

Proposal 1: Agree the pCR to TR 38.760-3 in Annex to capture the 5G HLS benefits for the RAN internal functional split and interfaces.

# 3 Conclusion

Below are the observations discussed in this document.

Observation 1: 5G provided flexibility to deploy RAN functions as an integrated gNB(i.e. CU/DU/RU at cell site) or as a disaggregated gNB where the CU/DU/RU can be deployed at different sites based on the Operator needs for the different deployment scenarios, to provide optimal system performance (e.g. reduced scheduling latency, efficient resource management) and efficient usage of shared or distributed hardware resources across multiple CUs/DUs.

Observation 2: 5G HLS (CU-DU split) allows flexibility to choose CU and DU from either a single vendor or multiple vendors.

Observation 3: Commercial gNB deployments today include both single vendor and multi-vendor split gNB as well as integrated gNB.

Observation 4: 5G HLS allows centralized CU to manage many DUs, which simplifies UE mobility handling for both L3 handovers and LTM cell switch (including subsequent LTM) as light-weight intra-gNB signaling procedures can be used.

Observation 5: 5G HLS allows centralized CU manage many DUs, which simplifies coordination for inter-DU NES, SON and CCO optimizations at central node.

Observation 6: 5G HLS provides flexibility to support a functional split within the CU dividing it into CU-CP and CU-UP to scale control and user plane resources independently and to evolve the CP and UP nodes independently.

Observation 7: CU-UP and CU-CP split allows the operator to deploy multiple CU-UPs based on use case, e.g.CU-UP closer to edge/tower for URLLC or centralized CU-UP for eMBB.

Observation 8: Keeping CU-CP and CU-UP separate helps avoid overload and performance bottlenecks, when high data rate user plane traffic is routed through the RAN node.

Proposal 1: Agree the pCR to TR 38.760-3 in Annex to capture the 5G HLS benefits for the RAN internal functional split and interfaces.

# 4 References

[1] RP-252912, Revised SID: Study on 6G Radio.

# 5 Annex: pCR to TR 38.760-3

# -------------------------------------------Beginning of changes------------------------------------

### 6.2.1	General Principles

Editor’s note: The aim of this section is to describe general design principles and requirements for RAN Internal Interfaces

Note: RAN3 shall study CU-DU split and CP-UP split of the RAN internal functional split and provide the study results by June 2026 for RAN-P to decide.

### 6.2. x	5G HLS study

### 6.2. x.1 CU-DU split: Benefits

5G RAN architecture has a clear functional and Higher Layer protocol layer split (CU-DU split) defined, along with standardized interfaces for inter-node communication. 5G HLS (CU-DU split) has many benefits as described below

Deployment flexibility

5G provided operators the flexibility to choose from an integrated gNB, where the CU/DU/RU are deployed together, or a disaggregated/split gNB, where the CU (handling RRC/PDCP), DU (handling RLC/MAC/High PHY) and RU are deployed at separate sites based on the needs of the different deployment scenarios as shown in below figures.

- Deployment Scenario 1: Integrated gNB, where CU/DU/RU are deployed together at the cell site edge to provide low latency scheduling and performance.

- Deployment Scenario 2: CU can be deployed in telco edge cloud to manage multiple DUs resources and DU can be deployed at the cell site edge to provide better scheduling latency and RF components at the top of the cell tower.

- Deployment Scenario 3: CU can be deployed in the central cloud, DU can be deployed in telco edge cloud to make use of cloud computing power by sharing common pool of hardware resources across multiple DUs and RF components at the top of the cell tower.

- Deployment Scenario 4: Both CU and DU can be deployed in telco edge cloud as NFV to make use of cloud computing power by sharing common pool of hardware resources across multiple CUs/DUs and RF components at the cell site.

5G HLS (CU-DU split) allows the flexibility to choose RAN functions from either a single vendor or multiple vendors to support the different deployment scenarios. In current 5G rollouts worldwide, some networks use single-vendor RAN solutions, while others adopt multi-vendor setups with disaggregated CU-DU components.

Simplified mobility

With centralized CU handling many DUs, mobility of UE across larger area is simplified for both L3 handovers and LTM cell switch (including subsequent LTM) as light-weight intra-gNB signaling procedures are used. For integrated gNBs, handovers and (subsequent) LTM cell switch becomes signaling-heavier as the inter-gNB procedures have to be used between the gNBs as shown in below figure.

Centralized coordination

With centralized CU handling many DUs, it is easier to support the NES, SON (CCO, MRO, etc) features as the central node has visibility to the data for multiple DUs.

Considering these benefits, RAN3 should consider supporting a similar CU-DU split in 6G.

### 6.2.x.3 CP-UP split: Benefits

5G HLS, with a further functional split within the CU dividing it into CU-CP (Control Plane) and CU-UP (User Plane) provides following benefits:

- scale control and user plane resources independently

- independent evolution of control plane and user plane nodes

- support flexible deployment strategies: for instance, CU-UP can be placed closer to the edge or tower to meet ultra-low latency requirements for URLLC, while a centralized CU-UP may be more suitable for eMBB services.

- avoid overload and performance bottlenecks, when high data rate user plane traffic is be routed through the RAN node.

As 6G aims to support additional performance-centric services like immersive communications, integrated sensing and communications, and AI-native communications, the user data volumes and service diversity will be significantly higher compared to 5G. Thus, keeping 6G CU-CP and CU-UP separate like 5G, helps scale CP/UP independently and operate independently based on service demands.

-------------------------------------------End of changes-------------------------------------------