3GPP TSG-RAN WG3 Meeting #130	R3-258176

Dallas, USA, 17 – 21 November, 2025

Agenda item:	12.2.1

Source:	Nokia

Title:	Multi-hop UE trajectory across gNBs

Document for:	pseudo CR

# 1	Introduction

In the last meeting we made the following agreements:

Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to one or more gNBs where the UE is expected to connect and these cells are listed in chronological order.

The following solutions are considered for supporting multi-hop UE trajectory:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

Focus on cell-based granularity for measured and predicted multi-hop UE trajectory.

Study multi-hop UE trajectory for L3 HO.

The delivery of the measured&predicted cell-based multi-hop UE trajectory should not impact the core network.

The request and reporting to support measured multi-hop UE trajectory between gNBs should utilize the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure, at least for the first hop.

During the discussions, companies brought up questions related to the use case of predicted multi-hop trajectory and the scenarios of interest. Namely, it was discussed whether predicted multi-hop trajectory is to be consumed internally by the predicting node or whether there is a benefit to forward the predicted information to a neighbour node. If the predicted multi-hop trajectory is transferred to a neighbour node, the expected node’s behavior was discussed. Namely, it was discussed whether a receiving node shall further forward the received predicted trajectory as received, or whether it is allowed to modify it, or even stop the forwarding.

In addition, different options for the data collection of measured UE Trajectory to enable the receiving node of the data to train a model to predict multi-hop UE Trajectory were discussed and captured as open points:

How to transfer measurement feedback:

•	Option1: Parallel transmission from each hop gNB to the initial source gNB (i.e., Node2 to Node1, Node3 to Node1)

•	Option2: Hop-by-hop transmission (i.e., Node3 to Node2, Node2 to Node1)

•	Option3: Final-hop to initial-hop transmission (i.e., Node2 to Node3, Node3 to Node1)

•	Option4: Measured UE trajectory transfer via AMF (i.e. Node3 to AMF, AMF to Node1).

In this paper, we provide our views with respect to the above aspects. We also provide a TP for the pseudo CR to TR 38.745 in the Annex.

# 2	Discussion

## 2.1 Use Case Description

The first aspect that needs to be addressed with respect to predicted multi-hop UE trajectory is the use case for which this will be useful. In Rel-18, it was assumed that the usage of predicted UE Trajectory is to enable a node to:

- Perform handover preparation: Based on the output of a predicted UE trajectory AI/ML Model, a node can determine a list of candidate nodes/cell(s) to initiate handover preparation.

- Perform a handover execution: Based on the output of a predicted UE Trajectory AI/ML Model, a node can determine the optimal point in time for handover execution, i.e. the point in time when the UE enters the coverage of the target cell/node.

In Rel-18, a node predicting multi-hop UE Trajectory is enabled to better plan its handover preparation and execution by using this information internally. In addition, by sending this information in the HANDOVER REQUEST message towards the target node, it provides assistance information to the target node with respect to the UE movement at the cells of the target. This can help the target node to better plan its resource reservation. Note that for a source to predict UE Trajectory over the cells of the target node, it needs to receive measured UE Trajectory information from the target node. Therefore, naturally, the target node having information of UE movement over its cells, and through the UE history information it receives related to incoming handovers, may be able to predict by itself the trajectory over its own cells. In a multi-vendor environment, if a node is able to derive its own predictions, it can naturally trust the results more for its own actions since the AI/ML Model is optimized for the node itself and the node can determine the AI/ML Model accuracy. However, it can still use input from its neighbours that would serve as additional input to cross-check accuracy of its model.

As we can see from the above, predicted UE Trajectory can be used by a:

- Local node, which collects data, trains a predicted UE Trajectory AI/ML Model and uses it for AI/ML Model inference of UE trajectory for own served UEs

- Receiving node, that receives predicted UE Trajectory from another node

The above, are also applicable in the context of multi-hop UE Trajectory and we discuss the two options next.

### 2.1.1 Usage of predicted multi-hop UE Trajectory at a local Node

A node could leverage on the output of a predicted multi-hop UE Trajectory model during the handover preparation process, in order to identify and prepare one or more candidate target cells prior to handover execution. One example is illustrated in Figure 1. If the gNB1 predicts that the UE will move to gNB2 and traverse the cells of gNB2 where it will stay for a very short period of time before it is handed over to gNB3, then the gNB1 can directly prepare a handover over the cells of gNB3.

Figure 1 Example of usage of predicted multi-hop UE Trajectory at gNB1 where the gNB1 knowing, through the predicted multi-hop UE Trajectory, that the UE will stay very shortly in gNB2 does not prepare cells in gNB2 but prepares cells directly in gNB3.

Proposal 1: Include in TR 38.745 the scenario that predicted multi-hop UE Trajectory can be used internally at a gNB (no standardization impacts) to avoid preparation of cells with very short stay.

Another example where predicted multi-hop UE Trajectory can be used at the local gNB is to avoid cell preparations for cells for which the gNB has a good understanding (through its predicted multi-hop UE Trajectory) that the UE will not visit. This can be seen in the example of Figure 2.

Figure 2 Example of usage of predicted multi-hop UE Trajectory at gNB1 where gNB1, knowing that the UE will follow the path through gNB2 and gNB3 does not prepare cells in gNB4 and gNB5.

Proposal 2: Include in TR 38.745 the scenario that predicted multi-hop UE Trajectory can be used internally at a gNB (no standardization impacts) to avoid preparations of cells that the UE is not expected to visit.

Note that, for both the above scenarios to be possible, the gNB1 has to have an Xn interface both to gNB2 and gNB3.

A scenario that was discussed also in the previous meeting of multi-hop trajectory prediction is to enable a source node to identify which are the candidate cells to prepare. According to legacy methods, a gNB in HANDOVER REQUEST message always sends a single trajectory where the UE is predicted to move in the cells of the subsequent gNB(s). This is the trajectory (path) with the maximum likelihood. This path indicates a list of cells in chronological order, e.g., cell 1 will be visited with a predicted time UE stays in the cell 1, T1, cell 2 will be visited with a predicted time UE stays in the cell 2, T2, and so on and so forth. Therefore, unless the predicted time T1 that UE is predicted to stay in cell 1 is very short, the gNB can assume that the two cells, cell 1 and cell 2, will not have overlapping coverage and therefore cannot be prepared simultaneously.

However, even though a gNB sends in HANDOVER REQUEST message a single predicted multi-hop UE Trajectory (the one with the maximum likelihood), this does not prevent a gNB (internally) to predict multiple trajectories, with different probabilities. In fact, there are many cases where the multi-hop UE trajectory prediction will not indicate with 100% certainty a single path where the UE will be handed over. Hence, a gNB, combining results of different multi-hop UE trajectory predictions can also determine a list of candidate cells to prepare for a handover.

Proposal 3: Include in TR 38.745 the scenario that predicted multi-hop UE Trajectory can be used internally at a gNB (no standardization impacts) when the node prepares multiple candidate cells across different gNBs, e.g., in case of inter-CU subsequent LTM (whereby the original configuration is reused).

So, we can see from the above that predicted multi-hop UE Trajectory can be used internally by a local node making the prediction in order to avoid handover preparation to cells with very short stay and in order to prepare multiple candidate cells across different gNBs, e.g., in case of inter-CU subsequent LTM. In all those scenarios, an Xn interface is available between the node making the prediction and the involved gNBs.

Observation 1: For the scenarios where multi-hop predicted UE trajectory is used internally at the predicting node, an Xn interface is available between the gNB making the prediction and the rest of the involved gNBs.

### 2.1.2 Usage of predicted UE Trajectory at a receiving node

Another scenario that was broadly discussed in the previous meeting is the scenario where predicted multi-hop UE Trajectory is used by a receiving node. During the discussions, some companies proposed to reuse the Rel-18 mechanism to send predicted multi-hop UE Trajectory from a source node to a target node (that may be multiple hops away) in a HANDOVER REQUEST message. A question that arises is a) what is the transferred predicted multi-hop UE Trajectory information sent from a source node to a target node and b) what is the use case that such signalling serves, namely, how can a node use a predicted multi-hop UE Trajectory for a “specific” UE when it only receives it during a handover.

We identify two different cases:

case 1: The predicted multi-hop UE Trajectory is forwarded along the path of gNBs where the UE is handed over in the path from a source node towards a distant gNB.

We here assume that the AI/ML prediction model has some mechanism to limit the number of hops (nodes) for which a prediction is made. In a simple case, this could be simply a configured number of gNBs. In case 1, a source node predicts multi-hop UE Trajectory of a list of cells across gNBs where the UE is expected to connect and sends it in a HANDOVER REQUEST message to a target node, which is the first node of the predicted trajectory. We also assume that the HANDOVER REQUEST message is used for the subsequent propagation of the predicted trajectory.

We can distinguish two scenarios.

There are two possibilities, but both are leading to the same conclusion:

- Each subsequent node simply forwards the predicted trajectory that it received from the previous hop(s) to its next hop target node.

- Each subsequent node may update the predicted trajectory that it received from the previous hop(s) before forwarding to the next hop target node.

This is illustrated in Figure 3.

Figure 3 Predicted multi-hop UE Trajectory forwarding from gNB1 to gNB4. Each intermediate node in the path either simply forwards the received predicted multi-hop UE Trajectory towards a further distant node or modifies it e.g., based on its own prediction.

A question that arises is how the multi-hop predicted UE Trajectory is expected to be used by a receiving node. Each intermediate node receiving predicted multi-hop UE Trajectory can only use the information related to its own cells, including the handover target used for outgoing handover to the next node. For instance, there are no actions at gNB2 beyond the handover to the first target cell in gNB3 that could be affected or improved based on how a UE moves at gNB3 and/or gNB4.

case 2: Each intermediate node receiving predicted multi-hop UE Trajectory forwards only the predicted multi-hop UE Trajectory corresponding to subsequent hops towards a further distant node.

In this case, a source node predicts multi-hop UE Trajectory and sends it to the next hop target. Each node receiving the predicted multi-hop UE Trajectory, can determine whether to send predicted multi-hop trajectory to the next node or not. For example, the node can use the predicted trajectory from the previous hop and determine (e.g., through its own local AI/ML Model) the predicted multi-hop UE trajectory for the subsequent hops. A node could also decide not to send the predicted multi-hop UE Trajectory to the next hop node if for instance it determines that the available prediction is not accurate enough. So, the predicted multi-hop UE trajectory that each intermediate gNB sends to the next hop contains of a list of cells belonging to gNBs where the UE is expected to connect in the future in chronological order.

The observations regarding to the received predicted multi-hop trajectory information at a receiving node are the same as in case 1, namely, how is the received multi-hop predicted UE Trajectory expected to be used by a receiving node, including a further distant node? Since predicted multi-hop trajectory is sent in a HANDOVER REQUEST message, the only useful information to a node receiving it corresponds to a prediction over the list of cells of the node itself and the handover target cell of the next node the UE moves to.

Figure 4 Predicted multi-hop UE Trajectory forwarding from gNB1 to gNB4. Each intermediate node in the path forwards only the predicted multi-hop UE Trajectory corresponding to subsequent hops towards a distant node.

Additionally, we assume that also the intermediate nodes will generate their own predicted UE trajectory for served UEs. Therefore, under all cases and scenarios we can think of, there is no use case where it would be beneficial to send predicted multi-hop UE Trajectory to subsequent nodes beyond the target cell for handover to the next node, if this information is sent in a HANDOVER REQUEST message.

Observation 2: In both cases 1 and case 2, if the predicted multi-hop UE Trajectory is sent in the HANDOVER REQUEST message,  the only useful information to each intermediate node of a multi-hop path is equivalent to the predicted single-hop UE trajectory received from the previous hop with respect to the node’s cells together with the handover target cell used for an outgoing handover from the intermediate node to the next hop.

On the other hand, as described above, there could be benefits to use internally predicted multi-hop UE Trajectory at the predicting gNB, e.g., to help it optimize its handover preparations.

## 2.2  Predicted paths in terms of anticipated traffic towards a gNB

As discussed in the previous section, we do not think that there is a use case justifying sending predicted multi-hop UE Trajectory from a gNB to another in a UE-associated way (i.e., in HANDOVER REQUEST message). However, enabling a node to receive predicted multi-hop trajectories, in terms of an anticipated traffic or number of UEs, towards its cells could be very beneficial for several use cases, such as for cell switch-off actions, resource reservation, load management to name a few. For instance, if a node knows that there is a lot of traffic expected to arrive to its cells it can postpone Energy Saving actions or it can better balance the load over its cells if it knows to which cells the traffic (in terms of number of anticipated UEs) is expected to arrive.

Proposal 4: Enabling a node to receive predicted multi-hop trajectories, in terms of an anticipated total traffic or number of UEs towards its cells could be very beneficial for several use cases, such as for cell switch-off actions, resource reservation, load management to name a few.

In order to avoid unnecessary or unrequested information being sent to the network nodes, we think that a node should only receive predicted multi-hop trajectory, if it has requested it. As can be shown in Figure 5, in practice, there could be a very large number of trajectories passing through a node, growing exponentially with the number of the allowed hops defining the multi-hop. We therefore think that in order to limit the complexity of the algorithm predicting multi-hop trajectory at a network node the number of hops of the requested predicted multi-hop UE trajectories should be limited.

Figure 5 Multi-hop trajectory passing from gNB1. n hop trajectory would exclude all paths that are farther from gNB 41.

Proposal 5: RAN3 to study how to enable a node to subscribe to other nodes for predicted multi-hop trajectories of a specific number of hops towards itself.

We illustrate the above in the following example in Figure 6. gNB1 requests predicted 2-hop trajectories from its one-hop neighbour, gNB 21, that pass through itself. gNB 21 also checks with its one hop neighbours (gNB 31 and gNB 32) in this example for one-hop trajectories that pass from gNB 21. Each node, gNB 31 and gNB 32, reply to gNB 21 of a list of predicted trajectories that pass from gNB 21. gNB 21 aggregates those trajectories and responds back to gNB 1. The benefit of this approach is that the UE context does not need to be maintained over a long period after the UE has left the source gNB, it simplifies multi-hop trajectory prediction and at the same time it provides a flow over a multi-hop path of gNBs.

Figure 6 Example of multi-hop trajectory corresponding to an overall anticipated traffic towards the cells of a gNB.

Proposal 6: Exchange predicted multi-hop UE trajectories between neighbours not associated to a single UE.

## 2.3 Data Collection for Predicted Multi-hop UE Trajectory Training

In the last meeting we captured 4 possible options to transfer measured UE Trajectory feedback to a source node. They can therefore provide the source node with input data in order to train an AI/ML Model for predicted multi-hop UE Trajectory.

•	Option 1: Parallel transmission from each hop gNB to the initial source gNB (i.e., Node2 to Node1, Node3 to Node1)

•	Option 2: Hop-by-hop transmission (i.e., Node3 to Node2, Node2 to Node1)

•	Option 3: Final-hop to initial-hop transmission (i.e., Node2 to Node3, Node3 to Node1)

•	Option 4: Measured UE trajectory transfer via AMF (i.e. Node3 to AMF, AMF to Node1).

Note that, Option 4 is eliminated based on the agreement:

The delivery of the measured&predicted cell-based multi-hop UE trajectory should not impact the core network.

We proceed next to provide some more explanation on the Options 1-3 (note that those options are only described in high-level and within each option multiple alternative solutions may be possible):

Option 1: Parallel Transmission (Figure 7)

In parallel transmission, gNB1 initiates Data Collection procedures with gNB2 and gNB2 initiates Data Collection procedures with gNB3 to request measured UE Trajectory. Each of the gNBs, gNB2 and gNB3, send their measured UE Trajectory directly to gNB1. The drawback of this approach is that it requires the availability of Xn interface between the subsequent gNBs and the source and it also requires that subsequent gNBs know the source node ID.

Figure 7 Parallel Transmission

Option 2: Hop-by-hop Transmission (Figure 8)

In hop-by-hop transmission gNB1 initiates Data Collection procedures with gNB2 and gNB2 initiates Data Collection procedures with gNB3 to request measured UE Trajectory. The measured (multi-hop) UE Trajectory is received at gNB1 through hop-by-hop transmission of measurements through gNB2. The drawback of this solution is that gNB2 will need to maintain the UE context until gNB1 receives the measurements.  In addition, it creates signalling load for intermediate nodes for the step of retro-propagation of the measured UE trajectory.

Figure 8 Hop-by-hop Transmission

Option 3: Final-hop to initial hop transmission (i.e., node 2 to node 3, node 3 to node 1) (Figure 9)

This approach is similar to Option 2 in that gNB1 initiates Data Collection procedures with gNB2 and gNB2 initiates data collection procedures with gNB3 to request measured UE Trajectory. Each node sends its measured UE Trajectory to the next hop which aggregates the received measured UE Trajectory with its own measurements and sends it to the next hop. The final hop forwards the measured multi-hop UE Trajectory back to the source node. One drawback of this approach is that the source node ID needs to be known to the subsequent hops so that the final-hop forwards the aggregated multi-hop trajectory back to the source node.

Figure 9 Final-hop to Initial Hop Transmission

We notice that option 2 creates signalling load for intermediate nodes for the step of retro-propagation of the measured UE trajectory. Option 1 and 3 avoid this drawback. We therefore propose to capture option 1 and option 3 for further study.

Proposal 7: Capture option 1 and option 3 in the TR for further study.

# 3	Conclusion

In this paper, we make the following observations and proposals:

Proposal 1: Include in TR 38.745 the scenario that predicted multi-hop UE Trajectory can be used internally at a gNB (no standardization impacts) to avoid preparation of cells with very short stay.

Proposal 2: Include in TR 38.745 the scenario that predicted multi-hop UE Trajectory can be used internally at a gNB (no standardization impacts) to avoid preparations of cells that the UE is not expected to visit.

Proposal 3: Include in TR 38.745 the scenario that predicted multi-hop UE Trajectory can be used internally at a gNB (no standardization impacts) when the node prepares multiple candidate cells across different gNBs, e.g., in case of inter-CU subsequent LTM (whereby the original configuration is reused).

Observation 1: For the scenarios where multi-hop predicted UE trajectory is used internally at the predicting node, an Xn interface is available between the gNB making the prediction and the rest of the involved gNBs.

Observation 2: In both cases 1 and case 2, if the predicted multi-hop UE Trajectory is sent in the HANDOVER REQUEST message,  the only useful information to each intermediate node of a multi-hop path is equivalent to the predicted single-hop UE trajectory received from the previous hop with respect to the node’s cells together with the handover target cell used for an outgoing handover from the intermediate node to the next hop.

Proposal 4: Enabling a node to receive predicted multi-hop trajectories, in terms of an anticipated total traffic or number of UEs towards its cells could be very beneficial for several use cases, such as for cell switch-off actions, resource reservation, load management to name a few.

Proposal 5: RAN3 to study how to enable a node to subscribe to other nodes for predicted multi-hop trajectories of a specific number of hops towards itself.

Proposal 6: Exchange predicted multi-hop UE trajectories between neighbours not associated to a single UE.

Proposal 7: Capture option 1 and option 3 in the TR for further study.

4	References

[1] TR 38.745, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3, v0.1.0

# Annex – pCR to TR 38.745

This pCR is based on [1].

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

# 4	Use cases and Solutions

## 4.1	Multiple-hop UE trajectory

### 4.1.1	Use case description

Editor’s Note: Capture the description of use case

In Rel-18, the cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to gNBs where the UE is expected to connect and these cells are listed in chronological order.

Predicted multi-hop UE Trajectory can be used internally by a gNB in order to:

- avoid preparation of cells with a very short stay

- avoid cell preparations for cells that the UE is not expected to visit

- prepare multiple candidate cells across different gNBs, e.g., in case of inter-CU subsequent LTM (whereby the original configuration is reused)

### 4.1.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions are considered for supporting multi-hop UE trajectory:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

4.1.2.2. Standardization Impacts

RAN3 to study two schemes for the measured UE Trajectory:

- Option 1: Parallel transmission from each hop gNB to the initial source gNB (i.e., Node2 to Node1, Node3 to Node1)

- Option 3: Final-hop to initial-hop transmission (i.e., Node2 to Node3, Node3 to Node1)

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>