3GPP TSG-RAN WG3 Meeting #130	R3-258177

Dallas, USA, 17 – 21 November, 2025

Agenda item:	12.2.3

Source:	Nokia

Title:	AI/ML-based Handover Enhancements

Document for:	pseudo CR

# 1	Introduction

Rel-20 NG-RAN AI/ML study focuses on AI/ML optimization of aspects pertaining to AI/ML mobility. In particular, one of the objectives is to study handover enhancements.

The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows:

- Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios:

  - multi-hop UE trajectory across gNBs;

  - intra-CU LTM;

- handover enhancements, e.g. inter-CU LTM.

NOTE: No UE impacts.

In the last meeting, companies could not reach consensus regarding the last scenario of the SID and we captured the following open point:

Study inter-CU LTM in R20 SI?

In this paper, we provide our views related to this as well as possible handover enhancements that can be considered in Rel-20.

# 2	Discussion

Inter-CU LTM is at the moment under work under MRO enhancements in SON/MDT work item. Traditionally, AI/ML optimization of a feature has been introduced after its optimization through non-AI/ML methods has completed. In addition, AI/ML Optimization in intra-CU LTM is under study under this SID. In our view, we should first study F1 impacts in Rel-20 for intra-CU LTM.

Proposal 1: Prioritize study of AI/ML Optimization of intra-CU LTM.

Having said this, relevant F1 agreements for intra-CU LTM can be ported to Xn interface, when it seems necessary. For, instance this could be possible for early TA acquisition.  However, such should not require inter-CU LTM study at the same time with the intra-CU LTM feature.

Proposal 2: Consider reuse of relevant F1 agreements for intra-CU LTM in Xn interface, for the purpose of supporting inter-CU LTM. (This does not require inter-CU LTM study.)

One of the core enhancements introduced by AI/ML for NG-RAN is the introduction of UE Performance feedback after a handover pertaining to AI/ML Optimization e.g., AI/ML Mobility, AI/ML Energy Saving or AI/ML Load Balancing. Various UE Performance metrics are sent from a target node to a source node after a handover, related to average UE throughput, average packet delay and average packet loss. UE Performance feedback was introduced to measure per UE Performance after a handover.

Handover in 5G is based on the principle of break-before-make, i.e. during handover execution the UE’s UL/DL radio transmission will disconnect from the source cell before it connects to the target cell. The UL/DL transmission interruption time during connected mode mobility is the lapse of time from the point the UE disconnected from the source cell and until it has reestablished transmission with the target cell. During this lapse of time the UE does not receive any service. The UL/DL transmission interruption time is therefore one important additional metric that characterizes the handover decision. Even though Layer 1/Layer 2 Triggered Mobility (LTM) was a feature introduced to reduce interruption time by performing early UL/DL synchronization, interruption time is a valid issue for all types of handovers.  Information on interruption time available at the source node can help the source node use interruption time as part of input information to an AI/ML Mobility optimization model which can determine for example the best timing for handover execution, or optimize target cell selection, based on a predicted interruption time.

Interruption time can be calculated by different entities, as can be shown in Figure 1, namely by:

- The UE as shown in steps 18-23

- The gNB-CU as shown in steps 19-24

- The candidate gNB-DU as shown in steps 20-23

- The source gNB-DU as shown in steps 18-25

Proposal 3: RAN3 to study AI/ML interruption time optimization and decide the right entity to calculate it.

Figure 1 Inter-gNB-DU LTM (Figure 8.2.1.5-1, TS 38.401)

Another interesting option for this study could be to look into handover strategies and coordination of handover strategies rather than improvement of individual handovers. For instance, so far UE Performance feedback has been used to evaluate  performance of a single handover event. However, in some scenarios, such as energy saving and load balancing, the network action involves offloading a number of UEs from a source node to a target node. In such cases, sending UE Performance on a per UE granularity is an unnecessary overhead. Instead, performance can be evaluated based on e.g. aggregated performance metrics for a group of UEs.

Proposal 4: RAN3 to study UE performance in scenarios with offload of multiple UEs.

# 3	Conclusion

In this paper we make the following observation and proposals:

Proposal 1: Prioritize study of AI/ML Optimization of intra-CU LTM.

Proposal 2: Consider reuse of relevant F1 agreements for intra-CU LTM in Xn interface, for the purpose of supporting inter-CU LTM.  (This does not require inter-CU LTM study.)

Proposal 3: RAN3 to study AI/ML interruption time optimization and decide the right entity to calculate it.

Proposal 4: RAN3 to study UE performance in scenarios with offload of multiple UEs.

4	References

[1] TR 38.745, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3, v0.1.0

# Annex – pCR to TR 38.745

This pCR is based on [1].

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

# 4	Use cases and Solutions

## 4.3	Other handover enhancements

Editor’s Note: Identify other handover enhancements via AI/ML, e.g., inter-CU LTM

On parameter that is valid for all types of handovers is the UL/DL transmission interruption time during connected mode mobility. It corresponds to the lapse of time from the point the UE disconnected from the source cell and until it has reestablished transmission with the target cell. During this lapse of time the UE does not receive any service. AI/ML techniques can be used to optimize UL/DL transmission interruption time.

Even though so far AI/ML techniques were considered only for a single handover, some scenarios, such as energy saving and load balancing, involve offloading a number of UEs from a source node to a target node. In such cases, sending UE Performance on a per UE granularity is unnecessary and considering UE Performance over a group of UEs could optimize the experienced overhead.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>