# 3GPP TSG-RAN WG3 Meeting #130															R3-258215

# Dallas, USA, 17-21, November 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0507 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections to the support of split architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, CMCC, China Unicom, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Following miscellaneous issues still remain: - AI/ML in NG-RAN is supported in split architecture scenarios, including CU/DU split and CU-CP/CU-UP split. It was agreed that model training or inference can be located in the gNB-CU-CP. This description is missing in the specification. - The trigger condition of the collection of UE performance feedback over E1 interface is missing in the specification. - The Stage 2 text for AI/ML-assisted CCO has no relevance to the content of the data collection and reporting chapter. One aspect is transmitted via the Configuration Update message, while the other references a new data collection reporting procedure. Therefore, placing the CCO description within the data collection description appears abrupt. We propose using a new section to encompass the CCO description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the deployment scenarios where model training or inference can be located in the gNB-CU-CP. - Add the trigger condition of the collection of UE performance feedback over E1 interface is missing in the specification. - Using a new section ”AI/ML assisted Coverage and Capacity Optimization” to encompass the CCO description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Miscellaneous issues about the support of split architecture still remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.11.1, 7.11.3, 7.11.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

7.11	Support of AI/ML for NG-RAN

7.11.1	General

The support of AI/ML for NG-RAN is specified in TS 38.300 [2].

In case of CU-DU split architecture, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

In case of gNB-CU-CP and gNB-CU-UP architecture, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU-CP.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU-CP.

For a split gNB, the Energy Cost (EC) of the gNB is the sum of the Energy Cost of its gNB-DUs.

7.11.2	OAM requirements

OAM configures the following on gNB-DU:

-	the minimum and maximum energy consumption values corresponding to the minimum and maximum EC index values respectively, based on an implementation-specific mapping rule, which is unified within a defined area.

-	the recommended time interval within which an gNB-DU selects an implementation-specific time window for averaging of the measurements of the gNB-DU’s consumed energy.

7.11.3	Data Collection and Reporting

The following information can be configured to be reported by a gNB-DU:

-	Energy Cost (EC),

-	UE performance feedback.

The collection of EC is configured through the Resource Status Reporting Initiation procedure while the reporting is performed through the Resource Status Reporting procedure. Packet delay measurement collection for UE Performance from gNB-DU is triggered through the UE CONTEXT SETUP REQUEST or UE CONTEXT MODIFICATION REQUEST messages and the packet delay measurements collected by the gNB-DU are reported from gNB-DU to gNB-CU-UP through the ASSISTANCE INFORMATION DATA frame, specified in TS 38.425 [33].

The following information can be configured to be reported by a gNB-CU-UP:

-	UE performance feedback.

The collection and reporting of UE performance feedback from a gNB-CU-UP are configured through the Data Collection Reporting Initiation procedure, while the reporting is performed through the Data Collection Reporting procedure.

The collection of UE performance feedback is triggered at successful bearer context setup or successful bearer context modification.

Signaling by a split gNB of the RLC Average UE UL/DL Throughput over the Xn interface is not supported in this version of the specifications.

7.11.x	AI/ML assisted Coverage and Capacity Optimization

At any given point in time, for a list of predicted affected cells and beams, there is only one predicted CCO issue generated by a gNB-CU.

A gNB-DU may receive, in a GNB-CU CONFIGURATION UPDATE message, predicted CCO assistance information, including a predicted CCO issue, the predicted affected cells and beams, and the time when the predicted CCO issue will happen.

A gNB-DU may also receive, in a GNB-CU CONFIGURATION UPDATE message, the future coverage configurations of a list of cells and beams not served by the gNB-DU with their corresponding future coverage cause and the time of application of the future coverage configuration. In response, the gNB-DU may indicate to the gNB-CU, in a GNB-DU CONFIGURATION UPDATE message, a future coverage configuration for its cells and beams, and an associated time and cause.

A gNB-DU may receive, in a GNB-CU CONFIGURATION UPDATE message, a notification that a previously received predicted CCO issue, associated to the list of affected cells and beams received with the predicted CCO issue, is cancelled.

The gNB-DU may also notify the gNB-CU, in the GNB-DU CONFIGURATION UPDATE message, that a previously notified non applied coverage state change, for a list of affected cells and beams corresponding to a cancelled predicted CCO issue, has been cancelled.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>