3GPP TSG-RAN WG3 Meeting #130	R3-258252

Dallas, Texas, USA, 17-21 November 2025

Agenda Item:		10.5.1

Source:		NEC

Title:			(pCR to TR38.760-3) AI/ML use cases for 6G

Document for:		Discussion and decision

# Introduction

In RAN3#129bis meeting, the following agreements are achieved [1]:

The design of AI/ML algorithms and models for RAN3 led use cases are implementation specific and out of RAN3 scope.

The following use cases will be studied in RAN3:

- AI/ML based Network energy saving

- AI/ML based mobility optimization

In this contribution, we provide our further considerations on AI/ML use cases for 6G.

# Discussion

2.1	AI/ML based network energy saving

Nowadays operators rank sustainability as the most important area for their network development. High energy consumption is an urgent issue to resolve, therefore energy saving becomes an important feature for operators contributing to operational cost savings and commitments to net zero. To address this issue, AI/ML is a powerful tool to help network achieve energy optimization.

In Rel-19, node level Energy Cost (EC) is introduced as the AI/ML metric and transferred between gNBs and from gNB-DU to gNB-CU upon request, and we also drew a conclusion after the study that

“Enhancements to the node-level EC to a finer granularity could be beneficial, assuming that such enhancements enable network-level energy saving”.

In 6G, we suggest to study AI/ML based finer granularity energy saving, for example cell level. Using AI/ML, an optimum cell on/off pattern can be obtained based on future resource needs, which can greatly reduce gNB TX/RX activity time and achieve maximum energy efficiency.

Proposal 1: Study AI/ML based network energy saving in finer granularity, including both node level and cell-level on/off.

2.2	AI/ML based mobility optimization

In Rel-18, to achieve optimal resources allocation for the handed over UE, cell-based UE trajectory prediction is introduced to be transferred from the source gNB to the target gNB during handover preparation procedure. Correspondingly, measured UE trajectory as feedback information for source gNB can be collected by target gNB across RRC states. In 6G, measured and predicted UE trajectory information still needs to be considered as an input data for AI/ML.

Besides, considering one unified mobility mechanism is preferred in 6G, to select an optimum mobility option and to decide a suitable mobility timing can be achieved by use of AI/ML. But this depends on the final mobility solution designed in 6G.

Proposal 2: Study how to select an optimum mobility option and decide a suitable mobility timing by use of AI/ML.

2.3	Other use cases for 6G

In 5G, RAN and SA conducted AI/ML studies separately, which lead to end-to-end network performance optimization which involves different network domains cannot be guaranteed. In order to enhance overall network performance and user experience for 6G, the valuable data from CN can be used as input data or feedback data to AI/ML model in RAN to enable optimum actions. Therefore, in general, we think cross network domain coordination needs to be considered for 6G AI/ML use cases.

Proposal 3: Consider cross network domain coordination for 6G AI/ML use cases.

6G will expand on top of the existing infrastructure of 5G, and it will be designed to surpass 5G by providing faster speeds, more efficient communication and wider network coverage than 5G. Additionally, 6G is expected to unlock a variety of innovative use cases that aim to leverage advancements in AI, sensing, NTN, immersive communication.

Besides existing services supported in 5G, new and promising future services become possible in 6G. Therefore, attempts to explore more AI/ML use cases, which provides new paradigms and opportunities across multiple fields, become necessary. Below are the other use cases we prefer to consider for 6G AI/ML.

2.3.1	AI/ML based network slicing

In 6G TR 38.914 [2], “The 6G RAN architecture shall allow for the operation of network slicing.” is captured as one of the requirements for architecture and migration, which means network slicing is a must to have feature for 6G.

In Rel-19, predicted radio resource status per slice, predicted slice available capacity and per slice UE performance are newly specified to supported by a gNB. Similarly, in order to facilitate network slicing in 6G, predicted radio resource status per slice and predicted slice available capacity should still be generated by AI/ML inference and per slice UE performance should still be measured and collected.

In addition, for end-to-end slice, if follow legacy, OAM is in charge of slice configuration, slice resource allocation and slice performance management. SLA requirements fulfilment in RAN, including PDCP throughput, PRB usage, is a useful information for OAM for dynamically controlling slice configurations or for AI/ML model training. Therefore, slice performance exchange between RAN and OAM is beneficial for AI/ML based network slicing.

2.3.2	AI/ML based QoE

QoE is defined as a mechanism targeting at collecting application layer measurements from UEs, and in 5G, QoE metrics includes average throughput, initial playout delay, buffer level. Considering QoE mechanism is a key feature for service quality evaluation, it should still be supported in 6G.

In order to optimize resource allocation per UE to improve user experience, QoE prediction by AI/ML model can be used in RAN for better resource reallocation. The study can include e.g. QoE measurement collection for AI/ML model training and inference, future QoE prediction for specific UE(s), QoE prediction exchange between RAN nodes for better handover decision and whole network optimization, actual QoE measurements feedback for model performance monitoring.

As agreed in TR 38.914 “The 6G RAN architecture shall support enhanced service awareness in RAN”, we think there will be more QoE metrics can be visible to RAN, therefore, it is beneficial to investigate this use case taking into account all RAN visible QoE parameters.

2.2.3	Agentic AI RAN

Agentic AI refers to AI systems composed of AI agents that can behave and interact autonomously in order to achieve their objectives.

In traditional RAN, the configuration is set in advance by OAM. Agentic AI RAN with which AI agents reside, on the other hand, are given a set of configurations by local AI agent and will then autonomously select which of those are best suited to achieve specific performance goals. This powerful agentic RAN with capability of autonomous configuration finding and dynamic environmental adaptation can largely reduce network maintenance complexity and meanwhile increase network performance. In the 6G study, we suggest to study how to support interaction and knowledge transfer among AI agents.

Proposal 3: In 6G AI/ML study, the following use cases can be considered:

- AI/ML based network slicing

- AI/ML based QoE

- Agentic AI RAN

TP to TR38.760-3 which captures the agreements from last meeting and the above analysis is in Annex.

Proposal 4: Agree the TP to TR38.760-3 for AI/ML use cases in Annex.

# Proposals

In this contribution, we provide our views on 6G AI/ML use cases and made the following proposals:

Proposal 1: Study AI/ML based network energy saving in finer granularity, including both node level and cell-level on/off.

Proposal 2: Study how to select an optimum mobility option and decide a suitable mobility timing by use of AI/ML.

Proposal 3: Consider cross network domain coordination for 6G AI/ML use cases.

Proposal 3: In 6G AI/ML study, the following use cases can be considered:

- AI/ML based network slicing

- AI/ML based QoE

- Agentic AI RAN

Proposal 4: Agree the TP to TR38.760-3 for AI/ML use cases in Annex.

# Reference

[1] RAN3_129bis_agenda_20251017_EOM2

[2] 3GPP TR 38.914 Study on 6G Scenarios and Requirements

# Annex	TP for TR38.760-3

<Start of change>

## 7.1	AI/ML use cases

Editor’s note: Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc…

    - AI/ML based network energy saving

In 6G, AI/ML based energy saving can be achieved in finer granularity, for example in both node level and cell level. Using AI/ML, an optimum on/off pattern can be obtained based on future resource needs, which can greatly reduce BS TX/RX activity time and achieve maximum energy efficiency.

    - AI/ML based mobility optimization

In Rel-18, to achieve optimal resources allocation for the handed over UE, cell-based UE trajectory prediction is introduced to be transferred from the source gNB to the target gNB during handover preparation procedure. Correspondingly, measured UE trajectory as feedback information for source gNB can be collected by target gNB across RRC states. In 6G, measured and predicted UE trajectory information still needs to be considered as an input data for AI/ML.

Besides, considering one unified mobility mechanism is preferred in 6G, to select an optimum mobility option and to decide a suitable mobility timing can be achieved by use of AI/ML.

    - AI/ML based network slicing optimization

In order to facilitate network slicing in 6G, predicted radio resource status per slice and predicted slice available capacity should still be generated by AI/ML inference and per slice UE performance should still be measured and collected.

In addition, for end-to-end slice, OAM is in charge of slice configuration, slice resource allocation and slice performance management. SLA requirements fulfilment in RAN, including PDCP throughput, PRB usage, is a useful information for OAM for dynamically controlling slice configurations or for AI/ML model training. Therefore, slice performance exchange between RAN and OAM is beneficial for AI/ML based network slicing.

    - AI/ML based QoE optimization

As agreed in TR 38.914 “The 6G RAN architecture shall support enhanced service awareness in RAN”, there will be more QoE metrics can be visible to RAN.

In order to optimize resource allocation per UE to improve user experience, QoE prediction by use of AI/ML can be used in RAN for better resource reallocation. The study includes e.g. QoE measurement collection for AI/ML model training and inference, future QoE prediction for specific UE(s), QoE prediction exchange between RAN nodes for better handover decision and whole network optimization, actual QoE measurements feedback for model performance monitoring.

    - Agentic AI RAN

Agentic AI refers to AI systems composed of AI agents that can behave and interact autonomously in order to achieve their objectives.

In traditional RAN, the configuration is set in advance by OAM. Agentic AI RAN with which AI agents reside, on the other hand, are given a set of configurations by local AI agent and will then autonomously select which of those are best suited to achieve specific performance goals. This powerful agentic RAN with capability of autonomous configuration finding and dynamic environmental adaptation can largely reduce network maintenance complexity and meanwhile increase network performance.

6G needs to support interaction and knowledge transfer among AI agents.

<End of change>