3GPP TSG-RAN WG3 Meeting #130	R3-258266

Dallas, TX, USA, November 17th – 21st 2025

Agenda Item:	10.2.2

Source:	Ericsson

Title:	(pCR for TR 38.760-3): 6G Data Collection

Document for:	Agreement

# 1	Introduction

At the 3GPP RAN Plenary meeting #108, the Rel-20 6G RAN SI was approved. The corresponding SID in [1] calls for the support for Data Collection. Accordingly, in this paper we discuss 6G Data Collection, its basic principles, and the data collection use cases that can be supported by the 6G Data Collection framework.

A pCR for TR 38.760-3 is provided in the Annex.

# 2	Support for Data Collection in 6G RAN Architecture

The Rel-20 6G RAN SID in [1] states the following:

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - […] - Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

Two main principles can be deduced from the above objectives:

- First, there should be one single 6G RAN architecture that supports both existing and new services, and the usage and deployment scenarios stipulated by TR 38.914.

- Second, this one single 6G RAN architecture shall support:

  - A user plane.

  - A control plane.

  - Exchange of radio signalling.

  - Transfer of collected data.

Following these principles, we deduce that 3GPP and RAN3 shall strive to define a unified architecture that is capable of supporting data transfer for various use cases, rather than defining a dedicated architecture for data transfer itself. In other words, instead of defining two different architecture options, one for data transfer and another for other different services, there should be one single 6G RAN architecture fulfilling all the requirements derived by the use cases of interest in 6G. This is in full accordance with the guidance from the SID that calls for avoidance of multiple options.

Observation 1-1: According to the 6G RAN SID, RAN3 shall define a common architecture framework that supports data transfer for various data types, meaning that the definition of a separate 6G RAN architecture, logical nodes and interfaces dedicated to data transfer shall be avoided.

The above principle is also followed by all previous generations of radio access networks, where data collection has been supported by means of enhancements to the common RAN interfaces and logical nodes used to support the user plane and the control plane, and without defining any specific RAN logical nodes or RAN interfaces dedicated to data collection.

With respect to the above, we acknowledge that legacy user and control plane interfaces may have shortcomings when it comes to transferring large amounts of data (which may also be non-UE associated). Consequently, we also acknowledge the need to define new communication mechanisms to transfer such data in a more efficient way. However, we believe that there is no need to introduce dedicated RAN logical entities to enable such new data transfer mechanisms.

Observation 1-2: Data collection and transfer shall be natively supported by the 6G RAN logical nodes and interfaces defined as part of a unified architecture framework.

In line with the above, we believe that the functionalities in charge of handling data transfer for various use cases should be internal to the RAN and should not be explicitly specified. The interfaces used to transfer data should be equivalent to legacy interfaces, such as the Uu interface, and the management interfaces (between the RAN and the OAM) which have been already used in the past to transfer data of various types.

With this vision, data collection and transfer becomes an integrated service natively provided by a common 6G RAN architecture.

Proposal 1: RAN3 to derive a common 6G RAN architecture that natively supports data collection and transfer, without defining a dedicated RAN architecture, dedicated RAN logical nodes or dedicated RAN interfaces for data collection and transfer.

# 3	The 6G Data Collection Framework

The 6G RAN SID in [1] calls for cooperation between RAN and SA WGs in fulfilling the SID objectives related to Data Collection:

- Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

In addition, an LS in [2] was sent from the SA#109 meeting, inviting SA2, SA5, SA3, SA6, TSG RAN, RAN2 and RAN3 to collaborate on 6G Data Collection. The LS in [2] states:

Regular coordination and collaboration among the WGs are expected, including checks on the progress of each group for 6G data related work tasks in March 2026 (TSG SA#111) and/or June 2026 (TSG#112), to avoid the duplicate design for the same functionality and to develop system wide consistent solutions.

Based on the above, we conclude that the work on the 6G Data Collection Framework involves 6G RAN, the 6G Core Network and the 6G OAM system, and that this work requires cooperation between relevant RAN WGs and SA WGs.

To initiate the work on 6G Data Collection Framework, RAN3 should discuss the requirements from its terms of reference.

Proposal 2-1: RAN3 to discuss and agree the RAN-specific requirements for the 6G Data Collection Framework.

One important requirement that can be deduced from previous discussions on 6G Data Collection, is that there is a need for enabling data reusability across the 6G system. Namely, data collected at one network entity should be made available at any other entity across the 6G system that may require this data, regardless of whether this entity is within the RAN domain, the core network domain or the OAM domain. For example, data generated at a 6G RAN node should be made available to other RAN nodes or to network entities in the 6G Core Network, or to the 6G OAM.

Observation 2-1: There is a need to make the collected data available across different domains of the 6G system, e.g., in the 6G RAN, in the 6G Core Network and in the 6G OAM system.

One important benefit of data reusability is that any network entity in need of data that has been previously collected by another network entity can acquire the data without having to initiate collection of this data again. Consequently, the processing load incurred by data collection, and the complexity of supporting some data collection use cases is greatly reduced. Note that, in some scenarios, data availability across the MNO domain is a necessity, rather than a bonus. One example where data storing and reusability is required is AI/ML data collection, where the training of AI/ML models can be hosted at the OAM. In this case, data produced by the RAN domain needs to be made available at the OAM domain.

Proposal 2-2: Data collected/generated by the 6G RAN should be made available to other entities and domains of the 6G system e.g., to other 6G RAN nodes, to the 6G Core Network, to in the 6G OAM system.

To enable data sharing and reusability, the 6G Data Collection Framework needs to include a Data Collection Repository, i.e., a network entity where the data collected by network nodes can be stored, and from which the data can be easily made available to all the network entities in the MNO domain that request the stored data. In that respect, RAN3 has traditionally identified the OAM (together with the RAN) as the entity that collects data and that also has additional responsibilities, such as, e.g., the Lifecycle Management of AI models.

Besides a Data Collection Repository that stores the data to be shared, there is a need for efficient mechanisms for data transfer between network entities and the Repository. Even in this respect, the OAM is a suitable candidate for hosting the Repository because the OAM can (even today) directly communicate with both the RAN and the Core Network for the purpose of data collection. This means that placing the Data Collection Repository in the OAM system enables data reusability across the 6G system in the best way. Moreover, the OAM architecture already provides streaming interfaces that can be used to transport data from the RAN to the OAM. Such interfaces are, for example, used to transport measurements collected by Trace or MDT, but they are also able to accommodate transport of any data in general. In fact, the streaming interface which the OAM uses to collect data from the RAN is specifically designed for data collection, as opposed to network interfaces whose main purpose is not data collection, and which would require significant enhancements to fulfil such a purpose.

All the above makes the OAM a natural and obvious choice for the placement of the data collection “hub”, i.e., the Data Collection Repository. Note that choosing the OAM for the placement of the Data Collection Repository means that the OAM becomes more capable and, as a consequence of the above, the impact of data collection on the 6G RAN Architecture is avoided.

Observation 2-2: To enable data reusability across the different domains of 6G system, placing the Data Collection Repository in the OAM is the optimal choice.

Proposal 2-3: The data collected/generated by the 6G RAN can be stored in a data collection repository in the OAM domain, from which this data can be made available to any network entity requesting this data.

From RAN3 perspective, one key aspect is the role of 6G RAN in data collection. Namely, the 6G RAN should be able to initiate data collection and to consume the data locally, for example in use cases such as AI/ML-based optimization, RAN-based model training, performance monitoring, intent fulfilment etc. This means that it should be possible for the 6G RAN to consume the data as soon as the data are generated, perhaps even before the data is stored (e.g., to support real-time use cases).

Proposal 2-4: The 6G RAN should be able to trigger data collection and to consume the collected data (e.g., as soon as the data are generated).

To enable data reusability and availability across different network domains, the 6G RAN should be able to export/expose the collected data to the Data Collection Repository.

Proposal 2-5: The 6G RAN should be able to provide the collected data to the data collection repository in the OAM system.

Finally, with respect to data reusability, a 6G RAN node should be able to request and consume the data collected by other network entities.

Proposal 2-6: The 6G RAN should be able to request and consume data generated by other network entities.

# 4	6G Data Collection Use Cases

The intention with the 6G Data Collection Framework is to support various data collection use cases. Some of the data collection use cases mentioned in previous discussions are (non-exhaustive list):

- AI/ML data collection.

- Observability data collection.

- QoE data collection.

- Sensing data collection.

- Positioning data collection.

Nevertheless, the intention of specifying the 6G Data Collection Framework is not to support all possible data collection use cases in 6G RAN. Instead, the Framework should support data collection for use cases that share the same requirements. To make an educated judgment about which data collection use cases can be supported by the Framework, it is necessary to establish a set of principles based on which data collection use cases can be classified. Based on the use case classification, it can be determined:

- Which data collection use cases can be supported by the 6G Data Collection Framework.

- Which data collection use cases should be supported by dedicated frameworks.

The essential purpose of the 6G Data Collection Framework is to enable availability and reusability of data across the 6G system, even in domains different from the domain where this data is collected. In other words, the main purpose of the 6G Data Collection Framework is to make data collected by a network entity available to any other entity that may need it, anywhere in the 6G system, provided that this other entity is authorized to use the data. Nevertheless, exporting the data outside the network entity that collected the data only makes sense if other entities can make use of this data.

Observation 3: Exporting the data outside the network entity that collected the data only makes sense if other entities can make use of the data and is authorized to use it.

As discussed in Section 3, the main functionalities of the 6G Data Collection Framework are:

- Data collection by network entities.

- Storing data in a Data Collection Repository.

- Making data available to other network entities, enabled by data transport between the Data Collection Repository and the network entities that produce or consume the data.

Therefore, the 6G Data Collection Framework should support collection of data that is:

- Useful, even after being collected and stored.

- Useful for network entities other than the network entity that collected/generated the data.

This translates into the following two requirements based on which it can be determined whether a specific data collection use case can be supported by the 6G Data Collection Framework:

- Temporal reusability of data: whether the data is reusable and valid after being collected and stored.

  - Data collection use cases that fulfil this requirement are those where it is beneficial to store data and distribute it to other network entities requesting the data. In other words, data collected in use cases that fulfil this requirement do not expire immediately.

  - One example scenario that fulfils this requirement is the collection of AI/ML data.

- Spatial reusability of data: whether the data collected at one network entity is useful for another network entity.

  - One example scenario that fulfils this requirement is data collection in a certain cell, used to train an AI/ML model that is addressing the entire network.

Proposal 3-1: The 6G Data Collection Framework should support data collection for use cases in which:

- The data is reusable after being collected and stored.

- The data is reusable at a network entity other than the network entity that collected the data.

The data collection use case currently discussed in RAN3 is AI/ML Data Collection. It is worth analysing whether this data collection use case fulfils the above two requirements.

With respect to temporal reusability of AI/ML data, we notice that the objective of AI/ML data collection is not limited to only understanding the current state of the network – in fact, the ambition is to understand how the network behaves over time because of certain events and changes. This leads to the conclusion that, in general, the data collected as a part of AI/ML data collection (e.g., a training dataset, consisting of the collected input and ground truth) does not expire and is always reusable with the correct baseline assumptions.

With respect to spatial reusability of AI/ML data, RAN3 has already specified several “AI/ML for NG-RAN” use cases in which RAN nodes exchange AI/ML data. This means that RAN3 has already verified that AI/ML data fulfils the spatial reusability requirement, i.e., that AI/ML data collected/generated at one network entity is reusable by another network entity.

Based on the above, we conclude that AI/ML data is a good fit for the 6G Data Collection Framework. In other words, to support the AI/ML data collection use case, there is a need to store the AI/ML data because this data does not expire immediately, and can be considered useful for network entities other than the one that collected/generated it. Consequently, it follows that AI/ML data collection fulfils both requirements and qualifies as one of the use cases to be supported by the 6G Data Collection Framework.

Proposal 3-2: The 6G Data Collection Framework should support AI/ML Data Collection.

With regards to any other possible use cases that could be tackled under the 6G Data collection Framework, we note that it is difficult to evaluate them due to lack of progress in 3GPP. For example, the Sensing data collection use case is right now under discussion for 5G and it will not be addressed in 6G until (likely) the second half of the 6G study. It is therefore premature to evaluate these use cases, and it is proposed to address them only when they will reach sufficient maturity.

Proposal 3-3: Address other use cases for the 6G Data Collection Framework once they reach sufficient maturity in 6G discussions.

- Conclusions

In this paper we discuss the 6G Data Collection Framework, the corresponding requirements, and the data collection use cases to be supported by the Framework.

The following is observed and proposed:

Support for Data Collection in 6G RAN Architecture

Observation 1-1: According to the 6G RAN SID, RAN3 shall define a common architecture framework that supports data transfer for various data types, meaning that the definition of a separate 6G RAN architecture, logical nodes and interfaces dedicated to data transfer shall be avoided.

Observation 1-2: Data collection and transfer shall be natively supported by the 6G RAN logical nodes and interfaces defined as part of a unified architecture framework.

Proposal 1: RAN3 to derive a common 6G RAN architecture that natively supports data collection and transfer, without defining a dedicated RAN architecture, dedicated RAN logical nodes or dedicated RAN interfaces for data collection and transfer.

The 6G Data Collection Framework

Proposal 2-1: RAN3 to discuss and agree the RAN-specific requirements for the 6G Data Collection Framework.

Observation 2-1: There is a need to make the collected data available across different domains of the 6G system, e.g., in the 6G RAN, in the 6G Core Network and in the 6G OAM system.

Proposal 2-2: Data collected/generated by the 6G RAN should be made available to other entities and domains of the 6G system e.g., to other 6G RAN nodes, to the 6G Core Network, to the 6G OAM system.

Observation 2-2: To enable data reusability across the different domains of 6G system, placing the Data Collection Repository in the OAM is the optimal choice.

Proposal 2-3: The data collected/generated by the 6G RAN can be stored in a data collection repository in the OAM domain, from which this data can be made available to any network entity requesting this data.

Proposal 2-4: The 6G RAN should be able to trigger data collection and to consume the collected data (e.g., as soon as the data are generated).

Proposal 2-5: The 6G RAN should be able to provide the collected data to the data collection repository in the OAM system.

Proposal 2-6: The 6G RAN should be able to request and consume data generated by other network entities.

6G Data Collection Use Cases

Observation 3: Exporting the data outside the network entity that collected the data only makes sense if other entities can make use of the data and is authorized to use it.

Proposal 3-1: The 6G Data Collection Framework should support data collection for use cases in which:

- The data is reusable after being collected and stored.

- The data is reusable at a network entity other than the network entity that collected the data.

Proposal 3-2: The 6G Data Collection Framework should support AI/ML Data Collection.

Proposal 3-3: Address other use cases for the 6G Data Collection Framework once they reach sufficient maturity in 6G discussions.

- References

[1] RP-251881, New SID: Study on 6G Radio.

[2] R3-256533 LS on Guidance on 6G data related work tasks (source: SA).

- Annex: pCR for TR 38.760-3

---------------------------------------Start of changes-------------------------------------------

# X	6G Data Collection

## X.1	6G Data Collection Framework

The 6G Data Collection Framework leverages the 6G RAN architecture to collect the data generated by the UE and the 6G RAN.

Data collected/generated by the 6G RAN should be made available to other entities and domains of the 6G system e.g., to other 6G RAN nodes, to the 6G Core Network, to the 6G OAM system.

The 6G RAN can trigger data collection and consume the collected data, as soon as the data is generated.

The data collected/generated by the 6G RAN can be provided to and stored in a data collection repository in the OAM system domain, from which it can be made available to any network entity requesting this data.

The 6G RAN can request and consume data generated by other network entities.

## X.2	Use cases for 6G Data Collection

The 6G Data Collection Framework supports the following data collection use cases:

-	AI/ML data collection.

--------------------------------------End of changes-------------------------------------------

Data Collection use cases should be distinguished from AI/ML use cases. AI/ML data collection is an example of a Data Collection use case. In turn, Network Energy Saving is an example of an AI/ML data collection use case.