3GPP TSG-RAN WG3 Meeting #130	R3-258334

# Dallas, US, 17 – 21 November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | draft | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on QoS parameters for QoS Notification Control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, Nokia Shanghai Bell, China Telecom, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-19 XR topic, when we discussed how to capture the PSDB, PSER in the QoS notification control parameters with/without alternative QoS profiles, we found a missalignment issue about the QoS notification control between RAN specifications (TS38.300 and TS37.340) and the SA2’s specification TS23.501: According to TS23.501, the GBR QoS parameters including GFBR, PDB and PER should be considered for QNC. The NG-RAN shall send notifications of “GFBR can no longer (or can again) be guaranteed” when it determines that the GFBR, PDB or PER of the QoS profile cannot be fulfilled (or can be fulfilled again). However, the description about QNC in TS 38.300 and TS 37.340 only include GFBR when considering the fulfilment of QoS, the PDB and PER were forgotten. RAN3 fixed this problem in Rel-19 XR, through replacing the GFBR by “GBR QoS” and adding reference to the TS23.501 in the RAN specifications. The corresponding agreed TPs can be found in R3-255867 (TP for TS38.300) and R3-255868 (TP for TS37.340). However, the misalignment issue haven’t been solved for previous releases (since Rel-16). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Replace the “GFBR” with “GBR QoS”, and add reference to TS23.501 in the paragraph about QoS notification control. Impact Analysis: Impact assessment towards the previous version of the specification (same release): - This CR has no impact with the previous version of the specification (same release). - This CR has impact on the functional point of view. The impact can be considered isolated because the change only affects the QoS notification control. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PDB and PER will not be considered when RAN determine the QoS profile can no longer (or again) be fullfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.13.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

10.13.2	MR-DC with 5GC

The Notification Control Indication procedure may be initiated either by the MN or by the SN and is used to indicate that GBR QoS for one or several QoS flows cannot be fulfilled any more or can be fulfilled again by the reporting node, as specified in TS23.501 [11].

Figure 10.13.2-1: Notification Control Indication procedure

Figure 10.13.2-1 shows an example signalling flow for the Notification Control Indication procedure.

1.	The MN may, for an SN terminated bearer, indicate, that the GBR QoS requested from the MN cannot be fulfilled anymore.
In case the SN terminated bearer is configured as a split bearer, the SN may decide to increase the share provided by the SN or it may decide to notify the MN that resources requested for the SN terminated bearer cannot fulfill the GBR QoS any more.

2.	Continuing the example message flow from step 1, the SN informs the MN that the GBR QoS for an SN terminated bearer cannot be fulfilled any more.

3.	The MN decides to inform the 5GC that NG-RAN cannot fulfill the GBR QoS for a GBR QoS flow any more.

End of Change