3GPP TSG-RAN WG3 Meeting #130	R3-258358

Dallas, US, 17 – 21 November 2025

Agenda item:	13.2

Source: 	China Telecom

Title:	On protocol stack solution for ISAC

Document for:	Discussion and decision

- Introduction

The first discussion on ISAC has taken in 3GPP RAN3#129bis meeting. Based on ISAC SI[1], the following agreements were made:

- Capture logical architecture for ISAC

- RAN3 focus on sensing protocol in coordination with SA2

Some agreements on logical network architecture and basic sensing call flow were captured in agreed TPs [2][3].

According to the guidance of last meeting, i.e. “To continue the discussion on protocol stacks, particularly the protocol stack for sensing reporting, to be aligned with SA2 conclusions.”, in this paper, we will discuss the protocol stack for ISAC and propose our views and proposals.

- Discussion

2.1 The functions of two protocol stacks

The current RAN-CN interface support two protocol stacks, i.e. NG-C control plane protocol stack and NG-U user data plane protocol stack.

2.1.1 NG-C control plane

The protocol stack for NG-C control plane is shown in figure 1[4].

Figure 1 Protocol stack for NG-C control plane

The Transport Network Layer of NG-C is based on IP transport, comprising SCTP on top of IP. SCTP is located between the control signalling application layer and the IP network layer. It uses the term "association" to define the protocol status between two peer SCTP users that exchange information. SCTP is connection-oriented.

The "association" of SCTP is more extensive than that of TCP connections. TCP connections have only one source address and one destination address. SCTP provides a way for each SCTP endpoint to provide a set of transport addresses for another peer endpoint, that is, transport address = a set of IP addresses + port number. SCTP multi-homing between two end-points, of which one or both is assigned with multiple IP addresses, made transport network redundancy can be achieved.

2.1.2 NG-U user plane

The protocol stack for NG-U user plane is shown in figure 2[5].

Figure 2: Protocol stack for NG-U user plane

GTP is a standardized, IP-based tunneling protocol defined by the 3GPP standards body (TS 29.281 [6]). It is the fundamental glue that holds together the user data plane in mobile networks from 3G (UMTS) to 4G (LTE) and 5G (NR). Its primary purpose is to carry user data (user internet traffic, voice packets, etc.) within the Core Network (5GC).

GTP encapsulates user IP packets (or Ethernet frames in 5G) inside a GTP header and a UDP/IP header. This creates a "tunnel" through the core network. The transport bearer is identified by the GTP-U TEID and the IP address (source TEID, destination TEID, source IP address, destination IP address).

The GTP enables mobility and the GTP tunnel allows your data session to persist seamlessly as you move between different cells. The tunnel's endpoints can be re-anchored without changing your device's IP address.

2.2 Analysis on protocol stack selection

- Protocol stack for control signalling

SCTP is connection-oriented and enabling for multi-homing, these features allow a single SCTP association to use multiple IP addresses, providing network-level fault tolerance. If the primary network path fails, SCTP can automatically switch to an alternative path using one of the other IP addresses, ensuring the connection remains active. This feature improves reliability for both endpoints and is especially useful in scenarios like mobile networks where network links can fail. Therefore, SCTP is applicable when you need maximum performance and reliability for data transport within a private, controlled network.

Based on the reliability and connection-oriented characteristics of SCTP, control signaling transport in both 4G and 5G consistently adopts the SCTP protocol stack. Therefore, we propose that sensing-specific control signaling can also reuse the SCTP-based protocol stack.

Proposal 1: Reuse the NG-C protocol stack for transporting sensing-specific control signaling.

- Protocol stack for sensing data transport

The 5G NR system uses the GTP protocol stack for user data transportation. GTP is like the passport and visa system for international travel. It doesn't care what's inside user IP packets; its job is to create a "tunnel" that allows your data session to move between different cells or base stations while maintaining the application identity (IP address) and ensuring user data get to their destination.

In 5G networks, user data is primarily transmitted through the internet. The internet is a public network, which is more open and less controllable, making it suitable for user data transmission by GTP protocol.

But sensing data is transmitted between NG-RAN and CN via the operator-managed transmission network, not through internet. The data transmission inside the 5G network normally uses SCTP protocol stack.

For now, both the positioning data report and Ambient IoT data report use the SCTP protocol stack to transport between NG-RAN and 5GC.

Figure 3[7], Figure 4[8] and Figure 5[8] show the architecture of the positioning data transmission and Ambient IoT data transmission respectively.

Figure 3: The interface between gNB and 5GC for positioning data transmission

Figure 4: Protocol Stack between AIoT Device and 5GC for NG-RAN Indirect Connectivity

Figure 5: Protocol Stack between AIoT Device and 5GC for NG-RAN direct Connectivity

As can be seen from the above picture, the positioning data transmission and indirect interface Ambient IoT data transmission reuse the NG-C interface. The direct Ambient IoT data transmission uses the AIOT2 interface which is also embed into the NG-C interface. “NGAP AIoT Information is the subset of NGAP information and is included in the NGAP messages over AIOT2 reference point” [8].

Observation: From the current technological developments in 5G, all data transmitted between NG-RAN and 5GC that does not require internet-based data exchange is transported via SCTP.

Sensing data transmitted between gNB and 5GC can be divided into the following four options according to different products of sensing processing:

- Option 1: Sensing raw data

- Option 2: Delay/angular/Doppler spectrums

- Option 3: Cloud of points

- Option 4: Sensing result

From Option 1 to Option 4, the sensing accuracy progressively decreases while the required amount of measurement reporting data also reduces. This represents a trade-off between sensing data volume and sensing performance.

The adoption of delay/angular/Doppler spectrums (Option 2) for sensing data reporting would require a total data rate of 69 Mbps (8.6 MB/s) for 64 antenna ports per beam, assuming a 30 kHz subcarrier spacing. This data rate is considered manageable. We therefore conclude that the data rate requirements for Options 2, 3, and 4 are controllable, and the SCTP protocol is capable of supporting the transmission for all three sensing data types.

Proposal 2: The SCTP protocol could be capable of supporting the controllable sensing data transmission.

- Conclusion

This paper has discussed two potential RAN architectures able to fulfil the gNB-based sensing requirement. Based on the above discussion on the RAN sensing architecture, we provide the following observations and proposals:

Observation: From the current technological developments in 5G, all data transmitted between NG-RAN and 5GC that does not require internet-based data exchange is transported via SCTP.

Proposal 1: Reuse the NG-C protocol stack for transporting sensing-specific control signaling.

Proposal 2: The SCTP protocol could be capable of supporting the controllable sensing data transmission.

- Reference

- RP-252819, Study on Integrated Sensing And Communication (ISAC) for NR, Xiaomi, CTC.

- R3-257325, Discussion on Network Architecture for ISAC, ZTE Corporation.

- R3-257326, Discussion on Sensing procedures and singalling, Xiaomi.

- TS 38.412, NG signalling transport (Rel-19), 3GPP.

- TS 38.414, NG data transport (Rel-19), 3GPP.

- TS 29.281, General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U) (Rel-19), 3GPP.

- TS 38.305, Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN (Rel-19), 3GPP.

- TS23.369, Ambient power-enabled Internet of Things; Stage 2 (Rel-19), 3GPP.