3GPP TSG-RAN WG3 Meeting #130	R3-258359

Dallas, US, 17 – 21 November 2025

Agenda item:	13.3

Source: 	China Telecom, BUPT

Title: 	(TP to 38.473) On RAN-CN functions and procedures for supporting ISAC

Document for:	Discussion and decision

- Introduction

The first discussion on ISAC has taken in 3GPP RAN3#129bis meeting. Based on ISAC SI[1], the agreement on the basic sensing call flow was captured in agreed TP [2].

According to the guidance of last meeting, i.e. “To continue the discussion on the necessary interface functions and the detailed signaling procedures for ISAC.”, in this paper, we will discuss the RAN-CN interface functions and procedures for ISAC and propose our views and proposals.

- Discussion

2.1 RAN-CN Interface sensing functions

2.1.1 Basic sensing functions

Before we discuss the RAN-CN sensing interface signalling and procedures, we should first clarify the fundamental functions supported by this interface.

During the last meeting, we approved the support for sensing request and sensing report functions over the RAN-CN interface and established the basic sensing call flow.
In addition to the sensing request and report functions, some companies have proposed support for the following functions:

- Sensing Release Request: The purpose of the Sensing Release Request function is to enable the gNB to request the Sensing Function (SF) node to release the sensing service. This function should be supported by a gNB initiated Class-1 procedure.

- Sensing Release: The purpose of the Sensing Release function is to enable the SF node to terminate all activities related to the requested sensing service. This function should be supported by the SF initiated Class-2 procedure.

- Sensing Modification: The purpose of the Sensing Modification function is to request the gNB to update operation of the ongoing sensing service. This function should be supported by SF initiated Class-1 procedure.

- Sensing Capability Coordination: The gNB sends its sensing capability info to the Sensing Function for capability coordination. This functionality should be implemented through either a gNB-initiated Class-1 procedure or enhancements to the existing RAN-CN interface setup procedures.

The three functions—Sensing Release Request, Sensing Release, and Sensing Modification—should be considered fundamental functionalities and should be supported.

Proposal 1: The RAN-CN interface should support Sensing Release Request, Sensing Release, and Sensing Modification functionalities for ISAC.

Regarding Sensing Capability Coordination, we believe that sensing capability information contains substantial content—such as sensing abilities, sensing coverage ranges, and sensing categories—which constitutes part of the fundamental gNB information. This information should be exchanged with the core network SF during the interface establishment process, enabling the SF to identify suitable sensing gNBs when configuring sensing services based on these sensing capabilities.

Some companies think that sensing capability information should be managed and configured by OAM. However, if managed through OAM, it remains uncertain whether and how OAM should administer the SF within the core network. This is particularly critical in network sharing scenarios where a shared gNB needs to connect to SFs belonging to different operators' core networks. In such cases, the OAM of the shared base station is typically managed by the host operator. It remains unclear whether the sharing operator would agree to let the host operator's OAM notify its SF.

Therefore, direct interaction between the base station and the SF for sensing information exchange is more suitable than OAM-managed sensing information.

Proposal 2：The RAN-CN interface should support Sensing Capability Coordination function.

The Sensing Capability Coordination function can be implemented within the RAN-CN sensing interface setup procedure.

If the gNB and SF are connected via a non-direct interface, the sensing interface setup procedures can reuse the setup procedure of the NG interface to achieve Sensing Capability Coordination. If they are connected via a direct interface, then dedicated sensing interface setup procedures need to be designed, which should support the Sensing Capability Coordination function.

Proposal 3：The Sensing Capability Coordination function can be implemented through the RAN-CN sensing interface setup procedure.

2.2 Sensing procedures

During the previous meeting, some companies discussed whether it's appropriate to combine all procedures into a single flowchart or to create separate flowcharts for each procedure. Our view is that basic procedures such as sensing request, data reporting, sensing modification, and sensing release can be integrated into one flowchart while other sensing procedures can be illustrated in separate flowcharts.

Figure 1 provides a basic flowchart for sensing reporting.

Figure 1: Basic RAN-CN sensing procedure

Step 1. The SF sends sensing request to the gNB.

Step 2. The gNB sends sensing response to the SF.

Step 3. If requested in Step 1, the gNB sends sensing report to SF.

Step 4. If requesting the gNB to update the ongoing sensing operation, the SF sends sensing modification request to the gNB.

Step 5. The gNB sends sensing modification response to the SF.

Step 6. If requested in Step 4, the gNB sends sensing report to SF.

Step 7. If requesting the gNB to terminate the ongoing sensing operation, the SF sends sensing release to the gNB.

Proposal 4: It is proposed to include the basic RAN-CN sensing procedure in the Annex as part of TR 38.765.

- Conclusion

Based on the above discussion on RAN-CN functions and procedures for supporting gNB-based sensing, we provide the following proposals:

Proposal 1: The RAN-CN interface should support Sensing Release Request, Sensing Release, and Sensing Modification functionalities for ISAC.

Proposal 2：The RAN-CN interface should support Sensing Capability Coordination function.

Proposal 3：The Sensing Capability Coordination function can be implemented through the RAN-CN sensing interface setup procedure.

Proposal 4: It is proposed to include the basic RAN-CN sensing procedure in the Annex as part of TR 38.765.

- Reference

- RP-252819, Study on Integrated Sensing And Communication (ISAC) for NR, Xiaomi, CTC.

- R3-257326, Discussion on Sensing procedures and singalling, Xiaomi.

- Annex: TP for RAN3 TR38.765

It is proposed to capture the following text proposal in TR38.765:

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

8.x	 Sensing Reporting

Editor’s Note: the following may need further refinement.

Figure 8.x-1: Message flow for basic sensing report procedure

1. The SF sends sensing request to the gNB.

2. The gNB sends sensing response to the SF.

3. If requested in Step 1, the gNB sends sensing report to SF.

4. If requesting the gNB to update the ongoing sensing operation, the SF sends sensing modification request to the gNB.

5. The gNB sends sensing modification response to the SF.

6. If requested in Step 4, the gNB sends sensing report to SF.

7. If requesting the gNB to terminate the ongoing sensing operation, the SF sends sensing release to the gNB.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>