3GPP TSG-RAN WG3 #130	R3-258393

Dallas, U.S.A., 17-21 November 2025

Title: 	(pCR to TR 38.745) AI/ML based mobility – Intra-CU LTM

Source: 	Ericsson

Agenda item:	12.2.2

Document Type:	Discussion and Decision

# 1. Introduction

At the RAN3#129-bis meeting, the following agreements and FFS were captured for AI/ML-based Intra-CU LTM:

Prioritize normal intra-CU LTM scenarios.

For CU-DU split architecture, the following two scenarios are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

AI/ML assisted candidate cell selection for LTM Handover Preparation

Study candidate beams along with candidate cells for AI/ML based Intra-CU LTM HO preparation.

RAN3 can further discuss whether the model inference can be located at gNB-DU to support AI/ML-based intra-CU LTM.

TA value and validity prediction for Intra-CU LTM in spatial and/or temporal domain

For AI/ML study we consider both L1 and L3 measurement based Intra-CU LTM.

Further discuss predicting TA value in the next meeting.

Further discuss prediction of validity time of a measured TA value.

Further discuss whether to study addition of beam prediction in UE Trajectory prediction.

In this contribution we continue the discussion on AI/ML-based mobility enhancements for intra-CU LTM.

# 2. Discussion

## 2.1 Model deployment

At the RAN3#129-bis meeting, the group confirmed the reuse of the framework already established for AI/ML Model Training and AI/ML Model Inference functions’ location, namely that in case of CU-DU split, the inference is located at gNB-CU:

For CU-DU split architecture, the following two scenarios are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

At the meeting, some companies also proposed that, in some cases, predictions at the gNB-DU should be allowed for intra-CU LTM, which resulted in the following FFS: RAN3 can further discuss whether the model inference can be located at gNB-DU to support AI/ML-based intra-CU LTM.

As a general consideration, while it cannot be precluded that in some implementations the gNB-DU produces some data as an output of an inference (as discussed, for example in the context of CCO in Rel-19), we think that it is not a good idea that the standard supports inference at two different logical functions. Such an approach would require coordination between the two logical functions (the gNB-DU and the gNB-CU), for example to make sure that one action decided by one function is not undone or overridden by the other, to handle potential conflicts, or race conditions. This will make the overall solution unnecessarily complex to implement and test, which is not desirable.

However, as we stated, it cannot be precluded that the gNB-DU has inference capabilities in some implementations. In these cases, any of the information produced at the gNB-DU via inference is consumed within the node. Hence, this behavior does not have any RAN3 impact.

Finally, we would also like to note that the current SI has the following scope:

Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture …

Therefore, we propose to confirm that the standard will only support inference at the gNB-CU.

Proposal 1: For AI/ML based intra-CU LTM, the standard only supports model inference at the gNB-CU.

## 2.4 L1 and L3 measurements

The following FFS has also been captured:

For AI/ML study we consider both L1 and L3 measurement based Intra-CU LTM.

At LTM preparation, a UE can be configured with SSB beams or CSI-RS beams to measure. If the LTM execution is based on L3 measurements, the gNB-CU will decide to trigger it; if the LTM execution is based on L1 measurement, the source gNB-DU will decide to trigger it. From this point of view, both L3 measurement based intra-CU LTM and L1 measurements based intra-CU LTM needs to be discussed.

If LTM is based on L3 measurements, since the group already agreed that inference can be deployed at the gNB-CU, we propose to reformulate the following agreement: AI/ML assisted candidate cell selection for LTM Handover Preparation as “For AI/ML based intra-CU LTM based on L3 measurements, the gNB-CU can predict candidate cells for LTM Handover Preparation”.

Proposal 2: For AI/ML based intra-CU LTM based on L3 measurements, the gNB-CU can predict candidate cells for LTM Handover Preparation.

If LTM is based on L1 measurements, as said, the gNB-DU will decide to trigger LTM execution. However, we do not think that there is a need for using L1 measurements for predictions (for example to predict when the cell switch will happen). There are a number of aspects to consider which make the use of L1 measurements problematic in this respect. A first aspect is that, due to their nature, L1 measurements are subject to many fluctuations. Another aspect is that different UE implementations will use different L1 filtering, and it can be expected that if different UEs are configured for LTM based on L1 measurements, their reporting to the serving gNB-DU of the same radio environment will be different. An additional aspect is the fact that a UE can be configured to measure and report L1 measurements for LTM depending on the UE capability. In summary, we think that, for the scenario when L1 measurements can be collected, L1 predictions based on such measurements will likely be inaccurate, or it would likely require very fine time-granularity for the prediction, placing significant demand on the gNB-DU.

We propose then not to consider L1 measurements as input for LTM related predictions.

Proposal 3: Do not consider L1 measurements for predictions in AI/ML based intra-CU LTM.

## 2.2 Predictions related to Timing Advance

In relation to predictions related to Timing Advance, the following open points were captured at the last RAN3 meeting:

TA value and validity prediction for Intra-CU LTM in spatial and/or temporal domain

Further discuss predicting TA value in the next meeting.

Further discuss prediction of validity time of a measured TA value.

We first observe that a TA value is used in the LTM Cell Switch Command to indicate to the UE the TA to consider for the target cell. In the LTM functionality, the TA is calculated by the candidate (target) gNB-DU for the candidate (target) cell, and if the TA value provided in the LTM Cell Switch Command is incorrect, the LTM execution fails.

Observation 1: If a wrong TA value is used for an LTM cell switch execution, the execution fails.

To resolve the FFS, we first analyze the feasibility of predicting TA values, taking Proposal 1 into account.

The source gNB-DU cannot estimate the TA value of a UE towards a cell in another gNB-DU since it lacks the understanding of UE location in relation to the coverage of candidate cell. Hence, we do not think the source gNB-DU predicting the TA value is a viable solution.

The gNB-CU may be able to estimate TA values using L3 measurements and some knowledge about the UE position and movement; however, it is unclear whether these estimates would be accurate enough so that the LTM cell switch succeeds.

Finally, we note that, in case of intra-gNB-DU LTM, where the source gNB-DU is also the target gNB-DU, the gNB-DU could estimate the TA value (e.g., based on lower-layer signaling received from the UE). However, this will be just used for handling intra-gNB-DU LTM mobility, and, of course, it is always the source gNB-DU that sends the TA value to the UE in the LTM Cell Switch command. Even if the gNB-DU can produce a predicted TA value, we see no benefit in sending a predicted TA value to the UE, when the gNB-DU can simply send to the UE a calculated TA value when triggering the LTM execution.

Taking the above analysis into account, we propose that for AI/ML based intra-CU LTM, prediction of TA value is not supported in the standard.

Proposal 4: Standard does not support TA value prediction.

We analyze now whether it is feasible to predict the lifespan of measured TA, i.e., whether a measured TA value will be valid (or not valid) sometime in the future, and whether this requires standard impact.

As described before; to perform this type of prediction, the gNB-CU could use some knowledge of L3 measurements,  UE position and movement, but it is unclear whether these estimates would be accurate.

The situation may be different at the source gNB-DU. For inter-gNB-DU LTM, the source gNB-DU receives a TA value calculated by the target gNB-DU (via the gNB-CU) when early UL sync is executed. The TA value is based on the pathloss of the UE towards the target cell and, at the time when the TA value was acquired, the source gNB-DU also knows the pathloss between the UE and the source cell. After TA acquisition, the source gNB-DU may observe a variation in pathloss between the UE and source cell, and it may deduce that a similar variation in pathloss exists between the UE and the target cell. There is, then, a possibility for the source gNB-DU to estimate when this variation will be so large that the acquired TA value will not be valid anymore. The same possibility is also valid for the intra-gNB-DU LTM case. However, this is an implementation aspect, and the source gNB-DU does not need to receive any other information to do this (from gNB-CU or target gNB-DU). The source gNB-DU can then simply send a new PDCCH order to the UE and trigger a new early UL synchronization. The target gNB-DU and the gNB-CU will see the “effect” of a new TA acquisition, which is already supported in the non-AI/ML based solution. In other words, there are no RAN3 impacts in relation to TA validity prediction.

Considering the above analysis, we think there is no need to support TA validity prediction in the standard.

Proposal 5: Standard does not support TA validity prediction.

## 2.3 Beam-related prediction

At the last RAN3 meeting, beam-related predictions were discussed and the following agreement captured:

Study candidate beams along with candidate cells for AI/ML based Intra-CU LTM HO preparation.

However, the current version of the TR for AI/ML based Intra-CU LTM (R3-257331) erroneously captures the following highlighted sentence, which does not reflect the current stage of the discussion in RAN3:

#### 4.1.2.3		Output data of AI/ML based Intra-CU LTM

For AI/ML optimization of Intra-CU LTM the following information can be considered as output data:

- Candidate cell and beam for LTM HO Preparation

Depending on the progress of the discussion, the “beam” part may be considered or not considered as output. Therefore, we propose to remove it for now, and wait until the discussion is finalized.

Proposal 6: The “candidate beam” as output data for AI/ML based Intra-CU LTM is pending RAN3 discussion, and it should not be mentioned in the TR until agreed.

In relation to beam prediction, there is also the following FFS from the last meeting:

Further discuss whether to study addition of beam prediction in UE Trajectory prediction.

Regarding a possible addition of beam granularity in the prediction of UE Trajectory, we do not think this can provide reliable information. Consider, for example, that one beam can reach a UE via different paths, due to multiple reflections or diffractions. It will be very complex to predict the UE trajectory at beam level, and this will probably vary in time because there are many factors outside network control (including, e.g., changes in the environment) impacting the directions and the strength at which an individual beam reaches the UE.

Even if we assume that beam prediction is added in UE Trajectory prediction, it will be even harder to use this for deciding when it is time to trigger a handover. To predict, for example, that a mobility procedure will be triggered because the UE will move from a certain beam in one cell, to another beam in another (candidate) cell, it is not enough to predict that the UE will be able to receive the beam of the candidate cell, at some point in time in the future, but there is also a need to predict the coverage that the UE will experience in the future for the current serving cell (beam) and for the candidate cell (beam). Considering what has already been mentioned above, this looks very complex to achieve, and a prediction at cell level is sufficient.

Proposal 7: Do not support beam prediction in UE trajectory prediction.

Proposal 8: For AI/ML based Intra-CU LTM HO preparation, prediction of candidate cell is sufficient.

## 2.5 LTM execution

If the gNB-CU decides that LTM is based on Layer 3 measurements, using radio measurements collected from the UE, the gNB-CU will be able to deduce when the LTM execution will happen, and inform the source gNB-DU (e.g., extending the already defined CU-DU Mobility Initiation F1AP procedure), and/or the target gNB-DU (e.g., extending the already defined CU-DU Cell Switch Notification). In particular, the target gNB-DU needs to allocate some resources (e.g., Configured Grant) to serve the incoming UE. If the gNB-CU predicts the time when the LTM execution will happen, it can sends this information to the target gNB-DU, which can use this piece of information optimize its resource allocation.

Proposal 9: For L3 based intra-CU LTM, the gNB-CU can predict when LTM execution will occur and send the timing information to the target gNB-DU.

Proposal 10: RAN3 to agree to the pCR to TR 38.745 in Appendix A.

# 3. Conclusion

# In this contribution, the following observations and proposals are made:

Proposal 1: For AI/ML based intra-CU LTM, the standard only supports model inference at the gNB-CU.

Proposal 2: For AI/ML based intra-CU LTM based on L3 measurements, the gNB-CU can predict candidate cells for LTM Handover Preparation.

Proposal 3: Do not consider L1 measurements for predictions in AI/ML based intra-CU LTM.

Observation 1: If a wrong TA value is used for an LTM cell switch execution, the execution fails.

Proposal 4: Standard does not support TA value prediction.

Proposal 5: Standard does not support TA validity prediction.

Proposal 6: The “candidate beam” as output data for AI/ML based Intra-CU LTM is pending RAN3 discussion, and it should not be mentioned in the TR until agreed.

Proposal 7: Do not support beam prediction in UE trajectory prediction.

Proposal 8: For AI/ML based Intra-CU LTM HO preparation, prediction of candidate cell is sufficient.

Proposal 9: For L3 based intra-CU LTM, the gNB-CU can predict when LTM execution will occur and send the timing information to the target gNB-DU.

Proposal 10: RAN3 to agree to the pCR to TR 38.745 in Appendix A.

# Appendix A – pCR to TR 38.745 – Intra-CU LTM

## 4.2	AI/ML assisted Intra-CU LTM

### 4.2.1	Use case description

Editor’s Note: Capture the description of use case

L1/L2 Triggered Mobility (LTM) is specified in TS 38.300 [2].

Intra-CU LTM is specified in TS38.401[3].

AI/ML can be used to optimise Intra-CU LTM procedures, e.g., to enhance Network and UE performance, optimize resource allocation and reduce mobility failures.

### 4.2.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

For CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.1.2.2		Input data of AI/ML based Intra-CU LTM

Editor’s Note: To be updated

For AI/ML optimization of Intra-CU LTM the following information can be considered as input data:

From local node:

-	Measured/Predicted radio resource status

-     Measured/Predicted UE trajectory

-     LTM failures

From the UE

-    Radio measurements related to serving cell and neighbouring cells associated with UE location information, e.g., RSRP, RSRQ, SINR.

-	SON Reports (e.g., RLF, CEF, RA)

#### 4.1.2.3		Output data of AI/ML based Intra-CU LTM

For AI/ML optimization of Intra-CU LTM the following information can be considered as output data:

- Candidate cell for LTM HO Preparation

#### 4.1.2.4	Potential standard impact

Following standard impacts are listed for subsequent Rel-20 normative work:

F1 interface (gNB-CU to gNB-DU):

-	Predicted LTM candidate cell for next LTM cell switch.

-	Predicted time of LTM cell switch.