3GPP TSG-RAN WG3 #130	R3-258434

Dallas, U.S.A., 17-21 November 2025

Title: 	Abnormal conditions for F1: gNB-CU Configuration Update

Source: 	Ericsson, Jio Platforms, InterDigital, Fibercop

Agenda item:	9.2

Document Type:	Discussion and Decision

# Introduction

In this contribution we discuss some abnormal conditions for the gNB-CU Configuration Update procedure related to the Rel-19 work on AI/ML support for CCO.

# Discussion

In the discussion below we focus on the content of the F1: gNB-CU Configuration Update message and on how to correct some aspects of this message to reduce the number of abnormal conditions that should otherwise need to be addressed in the specifications.

## Mandatory presence of some IEs

A first aspect to consider is that within the Predicted CCO Assistance Information IE, a gNB-DU always needs to receive the Predicted CCO Issue IE and the Predicted Affected Cells and Beams IE. Without these IEs, the gNB-DU cannot understand which type of predicted CCO issue (or cancellation) is being signalled, nor the cells and beams predicted to be affected.

These IEs are currently optional, but if the presence of these IEs is changed from optional to mandatory, the number of potential abnormal conditions is reduced, for example abnormal conditions in which only one of the IE is present, while the other is absent. For these reasons, we propose to change the presence of the Predicted CCO Issue IE and Predicted Affected Cells and Beams IE to mandatory.

Proposal 1: Change the presence of the Predicted CCO Issue IE and the Predicted Affected Cells and Beams IE in the Predicted CCO Assistance Information IE to mandatory.

## Missing information

An abnormal condition arises when a gNB-DU receives a F1: GNB-CU CONFIGURATION UPDATE message including the Neighbour Future Coverage Modification Notification IE, and the Predicted CCO Assistance Information IE is not present.

The gNB-DU cannot understand which action to take with its own cells and beams, because it cannot link the cells and beams included in Neighbour Future Coverage Modification Notification IE with any predicted CCO issue (new or to be cancelled). If this abnormal condition occurs, our proposal is for the gNB-DU to discard the Neighbour Future Coverage Modification Notification IE in this case.

Proposal 2: When a gNB-DU receives a F1: GNB-CU CONFIGURATION UPDATE message which includes the Neighbour Future Coverage Modification Notification IE, but the Predicted CCO Assistance Information IE is not present, the gNB-DU shall discard the Neighbour Future Coverage Modification Notification IE.

Another abnormal condition arises when a gNB-DU not serving the predicted affected cells and beams receives a F1: GNB-CU CONFIGURATION UPDATE message including a Predicted CCO Assistance Information IE, but the Neighbour Future Coverage Modification Notification IE is not present.

The gNB-DU cannot understand how it may adapt to the predicted CCO issue because of the missing information. The gNB-DU shall discard the Predicted CCO Assistance Information IE.

Proposal 3: When a gNB-DU not serving the predicted affected cells and beams receives a F1: GNB-CU CONFIGURATION UPDATE message which includes the Predicted CCO Assistance Information IE, but the Neighbour Future Coverage Modification Notification IE is not present, the gNB-DU discards the Predicted CCO Assistance Information IE.

## Non-matching information

Another abnormal condition arises when a predicted CCO issue is being cancelled, and the gNB-DU receives a F1: GNB-CU CONFIGURATION UPDATE message including the Predicted CCO Assistance Information IE where the Predicted CCO issue IE is set to “cancel” but the list of cells and beams in the Predicted Affected Cells and Beams IE does not match the list of cells and beams received in a previous Predicted CCO Assistance Information IE. In this case, the gNB-DU cannot understand that the predicted CCO issue being cancelled refers to the same cells and beams for which a future coverage was produced. The gNB-DU should discard the Predicted Affected Cells and Beams IE, and the cancelling is not executed.

Proposal 4: When a gNB-DU receives a F1: GNB-CU CONFIGURATION UPDATE message including the Predicted CCO Assistance Information IE with the Predicted CCO issue IE set to “cancel”, but the list of cells and beams is not the same as the list of cells and beams included in a previously received Predicted CCO Assistance Information IE, the gNB-DU discards the Predicted CCO Assistance Information IE, and the cancelling is not executed.

Another abnormal condition exists when a gNB-DU not serving the predicted affected cells and beams receives a F1: GNB-CU CONFIGURATION UPDATE message including both the Predicted CCO Assistance Information IE and the Neighbour Future Coverage Modification Notification IE, but the list of cells and beams in the Predicted Affected Cells and Beams IE not served by the gNB-DU does not match the list of cells and beams contained in the Neighbour Future Coverage Modification Notification IE. Since the lists of cells and beams are not the same, some of the possible interpretations of this message would be that:

- there are cells for which there is a future coverage modification in Neighbour Future Coverage Modification Notification for which the gNB-DU does not receive a corresponding predicted CCO issue, or

- there are cells in the Predicted Affected Cells and Beams IE not served by the gNB-DU, for which the gNB-DU does not know what the future coverage modification will be (and towards which it may need to adapt)

In both cases, the gNB-DU cannot deduce an unequivocal action to take. It is therefore proposed that the gNB-DU should discard both the Predicted CCO Assistance Information IE and the Neighbour Future Coverage Modification Notification IE.

Proposal 5: When a gNB-DU not serving the predicted affected cells and beams receives a F1: GNB-CU CONFIGURATION UPDATE message including both the Predicted CCO Assistance Information IE and the Neighbour Future Coverage Modification Notification IE, but the list of cells and beams in the Predicted Affected Cells and Beams IE not served by the gNB-DU is not the same as the list of cells and beams contained in the Neighbour Future Coverage Modification Notification IE, the gNB-DU discards both the Predicted CCO Assistance Information IE and the Neighbour Future Coverage Modification Notification IE.

# Conclusion

In this contribution, the following proposals are made:

Proposal 1: Change the presence of the Predicted CCO Issue IE and the Predicted Affected Cells and Beams IE in the Predicted CCO Assistance Information IE to mandatory.

Proposal 2: When a gNB-DU receives a F1: GNB-CU CONFIGURATION UPDATE message which includes the Neighbour Future Coverage Modification Notification IE, but the Predicted CCO Assistance Information IE is not present, the gNB-DU shall discard the Neighbour Future Coverage Modification Notification IE.

Proposal 3: When a gNB-DU not serving the predicted affected cells and beams receives a F1: GNB-CU CONFIGURATION UPDATE message which includes the Predicted CCO Assistance Information IE, but the Neighbour Future Coverage Modification Notification IE is not present, the gNB-DU discards the Predicted CCO Assistance Information IE.

Proposal 4: When a gNB-DU receives a F1: GNB-CU CONFIGURATION UPDATE message including the Predicted CCO Assistance Information IE with the Predicted CCO issue IE set to “cancel”, but the list of cells and beams is not the same as the list of cells and beams included in a previously received Predicted CCO Assistance Information IE, the gNB-DU discards the Predicted CCO Assistance Information IE, and the cancelling is not executed.

Proposal 5: When a gNB-DU not serving the predicted affected cells and beams receives a F1: GNB-CU CONFIGURATION UPDATE message including both the Predicted CCO Assistance Information IE and the Neighbour Future Coverage Modification Notification IE, but the list of cells and beams in the Predicted Affected Cells and Beams IE not served by the gNB-DU is not the same as the list of cells and beams contained in the Neighbour Future Coverage Modification Notification IE, the gNB-DU discards both the Predicted CCO Assistance Information IE and the Neighbour Future Coverage Modification Notification IE.

Proposal 6 Agree to the CR for TS 38.473 in R3-258435.