3GPP TSG-RAN WG3 Meeting #130	R3-258514

Dallas, USA, 17 – 21 November, 2025

Agenda Item:	9.2.2

Source:	Nokia

Title:	Discussion on Exit Conditions

Document for:	Decision

# 1	Introduction

In this paper we provide our views related to the existing exit conditions defined in TS 38.423 and provide some proposed updates. The corresponding CR is in R3-258172.

# 2	Exit Conditions for UE Performance Feedback

In the last meeting, we agreed CR R3-257243 with some changes to the exit conditions used to report UE Performance feedback. In particular, the following updates were agreed:

In our view, we still need to make some corrections on some aspects. We discuss those corrections next.

UE Performance feedback is reported after a UE is handed over to the target node and is used to evaluate the handover decision. This is done by requesting the target gNB to measure different UE performance metrics (e.g UL/DL Average Throughput, UL/DL Average Delay) per-DRB and aggregate/consolidate them per-UE before reporting them back to the source node. The observed UE Performance will depend on UE configuration including its bearer configuration and radio configuration, and therefore not only on the UE’s radio conditions in the target cell.

Observation 1: UE Performance measurements depend on UE configuration including its bearer configuration and radio configuration.

In addition, in RAN3 #129, it was agreed to introduce Data Collection ID IE in the E1AP BEARER CONTEXT MODIFICATION REQUEST message and allow collection of UE Performance Delay for new bearers to be setup in UE CONTEXT MODIFICATION REQUEST message over F1AP. This was agreed in order to allow a gNB to collect UE performance even for newly added DRB(s). An example where this could be useful is if a source node (before a handover) needs to set up additional bearers and needs to be able to measure UE Performance on those in order to compare the UE Performance before the handover to the UE Performance after the handover event.

Observation 2: Measuring UE Performance Delay for new bearers to be setup is useful in certain scenarios e.g., when a source node establishes new UE bearers for a UE before a handover and needs to measure the overall UE Performance Delay before and after the handover for its evaluation.

However, even though this update in F1AP makes sense for the source node of a handover, if the target node modifies the UE configuration (e.g. adds or removes a bearer) it would create a situation where the source node can’t compare the reported UE Performance Delay with its own observations when it served the UE. This is because the source cannot determine if the UE bearer configuration at the target e.g., with respect to the number of bearers has been changed during the data collection. Note that other configuration changes at the target node can create similar issues, e.g. if the target node sets up or deactivates carrier aggregation while the UE performance reporting is ongoing. This will create an issue of incomparability of the reported UE Performance at the source node.

Observation 3: If the UE bearer configuration at the target node, e.g., with respect to the number of bearers, changes, the source node cannot compare the reported UE Performance Delay by the target to the corresponding UE Performance it measured.

We think that this issue needs to be corrected. We identify two options:

- Option 1: New exit condition(s). Similarly to the other exit conditions that we have introduced, the target gNB can monitor if there is any change e.g., in the number of bearers while data collection is ongoing and terminatethe data collection in that case. This option is simple in that it does not require any special handling at the source node.

- Option 2: Continue to report the data with an indication at the source. Another alternative is that the target continues the data collection after the configuration changes mentioned above, but in that case, it would need to notify the source, otherwise the UE Performance received from the target node is under a different configuration from what the source assumes.

Proposal 1: Introduce option 1 to enable the target node to terminate data collection if the UE Configuration at the target node after the handover and during the data collection changes.

Another issue relates to the current way that NR-DC related exit conditions were incorporated in the general data collection termination conditions. In Rel-18 we had agreed that UE Performance terminates when UE is handed over to a subsequent cell, and therefore agreed the following exit condition: “UE is handed over to another cell”. This is due to the fact that for UE Performance we wanted to monitor UE behavior soon after a handover.

However, the same was not agreed for SN Addition. When Dual Connectivity is enabled, the MN needs to determine that the activation of Dual Connectivity is a good decision or a bad decision. Since PSCell selection is at the discretion of the SN it doesn’t matter from MN point of view if SN subsequently chooses different cells to (better) support Dual Connectivity. What matters to the MN is the overall UE Performance under Dual Connectivity. However, we believe there is some risk that the term “handed over” can be interpreted to include PSCell change, and we therefore propose to clarify that only PCell change is in scope of this exit condition.

Proposal 2: Update the exit condition “UE is handed over to another cell” to clarify that only PCell change is within the scope of the exit condition.

# 3	Conclusion

We make the following observations and proposals:

Observation 1: UE Performance measurements depend on UE configuration including its bearer configuration and radio configuration.

Observation 2: Measuring UE Performance Delay for new bearers to be setup is useful in certain scenarios e.g., when a source node establishes new UE bearers for a UE before a handover and needs to measure the overall UE Performance Delay before and after the handover for its evaluation.

Observation 3: If the UE bearer configuration at the target node, e.g., with respect to the number of bearers, changes, the source node cannot compare the reported UE Performance Delay by the target to the corresponding UE Performance it measured.

Proposal 1: Introduce option 1 to enable the target node to terminate data collection if the UE Configuration at the target node after the handover and during the data collection changes.

Observation 2: Measuring UE Performance Delay for new bearers to be setup is useful in certain scenarios e.g., when a source node establishes new UE bearers for a UE before a handover and needs to measure the overall UE Performance Delay before and after the handover for its evaluation.

Proposal 2: Update the exit condition “UE is handed over to another cell” to clarify that only PCell change is within the scope of the exit condition.