3GPP TSG-RAN WG3 Meeting #130	R3-258517

Dallas, U.S.A., November 17th – 21st, 2025

Agenda Item:	11.2.1

Source:	Ericsson

Title:	MRO for inter-gNB LTM

Document for:	Discussion and decision

# 1	Introduction

One of the main objectives of the SON/MDT Rel-20 WID [RP-252560] is to enhance the MRO features to optimize the LTM cell switch procedure.

- MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] If needed, co-operate with RAN1, SA5.

This contribution discusses potential MRO enhancements for inter-CU LTM cell switch.

# 2	Discussion

## 2.1	Definition and detection of connection failure

For inter-CU LTM, the definition and detection of connection failure was discussed in RAN3#129bis, and the following agreements were reached:

For inter-CU LTM, the definition of connection failure cases in 38.300 are taken as base line. For inter-CU LTM, the detection mechanism of connection failure cases in 38.300 are taken as base line.

In TS 38.300, when describing LTM related connection failure, there is no distinction between intra-CU and inter-CU LTM. For example, the following excerpt is cited as the definition of connection failure:

In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell.

- The current definition and detection of connection failure in TS 38.300 is applicable to inter-CU LTM.

Therefore, the current definition and detection of LTM related connection failure in TS 38.300 is naturally applicable to inter-CU LTM as well, without requiring any modification.

No modification is needed for stage 2 description on definition and detection of inter-CU LTM related connection failures.

## 2.2	Failure due to wrong beam and outdated TA

In the previous RAN3#129bis meeting, the use case of failure due to wrong beam and outdated TA was agreed to be supported, and the Rel-19 MRO solution should be used as baseline.

1. Use cases At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for inter-CU LTM, i.e.: BFR shortly after successful LTM cell switch, failure due to wrong beam and outdated TA; LTM connection failure: too late and too early LTM cell switch, LTM cell switch to wrong cell; Near failure case: SHR successful HO Successful case: ping/pong 2. Solution options Rel-19 MRO solutions for intra-CU LTM are taken as baseline for Rel-20 MRO, if applicable.

In Rel-19 MRO solution for failure due to wrong beam and outdated TA, the target gNB-DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell. The target gNB-DU sends the re-established or recovery beam information, the TA value used at successful RACH-based access, or both to the source gNB-DU via the gNB-CU.

The re-established or recovery beam information and the TA value used at successful RACH-based access should also be transferred in XnAP to extend the Rel-19 MRO solutions to support inter-CU LTM case.

Regarding the signalling in XnAP, we think the FAILURE INDICATION message is more suitable than ACCESS AND MOBILITY INDICATION message as the Failure Indication procedure is designed to transfer information regarding RRC re-establishment attempts, or received RLF Reports, between NG-RAN nodes. The FAILURE INDICATION message signalling takes place from the NG-RAN node at which a re-establishment attempt is made, or an RLF Report is retrieved, to an NG-RAN node to which the UE concerned may have previously been attached prior to the connection failure.

The beam and TA information for the failure due to wrong beam or outdated TA should be transmitted from the node of a re-establishment or recovering attempt to the source node, which is exactly the purpose and the message transfer as the FAILURE INDICATION message.

Use FAILURE INDICATION XnAP message to transmit beam and TA information for failure due to wrong beam and outdated TA.

## 2.3	BFD/BFR shortly after successful LTM cell switch

The case of BFD/BFR shortly after successful LTM cell switch was also agreed to be supported, and the Rel-19 MRO solution should be used as baseline as per agreement during the previous RAN3 meeting. Therefore, the recovery beam information should also be transferred using XnAP to extend the Rel-19 MRO solutions to support the inter-CU LTM case.

To align and leverage the specification and implementation efforts for the case of failure due to wrong beam and outdated TA, the same XnAP message is better to be used for the case of BFD/BFR shortly after successful LTM cell switch.

Use FAILURE INDICATION XnAP message to transmit the recovery beam information for BFD/BFR shortly after successful LTM cell switch.

For intra-CU LTM, the CU always has the UE context when a BFR happens shortly after successful LTM cell switch. However, after a successful inter-CU LTM execution, the source CU has the possibility to release the UE context. The configured time threshold for detecting BFR shortly after successful LTM cell switch, e.g. Tstore_UE_cntxt can be quite long. Below is the allowed range defined in TS 28.541, cl. 4.4.1, for Tstore_UE_cntxt for a reference.

Hence, the source CU may be required to retain the UE context for an extended period for MRO purposes solely in case an BFD/BFR issue arises afterwards, which could lead to additional costs. We propose that RAN3 discuss the topic of receiving FAILURE INDICATION messages where the source node has released the UE context for this LTM cell switch.

RAN3 to discuss BFR shortly after successful LTM cell switch, when the source gNB does not retain the UE context after successful inter-CU LTM execution.

## 2.4	LTM cell switch ping-pong

In Rel-19 MRO for intra-CU LTM cell switch, a CU can detect a potential LTM ping-pong issue based on local information and can send relevant information to the DU whenever the CU decides this is necessary. Currently, UHI information is included in HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE XnAP messages. During inter-CU LTM initial preparation, the HANDOVER REQUEST with the UHI information is transmitted to all the gNBs that have inter-CU LTM candidate cells. For a subsequent inter-CU LTM cell switch, the gNBs of the subsequent LTM candidate cell may not have an up-to-date UHI information about the cells visited by the UE after the LTM initial preparation phase. Without this up-to-date UHI information, the involving CU may not be able to detect a potential LTM ping-pong issue, and may not be able to send the last consecutive LTM cell switch entries from the UHI to the related DU.

- A candidate gNB for Inter-CU LTM may lack up-to-date UHI information after LTM initial preparation phase.

In order to leverage the Rel-19 MRO solution for LTM cell switch ping-pong, RAN3 should discuss and agree on whether and in what way inter-CU LTM candidate gNBs can be provided with up-to-date UHI information.

RAN3 to discuss whether and how inter-CU LTM candidate gNBs can be provided with up-to-date UHI information.

## 2.5	Too early cell switch, too late cell switch, and cell switch to wrong cell

In Rel-19 MRO for intra-CU LTM cell switch, there are provisions for sending information about the failure cases:

- Too early cell switch

- Too late cell switch

- Cell switch to wrong cell

For intra-CU LTM, RAN3 developed a solution using the F1 message ACCESS AND MOBILITY INDICATION for transferring information about these failure cases from CU to DU. For inter-CU LTM, RAN3 needs to develop a solution for transferring that information over Xn.

For legacy handover reporting over Xn, the FAILURE INDICATION and HANDOVER REPORT messages are used. We propose to reuse these messages the same way as for legacy handover.

Use FAILURE INDICATION and HANDOVER REPORT XnAP messages to transfer failure information for too early cell switch, too late cell switch, and cell switch to wrong cell.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The current definition and detection of connection failure in TS 38.300 is applicable to inter-CU LTM.

Observation 2	A candidate gNB for Inter-CU LTM may lack up-to-date UHI information after LTM initial preparation phase.

Based on the discussion in the previous sections we propose the following:

Proposal 1	No modification is needed for stage 2 description on definition and detection of inter-CU LTM related connection failures.

Proposal 2	Use FAILURE INDICATION XnAP message to transmit beam and TA information for failure due to wrong beam and outdated TA.

Proposal 3	Use FAILURE INDICATION XnAP message to transmit the recovery beam information for BFD/BFR shortly after successful LTM cell switch.

Proposal 4	RAN3 to discuss BFR shortly after successful LTM cell switch, when the source gNB does not retain the UE context after successful inter-CU LTM execution.

Proposal 5	RAN3 to discuss whether and how inter-CU LTM candidate gNBs can be provided with up-to-date UHI information.

Proposal 6	Use FAILURE INDICATION and HANDOVER REPORT XnAP messages to transfer failure information for too early cell switch, too late cell switch, and cell switch to wrong cell.