3GPP TSG-RAN WG3 #130								R3-258572

Dallas, US, 17 – 21 November 2025

Agenda Item:	10.3.1

Source:	China Telecom

Title: 	Discussion on Principles and Requirements for 6G RAN-CN Interface

Document for:	Discussion

Introduction

In RAN3#129bis meeting, the following principles and functionalities were captured in TR 38.760-3.

The general principles for the specification of the 6G RAN-CN interface are as follows:

-	the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN;

-	the 6G RAN-CN interface supports control plane and user plane separation;

-	the 6G RAN-CN interface supports future enhancements;

-	the 6G RAN-CN interface supports all possible RAN deployment scenarios;

-	the 6G RAN-CN interface supports RAN sharing between multiple operators;

-	the 6G RAN-CN interface supports the operation of network slicing;

-	the 6G RAN-CN interface supports enhanced service awareness in RAN;

-	the 6G RAN-CN control plane interface supports reliable signalling transmission;

-	the 6G RAN-CN interface is designed with a clear functional split between RAN and CN.

RAN-CN control plane interface supports following functions:

-	UE context management	: The functionality to manage UE context between the RAN and CN;

-	Transport of NAS messages: The functionality to transfer NAS messages between the CN and UE, subject to SA2 progress;

-	PDU Session Management: The functionality to manage PDU sessions and respective RAN resources, subject to SA2 progress.

This contribution aims to further discuss the general principles and requirements for RAN-CN interface.

Discussion

General Principles for 6G RAN-CN interface

In the 5G architecture, a RAN node connects to the AMF over the NG-C interface and to the UPF via the NG-U interface. A single NG-RAN node may establish multiple NG-C logical interfaces toward different AMFs. The NG-C interface supports two types of signalling services between the NG-RAN node and the AMF: non-UE-associated and UE-associated services. Non-UE-associated services relate to the entire NG interface instance between the NG-RAN node and the AMF and utilize a corresponding non-UE-associated signalling connection. UE-associated services, on the other hand, are specific to a particular UE and are supported by a dedicated UE-associated signalling connection maintained for that UE. From a control plane perspective, the fundamental principles governing the NG-C interface remain applicable to the 6G RAN-Core Network control plane interface. Furthermore, it is understood that both non-UE-associated and UE-associated services will continue to be supported in the 6G interface.

The NG User Plane (NG-U) interface is established between an NG-RAN node and a UPF. This interface serves to transport user plane PDUs (for both PDU Sessions and MBS Sessions) on a non-guaranteed delivery basis. Furthermore, a single NG-RAN node may maintain multiple logical NG-U interfaces toward the 5G Core.

Proposal 1: 6G RAN-CN interface is open and logical interface. If P2P interface is still supported in 6G, one 6G RAN node can establish multiple RAN-CN logical interfaces toward different CN.

Proposal 2: 6G RAN-CN control plane interface supports non-UE-associated services and UE-associated services.

Proposal 3: 6G RAN-CN user plane interface supports to transport user plane PDUs between RAN node and CN.

Current 5G standards support a diverse range of services, some, such as URLLC and V2X. For 6G, the focus should shift towards enhancing widely deployed services like eMBB and Cellular IoT, optimizing them with an acceptable performance-complexity trade-off. Conversely, continuing support for existing yet underutilized services should require strong market justification. The 6G system will also natively support a new suite of services, including AI/ML, sensing, and Ambient IoT. Given that these new services may involve independent functional entities within the core network, direct control plane interfaces could be established between the 6G RAN node and IoT function entity. This architectural approach would significantly reduce signalling load and control plane latency.

Proposal 4: 6G RAN-CN interface supports a direct connection interface between 6G RAN node and IoT function entity in CN.

To fully support new service requirements, the 6G RAN-CN interface must be designed with new principles from the start. This new requirement includes the RAN's role in collecting non-user-plane data (e.g., from AI and sensing) for the Core Network. The legacy user plane interface, being solely for PDU and MBS Sessions, cannot accommodate this data. A new, direct RAN-CN interface is therefore essential in 6G to transport non-session-based data.

Proposal 5: A new, direct RAN-CN interface is therefore essential in 6G to transport non-session-based data.

In last RAN3 meeting, the following agreement on 6G RAN-CN interface has been made:

- the 6G RAN-CN interface supports all possible RAN deployment scenarios;

The 6G system is designed to facilitate multiple deployment scenarios, such as macro and suburban. With the foundation of mobile base station (e.g., WAB) support established in Rel-19, 6G will extend this capability to mobile RAN nodes. The transport layer for the RAN-CN interface will be agnostic, operating over both wired and wireless media. A key functional requirement is the ability to detect breaks in the signalling connection.

Proposal 6: 6G RAN-CN interface natively supports mobile RAN nodes, evolving from mobile base station capabilities. And the transport layer of 6G RAN-CN interface will be agnostic, supporting both wired and wireless links.

RAN-CN Control Plane Interface functions

Furthermore, 6G RAN-CN control plane interface also needs to support following functions:

- RAN-CN interface management: to establish, maintain and release RAN-CN part of PDU sessions;

- Mobility function: to perform intra-RAT handover and inter-RAT handover

The support of additional functions should wait the progress of RAN2 and SA2.

Proposal 7: to capture the following 6G RAN-CN function in TR 38.760-3.

- RAN-CN interface management: to establish, maintain and release RAN-CN part of PDU sessions;

- Mobility function: to perform intra-RAT handover and inter-RAT handover

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: Proposal 1: 6G RAN-CN interface is open and logical interface. If P2P interface is still supported in 6G, one 6G RAN node can establish multiple RAN-CN logical interfaces toward different CN.

Proposal 2: 6G RAN-CN control plane interface supports non-UE-associated services and UE-associated services.

Proposal 3: 6G RAN-CN user plane interface supports to transport user plane PDUs between RAN node and CN.

Proposal 4: 6G RAN-CN interface supports a direct connection interface between 6G RAN node and IoT function entity in CN.

Proposal 5: A new, direct RAN-CN interface is therefore essential in 6G to transport non-session-based data.

Proposal 6: 6G RAN-CN interface natively supports mobile RAN nodes, evolving from mobile base station capabilities. And the transport layer of 6G RAN-CN interface will be agnostic, supporting both wired and wireless links.

Proposal 7: to capture the following 6G RAN-CN function in TR 38.760-3.

- RAN-CN interface management: to establish, maintain and release RAN-CN part of PDU sessions;

- Mobility function: to perform intra-RAT handover and inter-RAT handover

Reference

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

- RP-252870,pCR for 38.914 on requirements for architecture and migration for 6G, NTT DOCOMO, INC. (Moderator)

- RP-252948,pCR for 38.914 on Requirements of new and existing services for 6G,CMCC (moderator)