3GPP TSG-RAN WG3 Meeting # 130	R3-258635

Dallas, US, 17 – 21 November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1595 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to XnAP on CLI Indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, China Telecom, Pengcheng Laboratory, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN3#129 meeting, RAN3 finally agreed to use CLI Indication message for either CLI measurements reporting or SRS resource configuration information request between gNBs. But based on the current tabular design of this message, the CLI Measurement Result IE is mandatory which is not fully reflect the RAN3 agreement or ASN.1 design. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To fulfill the RAN3 agreement and ASN.1 design, correct the CLI Measurement Result range in the tabular from 1 to 0..1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The range of CLI Measurement Result in the CLI Indication message does not reflect RAN3 agreement or ASN.1 design. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.3.34 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.473 CR 1636 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Resubmit to RAN3#130. |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>>

9.1.3.34	CLI INDICATION

This message is sent by NG-RAN node1 to NG-RAN node2 to report the results of the CLI measurements or to indicate the need for SRS Resource Configuration information.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| CLI Measurement Result |  | 0..1 |  |  | YES | ignore |
| >CLI Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>SSB Index | O |  | INTEGER (0..63, …) | Strongest DL SSB beam information | – |  |
| >>NZP CSI-RS Resource Indication | O |  | INTEGER (1..64, …) | Strongest DL NZP CSI-RS beam information. The value is a relative index of the CSI-RS resources within the set of resources signalled. | – |  |
| >>CLI Mitigation Indication | O |  | ENUMERATED (true, …) | Indicates to request CLI mitigation. | – |  |
| SRS Resource Indication | O |  | ENUMERATED (true, …) | Indication to NG-RAN node2 that SRS-Resource configuration information is needed | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>