# 3GPP TSG-RAN WG3 Meeting #130	R3-258638

Dallas, US, 17-21 November , 2025

# Agenda Item:	10.3.2

# Source:	ZTE Corporation

# Title:	Consideration on 6G RAN-CN interface options

# Document for:	Discussions & Approval

1	Introduction

In the new 6G SI in RP-251881, the following objective was identified from mainly in RAN3’s scope.

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

This contribution provide our initial views on 6G RAN-CN interface options.

2	Discussion

2.1	Overview on NR interfaces between RAN and 5GC

In current 5G system, the NG-RAN architecture is illustrated in Figure 2-1 below.

Figure 2-1-1: Overall Architecture

As we know, NG Interface is the logical connection bridge between RAN node and 5G core network (5GC), which is designed to separate the control plane and user plane functions. It mainly includes two parts. The NG-C interface is responsible for control plane communication between RAN node and the AMF through the Next Generation Application Protocol (NGAP). For the reliable transport of signalling messages, SCTP is added on top of IP and provides guaranteed delivery of application layer messages. The NG-U interface is responsible for user plane data transmission between RAN node and UPF through IP transport and UDP/IP protocol.

On the other hand, service based concept is introduced in 5G CN, SBA divides the functional modules in the traditional core network into independent and reusable services. In SBA, each network function entity (NF) in the core network is decoupled and split at the functional level. These NFs can operate independently without relying on each other, and can be upgraded and elastically adjusted independently. In order to help different NFs to communicate with each other, the Network Repository Function (NRF) is introduced and it is responsible for the service registration management of NFs and provides a service discovery mechanism. Then, as a service consumer, an NF can find the applicable target NF through the NRF.

- In 5G, one Point to point (P2P) interface is defined between RAN and CN, and service based interfaces (SBI) are used for NFs in CN.

- In addition, the network capability exposure mechanism is a core component of the SBA according to which the capabilities and resources of the 5G core network can be exposed to external entities (such as third party application providers, vertical industry platforms) and internal network functions (NFs). In 5G networks, the Common API Framework (CAPIF) and the Network Exposure Function (NEF) are both responsible for implementing this mechanism. CAPIF provides a set of standardized interfaces and frameworks, which is responsible for the interaction with external application functions (AFs), authenticates and authorizes API invokers, and manages API information while NEF provides 5G network capabilities to Application Functions (AFs) through SBA.

- In 5G networks, the CAPIF and the NEF are closely related. CAPIF is a standard capability exposure framework defined by 3GPP, which provides a functional architecture for API callers to access and invoke service APIs. NEF is a network element responsible for network capability exposure in 5G, and provides 5G network capabilities to Application Functions (AFs) through service based architectures. It interacts with internal network elements of 5GC through south bound interfaces and provides open API interfaces to AFs through north bound interfaces. CAPIF provides a basis for the standardized operation of NEF in capability exposure, and NEF is the specific implementer of capability exposure under the CAPIF framework.

- In 5G, network capability exposure mechanism is supported by Common API Framework (CAPIF) and the Network Exposure Function (NEF) based on SBA in CN.

2.2	6G RAN-CN interface options

The following three options have been proposed for 6G RAN-CN interface based on companies’ contributions:

Opt1: Legacy P2P interface

Opt2: Service Based Interface

Opt3: Enhanced P2P interface

2.2.1 Legacy P2P interface

As we know, the P2P connection between RAN and CN is a traditional connection method inherited from the 4G/5G. The design concept of P2P connection is based on dedicated resource allocation and fixed topology, which could meet the requirements in the network environment at that time but lacks flexibility when facing the diverse service demands of 5G and 6G.  Main Characteristics of P2P interface is as below:

- CP Signaling Path:

In 5G, all Control Plane  signaling between the RAN node and NF in the CN shall be routed via the AMF , which serves as an anchor node. For example, to transfer CP signaling such as UE’s session request between UE and CN, the path should be:  UE → RAN → AMF → Target NF (e.g., SMF), with no shortcuts or dynamic rerouting. In 6G, it can assume that more NFs will be introduced in CN side in order to support 6G new services, how to achieve the direct connection between RAN and NFs as UE → RAN → Target NF (e.g., Sensing, AI) without routing delay needs to be solved.

- Protocol Stack:

Built on IP transport with SCTP (Stream Control Transmission Protocol) at the transport layer (above IP) to ensure reliable, ordered delivery of CP messages. The application layer uses 3GPP-specific protocols (e.g., NGAP for NG-C) to encapsulate CP signaling (e.g., PDU session establishment, paging).

Figure 2.2-1: existing NG-U and NG-C Protocol Stack

- Key Advantages

Mature 3GPP standardization, backward compatibility with 5G, and predictable signaling paths for legacy communication functions.

Considering  the traditional 5G P2P NG interface is primarily designed to support functions such as interface management, UE context management, mobility handling, NAS transport, paging, PDU session management. Notably, these capabilities have no inherent requirement for external exposure or invocation. Considering the costs and efficiency of network evolution, reusing the existing P2P interface for implementing these well-established and basic communication functions can leverage mature infrastructure, minimizes redevelopment efforts, and avoids unnecessary operational disruptions.

- Challenges

All RAN to NF/AF signaling is routed via AMF requiring AMF upgrade every time a service added, and it may lead to AMF overload, and the service latency will be increased due to unnecessary routing.

Different interface and signalling design for different 6G new services and it’s difficult to support customized RAN information retrieval for third-party applications.

Complex fault isolation due to intermediate entities, and all the intermediate entities have to be upgraded even they are not participating in the new service.

Furthermore, how to ensure the security of RAN information transmission (not UE traffic data) between RAN and NF needs to be studied.

- Proposal 1: Legacy P2P interface for implementing these well-established and legacy communication functions can leverage mature infrastructure, minimizes redevelopment efforts, and avoids unnecessary operational disruptions. While to support 6G new services, the above challenges need to be solved.

2.2.2 Service-based Interface

According to TS29.500 [2], the Service Based Architecture (SBA) is the core architectural framework for the 5G Core Network (5GC) defined by 3GPP, designed to enable flexible, scalable, and efficient deployment of network services. It replaces the traditional point-to-point interface model with a service-centric paradigm, which relies on a foundational NF Service Framework to enable standardized, secure, and efficient use of Network Function (NF) services. This framework is designed to facilitate interactions between NF Service Producers (NFs exposing services) and NF Service Consumers (NFs accessing services), and it operates through two key mechanisms: 1) NF service registration and de-registration to make other NFs aware of the available NF instances and supported services; 2) NF service discovery to enable a NF Service Consumer to discover NF Service Producer instance(s) which provide the expected NF service(s).

To support NF Service Consumer acquire service from NF Service Producer, the SBA relies on standardized Service Based Interfaces (SBIs)—the unified interfaces through which all NF service invocations occur. As mandated in the TS29.500 [2], each NF service is designed to be self-contained (operable independently of other services) and manageable separately (e.g., for scaling or updates). The standardized design of SBIs eliminates vendor lock-in, simplifies network integration, and supports the SBA’s core goal of flexible, scalable service deployment.

Main Characteristics of SBI interface is as below:

- CP Signaling Path:

RAN connects with CN NFs via standardized SBI. RAN node no longer relies on fixed P2P anchors (e.g., AMF) but directly discovers and invokes new services via the NRF (Network Function Repository Function).

- Protocol Stack:

Possible SBI Protocol Stack can be found as blow:

Figure 2.2-2: SBI Protocol Stack

SBIs may consider to use TCP and HTTP/2 protocol with JSON as already supported in 5GC.

SBIs may consider to use QUIC and HTTP/3 protocol in 6G, whether to use JSON or ASN.1 can be further studied.

In HTTP/2, a large message is segmented into a sequence of frames, starting with a HEADER frame followed by multiple DATA frames. These frames are transmitted over a single, persistent TCP connection. Within a given HTTP/2 stream, identified by a unique Stream ID, frames are delivered in order, allowing the receiver to reassemble the message . HTTP/3 provides a similar segmentation mechanism for large message sending, using the similar frame types (HEADER, DATA). However, these frames are carried over QUIC streams instead of a TCP connection. Each HTTP/3 stream is mapped to an independent QUIC stream, ensuring in-order delivery within the stream while avoiding head-of-line blocking across streams. In general, we think the HTTP2 or HTTP3 based SBI interface can support large size data transmission by segmentation mechanism.

Observation 3: From a HTTP2/3 protocol perspective, through the segmentation mechanism, there is no limitation on  large size of data transmission  over SBI.

- Key Advantages

- The standardized design of SBIs eliminates RAN-CN interface redundancy, simplifies network integration and enable operators to deploy and update network services independently, reducing operational complexity and accelerating the introduction of new 6G-enabled applications.

With the rapid emergence of new 6G services (e.g., sensing, AI), a readily extensible interface is critical to be considered in 6G. Compared with P2P interface, the Service Based Interface (SBI) is easier to achieve this due to its inherent flexibility. When new services arise, operators only need to define new standardized APIs for target NFs without overhauling the entire framework, avoiding the reconfiguration bottlenecks of traditional interfaces and unnecessary network entity upgrading. In short, SBI’s modular design and standardized APIs make it ideal for accommodating new services easily and efficiently. For example, if SBI-based interface can be supported for the new RAN-CN interface, the RAN node can consider to deploy several NFs, for example, one NF can be a Service Producer instance(s) providing sensing service, another NF can be a Service Producer instance(s) providing AI service. It enables direct RAN to NF communication without impact on legacy core while providing scalability to signaling RAN information by demand.

Similar like SBI applied in CN side, it provides the plug-and-play capability to support new NFs in 6G CN and dynamic service discovery and registration. And it's also easy to support authorized third-party applications via Common API Framework (CAPIF).

Compared with legacy P2P interface, it can simplify fault detection without intermediate node, only related NFs need to be upgraded for the new service.

It has been well applied in CN and better adapted to third-party authorization and access (TLS + OAuth) or QUIC depends on the transport protocol design.

Observation 4: The Service Based Interface (SBI) is natively compatible with third-party applications and ideal for accommodating new services easily and efficiently.

- Deployment Scalibility Challenge:

- If an SBI-based interface is used for the new RAN-CN interface, the deployment scalibility challenge is rasised to be discussed:

- RAN-Side Challenges: In typical internet, HTTP servers are implemented as scalable clusters with load balancing within cluster nodes to handle large volumes of HTTP requests efficiently. However, traditional base stations(e.g., deployed without cloudified CUs) are constrained by limited CPU and storage resources, often equipped with specialized rather than general-purpose processors. As a result, they are not designed to efficiently process high volumes of HTTP requests, e.g.,from massive third parties.

- Core-Side Challenges: While the Core Network typically benefits from centralized cloud infrastructure, scalability challenges can still emerge when processing concurrent SBI interactions from a large number of RAN nodes. Any bottleneck in SBI handling or service function response time may degrade end-to-end performance, particularly for time sensitive services.

- To address the above challenges, we propose the following migration strategy:

- Migration Strategy in RAN: No Need for Direct Connection between RAN and Third-Party Applications:

- RAN nodes connect to Core NEF/NF via SBI.

- NEF/NF aggregates mulitiple HTTP requests from massive third parties.

- NEF/NF requests data once from RAN, distributes to third parties.

- Similar data retrieval mechanism can also be applied on the direction from RAN to third parties.

- Migration Strategy in Core:

- Core side NFs interacting with a large number of RAN nodes/third parties can be implemented as scalable and elastic computing clusters/pool.

- When HTTP load increases or processing latency rises, these resources can be expanded through hardware scaling such as adding more physical servers or upgrading existing infrastructure to ensure performance and service reliability.

Based on above discussion ,we provide the following proposal.

Proposal 2: Service Based Interface for supporting 6G new services can natively compatible with third-party applications, ideal for accommodating new services easily and efficiently and avoids unnecessary routing latency. Furthermore, the deployment scalibility challenge can be solved by proper Migration Strategy as analyzed above.

2.2.3 Enhanced P2P interface (by using QUIC as transport layer)

The QUIC based NGAP interface represents an evolution of the traditional 5G NGAP P2P control plane (e.g., NG-C interface between gNB and AMF) , it replaces the legacy SCTP transport layer with QUIC.

- CP Signaling Path

Similar like legacy P2P.

- Protocol Stack:

If QUIC based NGAP interface is adopted, a typical Protocol Stack is as below:

Figure 2.2-3: QUIC based NG-U and NG-C Protocol Stack

The QUIC-based Protocol replaces SCTP with QUIC to address control-plane demands for low latency, multiplexing, and simplified deployment. In details, QUIC supports multiple independent streams over a single P2P connection, enables 1-RTT or 0-RTT connection setup and supports real-time congestion control algorithms.

NGAP remains the application-layer protocol for control-plane signaling , but it is optimized for QUIC’s transport model. Each NGAP control message is encapsulated into a dedicated QUIC stream.

- Key Advantages

- CT4 has evaluated QUIC in R16-R18 in TR 29.893 with following expected improvements, and further study and evaluation on QUIC will be continued in 6G:

- Clause 5.4 identifies features of QUIC (HTTP/3) that would be applicable to the 5GC Service Based Interfaces; the following potential (performance) improvements are anticipated in comparison to HTTP/2:

- 1)	QUIC allows to overcome HOL(Head-of-Line Blocking) blocking from which HTTP/2 is suffering if a TCP packet is lost or becomes corrupted;

- 2)	loss detection mechanisms of QUIC are using more accurate means to indicate lost bytes and RTT measurements resulting in assumedly more efficient recovery mechanism;

- 3)	faster connection establishment compared to TLS/TCP (1 RTT instead of 2), for short lived connections; however, when using persistent connections, this will not lead to a performance improvement;

- 4)	the connection may be migrated to a different network interface or local address by the client during the lifetime of the connection or by the server during the connection establishment.

- Challenges

Although QUIC may bring better performance than TCP in transport layer, its application together with NGAP in 3GPP network lacks verification. Introducing QUIC based NGAP would demand new conformance and interoperability test cases to verify that RAN and CN nodes from different vendors can correctly implement QUIC-based NGAP, including handling connection migration, 1-RTT or 0-RTT handshakes, and stream multiplexing as defined by IETF QUIC specifications (RFC 9000). For example, in extreme network conditions with high packet loss rates, the congestion control and retransmission mechanisms of QUIC may not yet match TCP’s maturity in 5G core networks, potentially affecting the stability and robustness of NGAP signaling transmission and service quality. The similar issue raised for QUIC based NG-U.

On the other hand, the current 5GC service based interfaces use HTTP/2 over TCP.  HTTP3 over QUIC is the latest version of HTTP which carries high volumes of web traffic. As we know, CT4 has evaluated QUIC in R16-R18 in TR 29.893 and some limitations of QUIC/HTTP3 were also concluded as below:

- Security aspects may need further coordination with SA3.

- QUIC may exhibit bandwidth unfairness when coexisting with traditional TCP flows.

- HTTP/3 proxy support based on QUIC seems immature according to TR29.893 “The QUIC layer is end-to-end encrypted, and the use of proxies between end-to-end QUIC connections is not sufficiently covered. There is no support, for instance, to enable an end-to-end QUIC connection through a proxy using HTTP CONNECT.”“How to support HTTP/3 (QUIC) for indirect communications via SCP would need to be further studied”.

Observation 5: For QUIC based NGAP and QUIC based NG-U in 3GPP network, the Stability&Reliability, Security aspects and the impacts on traditional TCP flows lack verification.

Proposal 3: For enhanced P2P interface (by using QUIC as transport layer), the Stability&Reliability, Security aspects and the impacts on traditional TCP flows should be verified.

2.2.4 Other consideration

In order to help decide which option is best for 6G interface(s), we propose to compare the above three RAN-CN interface options from the aspects addressing the CAPEX/OPEX , flexibility, future-proof, practical challenges, and reliability in 6G networks.

First, the impacts on legacy CN directly relates to network CAPEX/OPEX: How well each interface option adapts with the existing network architecture needs to be evaluated (e.g., taking the effort on retrofitting, backward compatibility and migration into account) as operators may want to maximize the use of their existing network investment.

Second, Integration & Evolution directly relates to flexibility: Assessing how each option to support new 6G technologies (e.g., sensing, AI) in a future-proof way, enabling operators to deploy new services quickly.

Third, Operational Efficiency also reflects CAPEX/OPE: this includes the factors like fault detection and scalability (e.g., the impacts on other NFs when deploying new services).

Fourth, Security is an important factor in 6G interface design, and it is necessary to consider how to ensure the security of signaling and data transmission. For example, TCP-based signalling may rely on external IPsec, QUIC (Quick UDP Internet Connections) integrates security as a fundamental, non-optional component of the protocol..

Finally, Stability&Reliability addresses practical challenges: this encompasses technical maturity (e.g., standardized protocols, Commercial Deployment Verification) and hardware/software compatibility (e.g., whether existing RAN/CN nodes can run the interface without hardware upgrades).

In conclusion, the above five aspects of comparison do not focus solely on technical specs, instead, they holistically evaluate each option against operators’ current constraints (such as legacy infrastructure and costs), future strategic goals (including scalability and evolution), and non-negotiable requirements (like security and operability). Only by considering these aspects can operators select an interface that balances short-term deployment practicality with long-term business and technical sustainability.

Proposal 4: It is suggested that comparison of different interface options should not only focus on technical metrics but also on deployment and operation.

In addition, based on the analysis among three options, we can find that both P2P and SBI options have their own strengths, weaknesses, and suitability for distinct scenarios. RAN3 is suggested to consider whether to support multiple interfaces between RAN and CN. One illustration is shown as figure 2-2-4. The P2P interface shows advantages to support legacy communication functions, while the SBI interface shows advantages to support new services in 6G.

- Figure 2.2-4 One illustration of multiple interfaces between RAN and CN in 6G

Proposal 5: Multiple interfaces can be supported between RAN and CN in 6G.

3	Conclusions

Following is our observations and proposals:

Observation 1: In 5G, one Point to point (P2P) interface is defined between RAN and CN, and service based interfaces (SBI) are used for NFs in CN.

Observation 2: In 5G, network capability exposure mechanism is supported by Common API Framework (CAPIF) and the Network Exposure Function (NEF) based on SBA in CN.

The following three options have been proposed for 6G RAN-CN interface:

Opt1: Legacy P2P interface

Opt2: Service Based Interface

Opt3: Enhanced P2P interface

- Proposal 1: Legacy P2P interface for implementing these well-established and legacy communication functions can leverage mature infrastructure, minimizes redevelopment efforts, and avoids unnecessary operational disruptions. While to support 6G new services, the above challenges need to be solved.

Observation 3: From a HTTP2/3 protocol perspective, through the segmentation mechanism, there is no limitation on  large size of data transmission  over SBI.

Observation 4: The Service Based Interface (SBI) is natively compatible with third-party applications and ideal for accommodating new services easily and efficiently.

Proposal 2: Service Based Interface for supporting 6G new services can natively compatible with third-party applications, ideal for accommodating new services easily and efficiently and avoids unnecessary routing latency. Furthermore, the deployment scalibility challenge can be solved by proper Migration Strategy as analyzed above.

Observation 5: For QUIC based NGAP and QUIC based NG-U in 3GPP network, the Stability&Reliability, Security aspects and the impacts on traditional TCP flows lack verification.

Proposal 3: For enhanced P2P interface (by using QUIC as transport layer), the Stability&Reliability, Security aspects and the impacts on traditional TCP flows should be verified.

Proposal 4: It is suggested that comparison of different interface options should not only focus on technical metrics but also on deployment and operation.

Proposal 5: Multiple interfaces can be supported between RAN and CN in 6G.

4	Reference

[1] RP-251881 New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

[2] TS29.500 Technical Realization of Service Based Architecture

5	Text Proposal for TR 38.914

6.1.3	RAN-CN Interface Options

Editor’s note: This chapter includes description of RAN-CN interface options including protocol stacks, considering new and existing services.

The following three options are proposed for 6G RAN-CN interface(s):

Opt1: Legacy P2P interface

Opt2: Service Based Interface

Opt3: Enhanced P2P interface

The detail definition and protocol stacks for each option can be found as below:

Opt1: Legacy P2P interface

As we know, the P2P connection between RAN and CN is a traditional connection method inherited from the 4G/5G.

CP Signaling Path:

In 5G, all Control Plane (CP) signaling between the RAN node and NF in the CN (Core Network) shall be routed via the AMF (Access and Mobility Management Function), which serves as an anchor node. For example, to transfer CP signaling such as UE’s session request between UE and CN, the path should be:  UE → RAN → AMF → Target NF (e.g., SMF), with no shortcuts or dynamic rerouting. In 6G, it can assume that more NF will be introduced in CN side in order to support 6G new services, how to achieve the direct connection between RAN and NFs as UE → RAN → Target NF (e.g., Sensing, AI) without routing delay needs to be solved.

Protocol Stack:

Built on IP transport with SCTP (Stream Control Transmission Protocol) at the transport layer (above IP) to ensure reliable, ordered delivery of CP messages (e.g., UE context transfer, mobility management). The application layer uses 3GPP-specific protocols (e.g., NGAP for NG-C) to encapsulate CP signaling (e.g., PDU session establishment, paging).

Figure 6.1.3-1: existing NG-U and NG-C Protocol Stack

Key Advantages:

Mature 3GPP standardization, backward compatibility with 5G, and predictable signaling paths for legacy communication functions.

Considering  the traditional 5G P2P NG interface is primarily designed to support functions such as interface management, UE context management, mobility handling, NAS transport, paging, PDU session management. Notably, these capabilities have no inherent requirement for external exposure or invocation. Considering the costs and efficiency of network evolution, reusing the existing P2P interface for implementing these well-established and basic communication functions can leverage mature infrastructure, minimizes redevelopment efforts, and avoids unnecessary operational disruptions.

Challenges:

All RAN to NF/AF signaling is routed via AMF requiring AMF upgrade every time a service added, and it may lead to AMF overload, and the service latency will be increased due to unnecessary routing.

Different interface and signalling design for different 6G new services and it’s difficult to support customized RAN information retrieval for third-party applications.

Complex fault isolation due to intermediate entities, and all the intermediate entities have to be upgraded even they are not participating in the new service.

Furthermore, how to ensure the security of RAN information transmission (not UE traffic data) between RAN and NF needs to be studied.

Opt2: Service Based Interface

As mandated in the TS29.500, each NF service is designed to be self-contained (operable independently of other services) and manageable separately (e.g., for scaling or updates). The standardized design of SBIs eliminates vendor lock-in, simplifies network integration, and supports goal of flexible, scalable service deployment.

CP Signaling Path:

RAN connects with CN NFs via standardized SBI. RAN node no longer relies on fixed P2P anchors (e.g., AMF) but directly discovers and invokes new services via the NRF (Network Function Repository Function).

Protocol Stack:

Figure 6.1.3-2: SBI Protocol Stack

SBIs may consider to use TCP and HTTP/2 protocol with JSON as already supported in 5GC.

SBIs may consider to use QUIC and HTTP/3 protocol in 6G, whether to use JSON or ASN.1 can be further studied.

The HTTP2 or HTTP3 based SBI interface can support large size data transmission by segmentation mechanism.

Key Advantages:

- The standardized design of SBI eliminates RAN-CN interface redundancy, simplifies network integration and enable operators to deploy and update network services independently, reducing operational complexity and accelerating the introduction of new 6G-enabled applications.

With the rapid emergence of new 6G services (e.g., sensing, AI), a readily extensible interface is critical to be considered in 6G. Compared with P2P interface, the Service Based Interface (SBI) is easier to achieve this due to its inherent flexibility. When new services arise, operators only need to define new standardized APIs for target NFs without overhauling the entire framework, avoiding the reconfiguration bottlenecks of traditional interfaces and unnecessary network entity upgrading. In short, SBI’s modular design and standardized APIs make it ideal for accommodating new services easily and efficiently. For example, if SBI-based interface can be supported for the new RAN-CN interface, the RAN node can consider to deploy several NFs, for example, one NF can be a Service Producer instance(s) providing sensing service, another NF can be a Service Producer instance(s) providing AI service. It enables direct RAN to NF communication without impact on legacy core while providing scalability to signaling RAN information by demand.

Similar like SBI applied in CN side, it provides the plug-and-play capability to support new NFs in 6G CN and dynamic service discovery and registration. And it’s also easy to support authorized third-party applications via Common API Framework (CAPIF).

Compared with legacy P2P interface, it can simplify fault detection without intermediate node, only related NFs need to be upgraded for the new service.

It has been well applied in CN and better adapted to third-party authorization and access (TLS + OAuth) or QUIC depends on the transport protocol design.

Deployment Scalibility Challenge:

- If an SBI-based interface is used for the new RAN-CN interface, the deployment scalibility challenge needs to be considered:

- RAN-Side Challenges: In typical internet, HTTP servers are implemented as scalable clusters with load balancing within cluster nodes to handle large volumes of HTTP requests efficiently. However, traditional base stations(e.g., deployed without cloudified CUs) are constrained by limited CPU and storage resources, often equipped with specialized rather than general-purpose processors. As a result, they are not designed to efficiently process high volumes of HTTP requests, e.g.,from massive third parties.

- Core-Side Challenges: While the Core Network typically benefits from centralized cloud infrastructure, scalability challenges can still emerge when processing concurrent SBI interactions from a large number of RAN nodes. Any bottleneck in SBI handling or service function response time may degrade end-to-end performance, particularly for time sensitive services.

- To address the above challenges, we propose the following migration strategy:

- Migration Strategy in RAN: No Need for Direct Connection between RAN and Third-Party Applications:

- RAN nodes connect to Core NEF/NF via SBI.

- NEF/NF aggregates mulitiple HTTP requests from massive third parties.

- NEF/NF requests data once from RAN, distributes to third parties.

- Similar data retrieval mechanism can also be applied on the direction from RAN to third parties.

- Migration Strategy in Core:

- Core side NFs interacting with a large number of RAN nodes/third parties can be implemented as scalable and elastic computing clusters/pool.

- When HTTP load increases or processing latency rises, these resources can be expanded through hardware scaling such as adding more physical servers or upgrading existing infrastructure to ensure performance and service reliability.

Opt3: Enhanced P2P interface

The QUIC based NGAP interface represents an evolution of the traditional 5G NGAP P2P control plane (e.g., NG-C interface between gNB and AMF) , it replaces the legacy SCTP transport layer with QUIC.

- CP Signaling Path:

Similar like legacy P2P.

- Protocol Stack:

The protocol stack of QUIC based P2P interface is shown as below:

Figure 6.1.3-3: QUIC based NG-U and NG-C Protocol Stack

Key Advantages:

- CT4 has evaluated QUIC in R16-R18 in TR 29.893 with following expected improvements, and further study and evaluation on QUIC will be continued in 6G:

- Clause 5.4 identifies features of QUIC (HTTP/3) that would be applicable to the 5GC Service Based Interfaces; the following potential (performance) improvements are anticipated in comparison to HTTP/2:

- 1)	QUIC allows to overcome HOL blocking from which HTTP/2 is suffering if a TCP packet is lost or becomes corrupted;

- 2)	loss detection mechanisms of QUIC are using more accurate means to indicate lost bytes and RTT measurements resulting in assumedly more efficient recovery mechanism;

- 3)	faster connection establishment compared to TLS/TCP (1 RTT instead of 2), for short lived connections; however, when using persistent connections, this will not lead to a performance improvement;

- 4)	the connection may be migrated to a different network interface or local address by the client during the lifetime of the connection or by the server during the connection establishment.

Challenges:

Although QUIC may bring better performance than TCP in transport layer, its application together with NGAP in 3GPP network lacks verification. Introducing QUIC based NGAP would demand new conformance and interoperability test cases to verify that RAN and CN nodes from different vendors can correctly implement QUIC-based NGAP, including handling connection migration, 1-RTT or 0-RTT handshakes, and stream multiplexing as defined by IETF QUIC specifications (RFC 9000). For example, in extreme network conditions with high packet loss rates, the congestion control and retransmission mechanisms of QUIC may not yet match TCP’s maturity in 5G core networks, potentially affecting the stability and robustness of NGAP signaling transmission and service quality. The similar issue raised for QUIC based NG-U.

On the other hand, the current 5GC service based interfaces use HTTP/2 over TCP.  HTTP3 over QUIC is the latest version of HTTP which carries high volumes of web traffic. As we know, CT4 has evaluated QUIC in R16-R18 in TR 29.893 and some limitations of QUIC/HTTP3 were also concluded as below:

- Security aspects may need further coordination with SA3.

- QUIC may exhibit bandwidth unfairness when coexisting with traditional TCP flows.

- - HTTP/3 proxy support based on QUIC seems immature according to TR29.893 “The QUIC layer is end-to-end encrypted, and the use of proxies between end-to-end QUIC connections is not sufficiently covered. There is no support, for instance, to enable an end-to-end QUIC connection through a proxy using HTTP CONNECT.”“How to support HTTP/3 (QUIC) for indirect communications via SCP would need to be further studied”.

6.1.4	Evaluation of RAN-CN Interface Options

Editor’s note: This chapter includes evaluation and comparison of RAN-CN interface options described in clause 6.1.3.

---------------End of the Change------------------