# 3GPP TSG-RAN WG3 #130																                R3-258660 Dallas, USA, 17th-21st November, 2025

# Title: 	Remaining issues on early sync

# Source: 	ZTE Corporation

# Agenda item:	9.2.5

# Document Type:	Discussion and Decision

Introduction

During offline discussion after last RAN3#129bis meeting, there still are some open issues related to LTM left. In this contribution, we will discuss options for Early RACH Resource Requester ID,  Early RACH Resource update and LTM configuration modification.

Discussion

Early RACH Resource Requester ID

During RAN3#127 meeting, RAN3 had discussed that in order to avoid explicitly exposing the gNB internal architecture, a new ID of early RACH configuration resource or other new name can be used to request early RACH configuration. However, for a UE configured with inter-CU LTM, if any of the configured candidate LTM cells belongs to other gNB, the source gNB is not aware of the associated gNB-DU ID or RACH resource request ID(s) so that it is unable to provide the gNB-DU ID or RACH resource request ID(s)  for the candidate LTM cell in the early RACH request. This issue has been confirm during the offline discussion after RAN3#129bis meeting, and a summary is listed  as below:

It is well acknowledged that the Early RACH Resource Requester ID needs to be transferred over Xn before handover preparation. Two options are available: - Option 1: The IDs are transferred during Xn Setup and NG-RAN Node Configuration Update procedures. - Option 2: The IDs are transferred during LTM preparation phase. Proposal: Companies continue discussing the solution between these two options.

According to previous analysis and discussion, both option 1 and option 2 can work.

For option 1, some companies think since Early RACH Resource Requester ID is acquired before the first round Handover Request and ACK procedure, then the source gNB-CU can send the Early RACH Resources Providers List which includes multiple pairs of (gNB ID, Early RACH Resources Requester ID) to the candidate gNB-CU to request early RACH configuration for other candidate gNB-DUs in the first round Handover Request and ACK procedure.

However, we should note that although the LTM candidate cell(s) are selected based on the UE L3 measurement report by the source CU, it still has to send Handover Request to the target node to ask candidate cell for confirmation.  Without receiving LTM Handover Information Request Acknowledge from the candidate cell, the source CU cannot take it as candidate cell of the UE. As all the Early RACH Resources Providers which associated to all the candidate cells has to be provided in the Early Sync Information Request, only when the source node confirms all the candidate cells and associated Early RACH Resources Requester ID, it can send Early Sync Information Request.

- 9.2.3.217 Early Sync Information Request This IE contains a request for the early synchronization information. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Early Sync Configuration Request; M; ENUMERATED (true, …). Early RACH Resources Providers List; 0..1. >Early RACH Resources Item; 1..< maxnoofEarlyRACHResourcesID>. >>Global gNB ID; M; 9.2.2.1. >>Early RACH Resources Requester ID; M; 9.2.3.220; Identifies the entity requesting Early RACH resources..

Before the first round Handover Request and ACK procedure, the source CU cannot confirm all the candidate cell of the UE, so that it cannot provide all the pairs of {gNB ID, Early RACH Resources Requester ID}which associated to all the candidate nodes to the candidate gNB-CU to request early RACH configuration.

- Before the first round Handover Request and ACK procedure, the source CU cannot confirm all the candidate cell(s) of the UE, so that it cannot provide all the pairs of {gNB ID, Early RACH Resources Requester ID}which associated to all the candidate nodes to request early RACH configuration even if it knows Early RACH Resources Requester ID of the candidate cell.

Only after all Handover Request and ACK procedure is finished, the Source node can confirm the LTM candidate cell(s) for the UE. After that, it can know all the Early RACH Resources Providers List and send early RACH resource request in the second round. In other word, for both option 1 and option 2, the early RACH resource is requested and allocated in the second round via XnAP LTM Configuration Update and F1AP UE Context Modification procedure. The delay for option 1 and option 2 are the same.

- Since the Early Sync Information Request can be sent only after the source CU confirm all the candidate cell(s) of the UE after the first round Handover Request and ACK procedure. Both the option 1 and option 2 can acquire Early Sync response in the second round, which means the delay are the same.

When determining which option is better, we think following two factors should be considered and evaluated:

- Whether is easy to avoid exposing the gNB internal architecture since it is the main reason of introducing Early RACH Resources Requester ID.

- Impacts on specification and RAN node.

Proposal 1   When determining which option is better, following two factors should be considered and evaluated:

-  Whether is easy to avoid exposing the gNB internal architecture since it is the main reason of introducing Early RACH Resources Requester ID.

-  Impacts on specification and RAN node.

For option 1, Early RACH Resources Requester ID are transferred during Xn Setup and NG-RAN Node Configuration Update procedures. In normal case, the information exchanged during Xn Setup is relatively stable and does not change frequently. However, if the Early RACH Resources Requester ID of the cell is stable and not changed, it is obvious that this Early RACH Resources Requester ID is highly likely the DU ID. It is hard to avoid exposing the gNB internal architecture. On the other hand, if the Early RACH Resources Requester ID of the cell is changed frequently, it is hard for other gNBs to detect the gNB internal architecture, but this may have huge impacts on ongoing LTM procedures since if the Early RACH Resources Requester ID changes, the ongoing  Early RACH configuration cannot work. As regarding the impacts on specification and RAN node, since Early RACH Resources Requester ID are transferred during Xn Setup procedure, it has impacts on all the RAN node even if the RAN node decides not to use LTM for handover purposes and in current Xn Setup procedure, there is none of existing LTM related IE, new feature is introduced in Xn Setup procedure.

For option 2, since the Early RACH Resources Requester ID is configured per UE per candidate cell, if the gNB does not want to expose the gNB internal architecture, it can consider to map the Early RACH Resources Requester ID to cell ID flexible. For example, for the same candidate cell, the candidate node can use Early RACH Resources Requester ID-1 for UE-1 while use  Early RACH Resources Requester ID-2 for UE-2. or it can use Early RACH Resources Requester ID-1 for some time duration while use  Early RACH Resources Requester ID-2 for another time duration. As long as it is ensured that the Early RACH Resources Requester ID remains unchanged during the LTM procedure of a UE, it can work. This flexibility is more beneficial to achieving the concealment of the internal architecture. As regarding the impacts on specification, the candidate cell needs to provide  Early RACH Resources Requester ID in legacy Handover Request Acknowledge message if it confirms to be the candidate cell of the UE. Similarly, for LTM NR DC case, the candidate cell needs to provide Early RACH Resources Requester ID in S-NODE ADDITION REQUEST ACKNOWLEDGE. In detail, the Early RACH Resources Requester ID shall be included in the LTM Candidate PSCell Prepared List IE. Since there are existing LTM related IEs in Handover Request Acknowledge and S-NODE ADDITION REQUEST ACKNOWLEDGE message, only a new parameter is added.

- Table 1: Comparison between option 1 and option 2

|  | How to avoid exposing the gNB internal architecture | Impacts on specification | Impacts on RAN node | Delay of acquiring early RACH resource |
| --- | --- | --- | --- | --- |
| Option 1 | If the Early RACH Resources Requester ID of the cell is stable and not changed, it is hard to avoid exposing the gNB internal architecture. On the other hand, if it is changed frequently, the ongoing Early RACH configuration cannot work. | Has impacts on legacy Xn Setup and NG-RAN Node Configuration Update procedures, since there is none of existing LTM related IE in these message, a first level IE with new feature is introduced. | Has impacts on all the NG-RAN Node even if it is not a candidate LTM node. | Since the Early Sync Information Request can be sent only after the source CU confirm all the candidate cell(s) of the UE after the first round Handover Request and ACK procedure, Option 1 can acquire the early RACH resource in the second round. |
| Option 2 | - The mapping between Early RACH Resources Requester ID and cell is configured per UE and is more flexible, so it is easier to avoid exposing the gNB internal architecture. | Has impacts on legacy Handover Request Acknowledge and S-NODE ADDITION Request Acknowledge procedures.Since there are existing LTM related IEs in these message, only a new parameter is added. | Has impacts on LTM related node | Since the Early Sync Information Request can be sent only after the source CU confirm all the candidate cell(s) and its Early RACH Resources Requester ID(s) of the UE after the first round Handover Request and ACK procedure. Option 2 can acquire the early RACH resource in the second round. |

- Option 1 is associated to non-UE associated Xn Setup procedure, if the Early RACH Resources Requester ID of the cell is stable and not changed, it is hard to avoid exposing the gNB internal architecture. On the other hand, if it is changed frequently, the ongoing Early RACH configuration cannot work.

- Compared to option 1, Option 2 is UE associated LTM procedure, the mapping between Early RACH Resources Requester ID and cell is more flexible and is easier to avoid exposing the gNB internal architecture.

- Option 1 has impacts on all the NG-RAN Node even if it is not a LTM node while Option 2 only has impacts on LTM related node.

- Option 1 introduce a first level IE with new feature on legacy message while option 2 introduce a new parameter associated to a existing IE on legacy message.

- Based on above Comparison between option 1 and option 2, the option 2 is easier to avoid exposing the gNB internal architecture  and has less impacts on specification and RAN node. So we prefer option 2.

Proposal 2    Each candidate gNB shall provide Early RACH Resource Request ID(s) of the accepted candidate LTM cell(s) in the legacy Handover Request Acknowledge and S-NODE ADDITION Request Acknowledge procedures.

Early RACH Resource update

During offline discussion after last RAN3#129bis meeting, the issue on introducing a mechanism for PRACH resource release for early RACH has been discussed and a summary is listed  as below:

4 companies support introducing a mechanism for PRACH resource release. 4 companies are open to continue the discussion. This topic is related to the modification of LTM configurations. Proposal: Companies bring contributions to continue the discussion on this topic and to propose whether a new procedure should be introduced or combined with the potential LTM configuration modification procedure.

According to current mechanism, after the candidate node receive Early Sync Information Request, it will send the Early Sync Configuration including the RACH Configuration and Preamble Index List allocated to each candidate node. After the source node receives the early Sync Configuration from all the candidate nodes, it will send the collected  early Sync Configuration to all the candidate nodes. After that, there is no procedure to allow the candidate node to update the Early Sync Configuration.

However,  in case the accessing UE of the candidate node becomes more and more in some time duration, the PRACH resource becomes nervous and the left PRACH available for non-LTM UE RACH purposes may be not enough, then it may want to deallocate some PRACH resource previously allocated for early sync. On the other hand, in case the LTM UE becomes more and more but non-LTM UE becomes less, it can also consider to reallocate/add some  PRACH resource for early sync. In a word, it is beneficial to support candidate node updating the allocated Early Sync resource  after sending the Early Sync Configuration.

Although the Early Sync Configuration is provide via UE-associated procedure since the RACH Configuration is provided for each UE, but the Early RACH Resources is allocated and shared for candidate node. Therefore, since only PRACH resource used for Early RACH rather than RACH Configuration needs to be updated. Introducing a non-UE associated procedure to support  Early RACH Resources updating is more reasonable and simple.

- The PRACH resource used for Early RACH is allocated and shared for each candidate node. And only PRACH Resources rather than RACH Configuration needs to be updated.

- According to current TA Information List as below, we can find that the TA value is indicated per PRACH resource, and the pair of {Preamble Index, RA-RNTI} is used to identify a PRACH resource. So  Early RACH Resources Update may include a added or released list of {Preamble Index, RA-RNTI}.

Proposal 3    Introducing a non-UE associated procedure to support Early RACH Resources Update.

- Then if candidate node indicate a released list of {Preamble Index, RA-RNTI}, then the source node is not allowed to allocate the PRACH resource indicated in the released list to any UE for early RACH purpose.

- 9.1.5.4 TA INFORMATION TRANSFER This message is sent by the NG-RAN node1 to inform the NG-RAN node2 about the Timing Advance value and related information. Direction: NG-RAN node1  NG-RAN node2. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; reject. TA Information List; 1; YES; reject. >TA Information Transfer Item; <1… maxnoofTAList>; –. >>Early RACH Resources Requester ID; M; 9.2.3.220; Identifies the entity requesting Early RACH resources.; –. >>Candidate Cell ID; M; NR CGI 9.2.2.7; –. >>TA Value; M; INTEGER (0..4095); Indicates the TA value as defined in TS 38.213 [40].; –. >>Preamble Index; M; INTEGER (0..63); –. >>RA-RNTI; M; INTEGER (0..65535, ...); RA-RNTI as defined in TS 38.321 [35].; –. >>Tag ID Pointer; O; OCTET STRING; Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [10].; –.

LTM configuration modification

There is an open issue left after last RAN3#129bis meeting as below:

Whether the modification of LTM configurations is allowed over network signaling? If yes, which configuration needs to be modified? When can LTM modification be triggered? For example, during or after LTM preparation? Is it possible to modify during LTM execution or in subsequent LTM? If there is any configuration that can be modified, which way is preferred? a. Existing LTM Cancel procedure, and the source re-initiates the configuration procedure b. New procedure

Unlike the Early RACH Resource update, the modification of UE's CSI-RS resource and CFRA resource have impacts on UE and only UE-associated procedure can be used. Considering the complexity/significant spec impacts and R19 is to be freezing, we prefer to discuss the support of LTM modification in Rel-20. In fact, if a candidate cell wants to modify the LTM configuration since the current configuration can not be applied well, it can trigger existing LTM cancel procedures to release current LTM configuration. If necessary, the source can re-initiates the LTM request procedure to the candidate cell and then the candidate cell can provide the modified LTM configuration. By this way, LTM configuration modification can be realized without impacts on specification.

Proposal 4    In order to modify LTM configuration, a candidate cell can trigger existing LTM cancel procedures to release current LTM configuration, if necessary, the source can re-initiates the LTM request procedure to the candidate cell and then the candidate cell can provide the modified LTM configuration.

The stage 3 updates to capture the above proposals are detailed in R3-258661, R3-258662, R3-258663 and R3-258664.

Conclusions and Proposals

In this contribution we made observations and proposals for some remaining LTM topics as below:

Early RACH Resource Requester ID:

- Before the first round Handover Request and ACK procedure, the source CU cannot confirm all the candidate cell(s) of the UE, so that it cannot provide all the pairs of {gNB ID, Early RACH Resources Requester ID}which associated to all the candidate nodes to request early RACH configuration even if it knows Early RACH Resources Requester ID of the candidate cell.

- Since the Early Sync Information Request can be sent only after the source CU confirm all the candidate cell(s) of the UE after the first round Handover Request and ACK procedure. Both the option 1 and option 2 can acquire Early Sync response in the second round, which means the delay are the same.

- Proposal 1   When determining which option is better, following two factors should be considered and evaluated:

                  - -  Whether is easy to avoid exposing the gNB internal architecture since it is the main reason of introducing Early RACH Resources Requester ID.

                  - -  Impacts on specification and RAN node.

- Option 1 is associated to non-UE associated Xn Setup procedure, if the Early RACH Resources Requester ID of the cell is stable and not changed, it is hard to avoid exposing the gNB internal architecture. On the other hand, if it is changed frequently, the ongoing Early RACH configuration cannot work.

- Compared to option 1, Option 2 is UE associated LTM procedure, the mapping between Early RACH Resources Requester ID and cell is more flexible and is easier to avoid exposing the gNB internal architecture.

- Option 1 has impacts on all the NG-RAN Node even if it is not a LTM node while Option 2 only has impacts on LTM related node.

- Option 1 introduce a first level IE with new feature on legacy message while option 2 introduce a new parameter associated to an existing IE in the legacy messages.

- Based on above Comparison between option 1 and option 2, the option 2 is easier to avoid exposing the gNB internal architecture  and has less impacts on specification and RAN node. So we prefer option 2.

Proposal 2    Each candidate gNB shall provide Early RACH Resource Request ID(s) of the accepted candidate LTM cell(s) in the legacy Handover Request Acknowledge and S-NODE ADDITION Request Acknowledge procedures.

The CRs to TS 38.423 and TS 38.473 for Early RACH Resource Requester ID are given in R3-258661 and R3-258662.

Early RACH Resource update:

- The PRACH resource used for Early RACH is allocated and shared for each candidate node. And only PRACH Resources rather than RACH Configuration need to be updated.

- According to current TA Information List as below, we can find that the TA value is indicated per PRACH resource, and the pair of {Preamble Index, RA-RNTI} is used to identify a PRACH resource. So Early RACH Resources Update may include a added or released list of {Preamble Index, RA-RNTI}.

Proposal 3      Introducing a non-UE associated procedure to support Early RACH Resources Update.

The stage 3 updates to capture the above proposals are detailed in R3-258663 and R3-258664.

LTM Configuration modification:

Proposal 4    In order to modify LTM configuration, a candidate cell can trigger existing LTM cancel procedures to release current LTM configuration, if necessary, the source can re-initiates the LTM request procedure to the candidate cell and then the candidate cell can provide the modified LTM configuration.

References

- R3-258661, Corrections on early RACH Resource Requester ID - Xn, ZTE Corporation

- R3-258662, Corrections on early RACH Resource Requester ID - F1, ZTE Corporation

- R3-258663, Corrections on LTM early sync resource update - Xn, ZTE Corporation

- R3-258664, Corrections on LTM early sync resource update - F1, ZTE Corporation