3GPP TSG-RAN WG3 #131																		R3-260618

# Gothenburg, Sweden, 9-13 February 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1698 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for modification of LTM configurations for Option 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Ofinno, Jio Platforms, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2026-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To support network signaling for possible updates of LTM configurations that is initiated by the candidate gNB, allowing UEs and the network to handle configuration changes correctly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The existing LTM Configuration Update procedure is extended to allow a candidate gNB to inform the source gNB that the LTM configurations have been updated, e.g., early UL sync configurations, CSI-RS resource configurations, TCI states information, and CFRA resource configurations. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because the change only affects the LTM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UEs and the network may fail to handle LTM configuration updates, causing signaling issues or inconsistent behavior. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.1, 9.1.5.1, 9.3.4, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.473 CR 1702 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

## 8.6	Procedures for L1/L2 Triggered Mobility

### 8.6.1	LTM Configuration Update

#### 8.6.1.1	General

The purpose of the LTM Configuration Update procedure is to update LTM configuration data, or to inform the change of serving NG-RAN node and setup the UE associated signalling between the new serving NG-RAN node and other candidate NG-RAN node(s). The procedure uses UE associated signalling. The procedure can also be initiated by the candidate NG-RAN node to modify the LTM configuration data and inform the serving NG-RAN node.

#### 8.6.1.2	Successful Operation

Figure 8.6.1.2-1: LTM Configuration Update procedure. Successful operation

The NG-RAN node1 initiates the procedure by sending the LTM CONFIGURATION UPDATE message to NG-RAN node2.

The LTM CONFIGURATION UPDATE message may contain:

-	the LTM Updates to Candidate Node Information IE;

-	the LTM Updates to Candidate Cell Information List IE;

-	the LTM UE Security Information for LTM IE;

-	the LTM Updates to Source Node Information List IE.

If the Reference Configuration IE is contained in the LTM Updates to Candidate Node Information IE included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, take it into account for generating the LTM configuration.

If the LTM Configuration ID Mapping List IE is contained in the LTM Update to Candidate Node Information IE included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider this as the mapping information for the LTM candidate cell(s).

If the CSI Resource Configuration IE is contained in the LTM Update to Candidate Node Information IE included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, use it to generate the LTM CSI reporting configuration.

If the LTM Updates to Candidate Cell Information List IE is contained in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, use it as specified in TS 38.300 [9].

If the LTM Updates from Candidate Cell Information List IE is contained in the LTM CONFIGURATION UPDATE ACKNOWLEDGE message, the NG-RAN node1 shall, if supported, use it as specified in TS 38.300 [9].

If the LTM UE Security Information for LTM IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall use the information for subsequent LTM.

If the Data Forwarding Info for LTM List IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall use the information for data forwarding.

If the LTM Updates to Source Node Information List is contained in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, overwrite any previously received LTM configurations and use the received information to reconfigure the UE as defined in TS 38.300 [9].

#### 8.6.1.3	Unsuccessful Operation

Figure 8.6.1.3-1: LTM Configuration Update procedure. Unsuccessful Operation

If the NG-RAN node2 cannot accept the update, it shall respond with the LTM CONFIGURATION UPDATE FAILURE message with an appropriate cause value.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.1.5	Messages for L1/L2 Triggered Mobility

#### 9.1.5.1	LTM CONFIGURATION UPDATE

This message is sent by the NG-RAN node1 to update LTM configuration data.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1 | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2 | YES | reject |
| LTM Updates to Candidate Node Information |  | 0..1 |  |  | YES | ignore |
| >Reference Configuration | O |  | OCTET STRING | Includes the ltm-ReferenceConfiguration as defined in TS 38.331 [10]. | – |  |
| >LTM Configuration ID Mapping List | O |  | 9.2.3.221 |  | – |  |
| >CSI Resource Configuration | O |  | 9.2.3.223 |  | – |  |
| LTM Updates to Candidate Cell Information List |  | 0..1 |  |  | YES | ignore |
| >LTM Updates to Candidate Cell Information Item |  | 1.. <maxnoofLTMCells> |  |  | – |  |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>Early Sync Information | O |  | 9.2.3.218 | Indicates the consolidated early synchronization information to be used for subsequent LTM. | – |  |
| >>LTM CFRA Resource Information | O |  | 9.2.3.232 |  | – |  |
| >>UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [10], for the LTM candidate cell identified by the Candidate Cell ID IE. | – |  |
| >>AS Security Information | O |  | 9.2.3.50 |  | – |  |
| >>LTM No Security Change ID | O |  | 9.2.3.231a |  | – |  |
| >>Data Forwarding Info for LTM List |  | 0..1 |  |  | – |  |
| >>>Data Forwarding Info for LTM Item |  | 1..<maxnoofPDUSessions> |  |  | – |  |
| >>>>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>>>Data Forwarding Info from target NG-RAN node | M |  | 9.2.1.16 |  | – |  |
| >>Last Target NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| LTM UE Security Information | O |  | 9.2.3.230 |  | YES | ignore |
| LTM Updates to Source Node Information List |  | 0..1 |  |  | YES | ignore |
| >LTM Updates to Source Node Information Item |  | 1.. <maxnoofLTMCells> |  |  | – |  |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>Early Sync Configuration | O |  | 9.2.3.218 |  | – |  |
| >>LTM CFRA Resource Information | O |  | 9.2.3.232 |  | – |  |
| >>TCI States Configurations List | O |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [10]. | – |  |
| >>CSI-RS Resource Configuration for L1 Measurements | O |  | CSI-RS Resource Configuration 9.2.3.224 |  | – |  |
| >>CSI-RS Resource Configuration for Early CSI Acquisition | O |  | CSI-RS Resource Configuration 9.2.3.224 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

#### 9.1.5.2	LTM CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by the NG-RAN node2 to inform the NG-RAN node1 to acknowledge update of information associated to an LTM configuration.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1. | YES | reject |
| NG-RAN node 2 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2. | YES | reject |
| LTM Updates from Candidate Cell Information List |  | 0..1 |  |  | YES | ignore |
| >LTM Candidate Cell Information Item |  | 1.. <maxnoofLTMCells> |  |  | – |  |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>LTM Candidate Configuration | M |  | OCTET STRING | Includes the ltm-CandidateConfig as defined in TS 38.331 [10]. | – |  |
| >>Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | – |  |
| >>CSI-RS Report Configuration for Early CSI Acquisition | O |  | OCTET STRING | Includes the ltm-CSIReportConfig as defined in TS 38.331 [10]. | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< ASN.1 Changes >>>>>>>>>>>>>>>>>>>>

### 9.3.4	PDU Definitions

(skip unchanged)

LPWUSPSassistanceInformation,

FurtherExtended-UEIdentityIndexValue,

Future-Coverage-Modification-List,

TimeSCG-Failure,

FailureType,

LTMInformationSCG-AddReq,

LTMInformationSCG-AddReqAck,

LTMInformationSCG-ModReq,

LTMInterSNExecutionNotification,

LTMPSCellInformation-AddReq,

LTMPSCellInformation-AddReqAck,

LTMPSCellInformation-UpdateReq,

LTMPSCellInformation-UpdateReqAck,

LTMPSCellInformation-UpdateReqired,

LTMPSCellInformation-UpdateConfirm,

LTMPSCellInformation-ChangeRequired,

LTMPSCellInformation-ChangeConfirm,

LTM-DC-DataForwarding-Indicator,

LTMConfig-to-Modify-List

FROM XnAP-IEs

(skip unchanged)

id-LTMPSCellInformation-AddReq,

id-LTMPSCellInformation-AddReqAck,

id-LTMPSCellInformation-UpdateReq,

id-LTMPSCellInformation-UpdateReqAck,

id-LTMPSCellInformation-UpdateReqired,

id-LTMPSCellInformation-UpdateConfirm,

id-LTMPSCellInformation-ChangeRequired,

id-LTMPSCellInformation-ChangeConfirm,

id-LTM-DC-DataForwarding-Indicator,

id-LTMConfig-to-Modify-List,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows,

maxnoofServedCellsIAB,

maxnoofTrafficIndexEntries,

maxnoofTLAsIAB,

maxnoofBAPControlPDURLCCHs,

maxnoofServingCells,

maxnoofSSBAreas

FROM XnAP-Constants;

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- LTM CONFIGURATION UPDATE

--

-- **************************************************************

LTMConfigurationUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{LTMConfigurationUpdate-IEs}},

...

}

LTMConfigurationUpdate-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NGRAN-Node1-ID								CRITICALITY reject	TYPE NG-RANnodeUEXnAPID										PRESENCE mandatory}|

{ ID id-NGRAN-Node2-ID									CRITICALITY reject	TYPE NG-RANnodeUEXnAPID										PRESENCE optional }|

{ ID id-LTMUpdatesToCandidateNodeInformation		CRITICALITY ignore	TYPE LTMUpdatesToCandidateNodeInformation				PRESENCE optional }|

{ ID id-LTMUpdatesToCandidateCellInformation-List	CRITICALITY ignore	TYPE LTMUpdatesToCandidateCellInformation-List		PRESENCE optional }|	{ ID id-LTMUESecurityInformation					CRITICALITY ignore	TYPE LTMUESecurityInformation								PRESENCE optional }|

{ ID id-LTMConfig-to-Modify-List					CRITICALITY ignore	TYPE LTMConfig-to-Modify-List								PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- LTM CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

LTMConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{LTMConfigurationUpdateAcknowledge-IEs}},

...

}

LTMConfigurationUpdateAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NGRAN-Node1-ID								CRITICALITY reject	TYPE NG-RANnodeUEXnAPID										PRESENCE mandatory}|

{ ID id-NGRAN-Node2-ID								CRITICALITY reject	TYPE NG-RANnodeUEXnAPID										PRESENCE mandatory}|

{ ID id-LTMUpdatesFromCandidateCellInformation-List	CRITICALITY ignore	TYPE LTMUpdatesFromCandidateCellInformation-List		PRESENCE optional }|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics									PRESENCE optional },

...

}

-- **************************************************************

--

#### -- LTM CONFIGURATION UPDATE FAILURE

--

-- **************************************************************

LTMConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{LTMConfigurationUpdateFailure-IEs}},

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

(skip unchanged)

maxnoofLTMCells,

maxNoSSBs,

maxnoofTAList,

maxnoofPreambleIndexes,

maxnoofCSIResourceConfigurationsPlus1,

maxnoofNZP-CSI-RS-ResourcesPerSet,

maxnoofSRS-Resource,

maxnoofFailedSliceMeasObjects,

maxnoofSliceItemsforMDT,

maxnoofAreaNTNforMDT,

maxnoofLTMCellsPlusOne,

maxnoofSCGSecurityConfigurations

FROM XnAP-Constants

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- L

Local-NG-RAN-Node-Identifier ::= CHOICE {

full-I-RNTI-Profile-List					Full-I-RNTI-Profile-List,

short-I-RNTI-Profile-List					Short-I-RNTI-Profile-List,

choice-extension							ProtocolIE-Single-Container { { Local-NG-RAN-Node-Identifier-ExtIEs} }

}

Local-NG-RAN-Node-Identifier-ExtIEs XNAP-PROTOCOL-IES ::= {

{ ID id-Full-and-Short-I-RNTI-Profile-List		CRITICALITY ignore	TYPE Full-and-Short-I-RNTI-Profile-List	PRESENCE mandatory},

...

}

(skip unchanged)

LTMConfig-to-Modify-List ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF LTMConfig-to-Modify-Item

LTMConfig-to-Modify-Item ::= SEQUENCE {

candidateCellID								NR-CGI,

earlySyncInformation						EarlySyncInformation				OPTIONAL,

lTMCFRAResourceInformation					LTMCFRAResourceInformation 			OPTIONAL,

cSI-RSResourceConfigurationForLayer1Measurements		CSI-RSResourceConfiguration		OPTIONAL,

cSI-RSResourceConfigurationForEarlyCSIAcquisition		CSI-RSResourceConfiguration		OPTIONAL,

iE-Extensions                               ProtocolExtensionContainer { { LTMConfig-to-Modify-Item-ExtIEs} } OPTIONAL,

...

}

LTMConfig-to-Modify-Item-ExtIEs   XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.7	Constant definitions

(skip unchanged)

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxEARFCN									INTEGER ::= 262143

maxnoofAllowedAreas							INTEGER ::= 16

maxnoofAMFRegions							INTEGER ::= 16

maxnoofAoIs									INTEGER ::= 64

maxnoofBluetoothName						INTEGER ::= 4

maxnoofBPLMNs								INTEGER ::= 12

maxnoofCAGs									INTEGER ::= 12

maxnoofCAGsperPLMN							INTEGER ::= 256

maxnoofCellIDforMDT							INTEGER ::= 32

maxnoofCellsinAoI							INTEGER ::= 256

maxnoofCellsinUEHistoryInfo					INTEGER ::= 16

maxnoofCellsinNG-RANnode					INTEGER ::= 16384

maxnoofCellsinRNA							INTEGER ::= 32

maxnoofCellsUEMovingTrajectory				INTEGER ::= 16

maxnoofDRBs									INTEGER ::= 32

maxnoofEUTRABands							INTEGER ::= 16

maxnoofEUTRABPLMNs							INTEGER ::= 6

maxnoofEPLMNs								INTEGER ::= 15

maxnoofExtSliceItems						INTEGER ::= 65535

maxnoofEPLMNsplus1							INTEGER ::= 16

maxnoofForbiddenTACs						INTEGER ::= 4096

maxnoofFreqforMDT							INTEGER ::= 8

maxnoofMBSFNEUTRA							INTEGER ::= 8

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofMultiConnectivityMinusOne			INTEGER ::= 3

maxnoofNeighbours							INTEGER ::= 1024

maxnoofNeighPCIforMDT						INTEGER ::= 32

maxnoofNIDs									INTEGER ::= 12

maxnoofNRCellBands							INTEGER ::= 32

maxnoofPLMNs								INTEGER ::= 16

maxnoofPDUSessions							INTEGER ::= 256

maxnoofProtectedResourcePatterns			INTEGER ::= 16

maxnoofQoSFlows								INTEGER ::= 64

maxnoofQoSParaSets							INTEGER ::= 8

maxnoofRANAreaCodes							INTEGER ::= 32

maxnoofRANAreasinRNA						INTEGER ::= 16

maxnoofRANNodesinAoI						INTEGER ::= 64

maxnoofSCellGroups							INTEGER ::= 3

maxnoofSCellGroupsplus1						INTEGER ::= 4

maxnoofSensorName							INTEGER ::= 3

maxnoofSliceItems							INTEGER ::= 1024

maxnoofSNPNIDs								INTEGER ::= 12

maxnoofsupportedPLMNs						INTEGER ::= 12

maxnoofsupportedTACs						INTEGER ::= 256

maxnoofTAforMDT								INTEGER ::= 8

maxnoofTAI									INTEGER ::= 16

maxnoofTAIsinAoI							INTEGER ::= 16

maxnooftimeperiods							INTEGER ::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofUEContexts							INTEGER ::= 8192

maxNRARFCN									INTEGER ::= 3279165

maxNrOfErrors								INTEGER ::= 256

maxnoofslots								INTEGER ::= 5120

maxnoofExtTLAs								INTEGER ::= 16

maxnoofGTPTLAs								INTEGER ::= 16

maxnoofCHOcells								INTEGER ::= 8

maxnoofPC5QoSFlows 							INTEGER ::= 2064

maxnoofSSBAreas								INTEGER ::= 64

maxnoofRAReports							INTEGER ::= 64

maxnoofNRSCSs								INTEGER ::= 5

maxnoofPhysicalResourceBlocks				INTEGER ::= 275

maxnoofAdditionalPDCPDuplicationTNL			INTEGER ::= 2

maxnoofRLCDuplicationstate					INTEGER ::= 3

maxnoofWLANName								INTEGER ::= 4

maxnoofNonAnchorCarrierFreqConfig			INTEGER ::= 15

maxnoofDataForwardingTunneltoE-UTRAN		INTEGER ::= 256

maxnoofMBSFSAs								INTEGER ::= 256

maxnoofUEIDIndicesforMBSPaging				INTEGER ::= 4096

maxnoofMBSQoSFlows							INTEGER ::= 64

maxnoofMRBs									INTEGER ::= 32

maxnoofCellsforMBS							INTEGER ::= 8192

maxnoofMBSServiceAreaInformation			INTEGER ::= 256

maxnoofTAIforMBS							INTEGER ::= 1024

maxnoofAssociatedMBSSessions				INTEGER ::= 32

maxnoofMBSSessions							INTEGER ::= 256

maxnoofSuccessfulHOReports					INTEGER ::= 64

maxnoofPSCellsPerSN							INTEGER ::= 8

maxnoofNR-UChannelIDs						INTEGER ::= 16

maxnoofCellsinCHO							INTEGER ::= 8

maxnoofCHOexecutioncond						INTEGER ::= 2

maxnoofServedCellsIAB						INTEGER ::= 512

maxnoofServingCells							INTEGER ::= 32

maxnoofBHInfo								INTEGER ::= 1024

maxnoofTrafficIndexEntries					INTEGER	::=	1024

maxnoofTLAsIAB								INTEGER	::=	1024

maxnoofBAPControlPDURLCCHs					INTEGER ::= 2

maxnoofIABSTCInfo							INTEGER ::= 45

maxnoofSymbols								INTEGER ::= 14

maxnoofDUFSlots								INTEGER ::= 320

maxnoofHSNASlots							INTEGER ::= 5120

maxnoofRBsetsPerCell						INTEGER ::= 8

maxnoofRBsetsPerCell1						INTEGER ::= 7

maxnoofChildIABNodes						INTEGER ::= 1024

maxnoofPSCellCandidates						INTEGER ::= 8

maxnoofTargetSNs							INTEGER ::= 8

maxnoofUEAppLayerMeas						INTEGER ::= 16

maxnoofSNSSAIforQMC							INTEGER ::= 16

maxnoofCellIDforQMC							INTEGER ::= 32

maxnoofPLMNforQMC							INTEGER ::= 16

maxnoofTAforQMC								INTEGER ::= 8

maxnoofMTCItems								INTEGER ::= 16

maxnoofCSIRSconfigurations					INTEGER ::= 96

maxnoofCSIRSneighbourCells					INTEGER ::= 16

maxnoofCSIRSneighbourCellsInMTC				INTEGER ::= 16

maxnoofNeighbour-NG-RAN-Nodes				INTEGER ::= 256

maxnoofSRBs									INTEGER ::= 5

maxnoofSMBR									INTEGER ::= 8

maxnoofNSAGs								INTEGER ::= 256

maxnoofTargetSNsMinusOne					INTEGER ::= 7

maxnoofThresholdsForExcessPacketDelay		INTEGER ::= 255

maxnoofESNPNs								INTEGER ::= 15

maxnoofSuccessfulPSCellChangeReports		INTEGER ::= 64

maxnoofUEsforRAReportIndications			INTEGER ::= 64

maxnoofPSCellsinCPAC						INTEGER ::= 8

maxnoofCPACexecutioncond					INTEGER ::= 2

maxnoofLBTFailureInformation				INTEGER ::= 64

maxnoofCellsTrajectoryPredict				INTEGER ::= 16

maxnoofCellsTrajectory						INTEGER	::=	16

maxFailedCellMeasObjects					INTEGER ::= 124

maxFailedMeasPerNode						INTEGER ::= 124

maxnoofUEReports							INTEGER ::= 16

maxnoofCandidateRelayUEs					INTEGER ::= 32

maxnoofCAGforMDT							INTEGER ::= 256

maxnoofMDTSNPNs								INTEGER ::= 16

maxnoofSecurityConfigurations				INTEGER ::= 8

maxnoofRSPPQoSFlows							INTEGER ::= 2048

maxnoofThresholds							INTEGER ::= 8

maxnoofEarlyRACHResourcesID					INTEGER	::= 8

maxnoofLTMCells								INTEGER	::= 8

maxNoSSBs									INTEGER	::= 255

maxnoofTAList								INTEGER	::= 8

maxnoofPreambleIndexes						INTEGER ::= 64

maxnoofCSIResourceConfigurationsPlus1		INTEGER ::= 112

maxnoofNZP-CSI-RS-ResourcesPerSet			INTEGER ::= 64

maxnoofSRS-Resource							INTEGER ::= 64

maxnoofFailedSliceMeasObjects				INTEGER ::= 124

maxnoofSliceItemsforMDT						INTEGER ::= 1024

maxnoofAreaNTNforMDT						INTEGER ::= 32

maxnoofLTMCellsPlusOne						INTEGER ::= 9

maxnoofSCGSecurityConfigurations			INTEGER ::= 8

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-ActivatedServedCells																				ProtocolIE-ID ::= 0

id-ActivationIDforCellActivation																	ProtocolIE-ID ::= 1

id-admittedSplitSRB																					ProtocolIE-ID ::= 2

id-admittedSplitSRBrelease																			ProtocolIE-ID ::= 3

id-AMF-Region-Information																			ProtocolIE-ID ::= 4

id-AssistanceDataForRANPaging																		ProtocolIE-ID ::= 5

id-BearersSubjectToCounterCheck																		ProtocolIE-ID ::= 6

id-Cause																							ProtocolIE-ID ::= 7

####### skip unchanged #######

id-LTMInformationSCG-AddReq																			ProtocolIE-ID ::= 532

id-LTMInformationSCG-AddReqAck																		ProtocolIE-ID ::= 533

id-LTMInformationSCG-ModReq																			ProtocolIE-ID ::= 534

id-LTMInterSNExecutionNotification																	ProtocolIE-ID ::= 535

id-LTMPSCellInformation-AddReq																		ProtocolIE-ID ::= 536

id-LTMPSCellInformation-AddReqAck																	ProtocolIE-ID ::= 537

id-LTMPSCellInformation-UpdateReq																	ProtocolIE-ID ::= 538

id-LTMPSCellInformation-UpdateReqAck																ProtocolIE-ID ::= 539

id-LTMPSCellInformation-UpdateReqired																ProtocolIE-ID ::= 540

id-LTMPSCellInformation-UpdateConfirm																ProtocolIE-ID ::= 541

id-LTMPSCellInformation-ChangeRequired																ProtocolIE-ID ::= 542

id-LTMPSCellInformation-ChangeConfirm																ProtocolIE-ID ::= 543

id-LTM-DC-DataForwarding-Indicator																	ProtocolIE-ID ::= 544

id-LTMConfig-to-Modify-List																			ProtocolIE-ID ::= xxx – to be reassigned by MCC

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>