3GPP TSG-RAN WG3 Meeting #129bis	R3-257202

Prague, Czech, 13 ~17 Oct, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.443 | CR | 0135 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Time Interleaving parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Pengcheng Laboratory, China Unicom, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast_Ph2-Core |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The 3GPP LTE-based 5G Broadcast Phase 2 project was finalized in Release 19. The newly introduced PMCH-TFI-Config Information Element (IE) in M2AP must be aligned with the specifications in 36.331. However, as per the agreed RAN2 Change Request (CR) R2-2506344, there is a mismatch in the value range of the Cyclic Shift Alpha parameter defined in the PMCH-TFI-Config IE compared to 36.331. This discrepancy requires a revision to ensure consistency between the two specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To update IE type and reference part of Cyclic Shift Alpha IE inside of Time Interleaving parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support LTE-based 5G Broadcast Phase 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.30, 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: on top of R3-256751, update the WI code and update the revision history part. |  |  |  |  |  |  |  |  |

CHANGES START

#### 9.2.1.30	Time Interleaving parameters

This IE defines the time interleaving parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Value M | M |  | ENUMERATED (sf4, sf8, sf16, sf32, …) | Encoded as the pmch-TimeInterleavingM-r19 contained in the PMCH-TFI-Config-r19 IE in TS 36.331 [11]. |
| Value N | M |  | ENUMERATED (n2, n4, n8, n16, …) | Encoded as the pmch-TimeInterleavingN-r19 contained in the PMCH-TFI-Config-r19 IE in TS 36.331 [11]. |
| Scaling factor Beta | M |  | ENUMERATED (one32th, one5th, one3rd, three8th, five12th, onehalf, five8th, two3rd, five6th, one, …) | Encoded as the scalingFactorBeta-r19 in the softBufferSizeParameters-r19 contained in the PMCH-TFI-Config-r19 IE in TS 36.331 [11]. |
| Reference UE Category | M |  | INTEGER (4..26, ...) | Encoded as the refUE-CategoryDL-r19 in the softBufferSizeParameters-r19 contained in the PMCH-TFI-Config-r19 IE in TS 36.331 [11]. |
| Value M-Last MTCH | O |  | ENUMERATED (sf4, sf8, sf16, sf32, …) | Encoded as the pmch-TimeInterleavingM-LastMTCH-r19 contained in the PMCH-TFI-Config-r19 IE in TS 36.331 [11]. |
| Value N-Last MTCH | O |  | ENUMERATED (n2, n4, n8, n16, …) | Encoded as the pmch-TimeInterleavingN-LastMTCH-r19 contained in the PMCH-TFI-Config-r19 IE in TS 36.331 [11]. |
| Cyclic Shift Alpha | O |  | ENUMERATED (alphaOne, alphaTwo, alphaThree, …) | Encoded as the pmch-CyclicShiftAlpha-r19 contained in the PMCH-TFI-Config-r19 IE in TS 36.331[11]. |

NEXT CHANGE

### 9.3.5	Information Element definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

### //skip unchanged part

#### -- T

TimeInterleavingParameters ::= SEQUENCE {

valueM					ENUMERATED {sf4, sf8, sf16, sf32, ...},

valueN					ENUMERATED {n2, n4, n8, n16, ...},

scalingfactorBeta		ENUMERATED {one32th, one5th, one3rd, three8th, five12th, onehalf, five8th, two3rd, five6th, one, ...},

referenceUECategory		INTEGER (4..26,...),

valueM-LastMTCH			ENUMERATED {sf4, sf8, sf16, sf32, ...}								OPTIONAL,

valueN-LastMTCH			ENUMERATED {n2, n4, n8, n16, ...}									OPTIONAL,

cyclicShiftAlpha		ENUMERATED {alphaOne, alphaTwo, alphaThree, ...}								OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { TimeInterleavingParameters-ExtIEs} } OPTIONAL,

...

}

...

}

CHANGES END