3GPP TSG-RAN WG3 Meeting #129bis	R3-257221

# Prague, Czech Republic, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1351 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding Extended AMF Name to NG REMOVAL RESPONSE message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Thales, LG Electronics, Xiaomi, CATT, ZTE, Nokia, Nokia Shanghai Bell, Eutelsat Group, Deutsche Telekom, Qualcomm Incorporated, Airbus, Jio Platforms, Huawei, NEC, CMCC, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Extended AMF Name IE was added to the NG SETUP RESPONSE message following a request from operators to have a human readable AMF name signaled to the NG-RAN node. Currently the NG REMOVAL RESPONSE message carries the AMF Name IE but it is not possible to add the Extended AMF Name IE. If the AMF had previously signaled the Extended AMF Name IE in the NG SETUP RESPONSE message, the AMF Name IE was ignored by the gNB. Hence, when removing NG to the same AMF, it may not be possible to correlate the information received in the two procedures. This may cause errors. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the Extended AMF Name IE to the NG REMOVAL RESPONSE message. When this IE is signaled, the AMF Name IE is ignored. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Human readable AMF name is not available to the gNB when removing NG. This may cause errors if the AMF had previously signaled the Extended AMF Name IE at NG setup. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.9.2, 9.2.6.17, 9.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 8.7.9	NG Removal

#### 8.7.9.1	General

The purpose of the NG Removal procedure is to remove the interface between the NG-RAN node and the AMF in a controlled manner. If successful, this procedure erases any existing application level configuration data in the two nodes.

The procedure uses non-UE associated signaling.

#### 8.7.9.2	Successful Operation

Figure 8.7.9.2-1: NG Removal, successful operation

The NG-RAN node initiates the procedure by sending the NG REMOVAL REQUEST message to the AMF. Upon reception of the NG REMOVAL REQUEST message, the AMF shall reply with the NG REMOVAL RESPONSE message. After receiving the NG REMOVAL RESPONSE message, the NG-RAN node shall initiate removal of the TNL association towards the AMF and may remove all resources associated with that interface. The AMF may then remove all resources associated with the NG-RAN node.

If the Extended AMF Name IE is included in the NG REMOVAL RESPONSE message, the NG-RAN node may use this IE as a human readable name of the AMF and shall ignore the AMF Name IE.

NEXT CHANGE

#### 9.2.6.17	NG REMOVAL RESPONSE

This message is sent by the AMF to acknowledge the initiation of removal of the interface.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF Name | M |  | 9.3.3.21 | This IE is ignored if the Extended AMF Name IE is present. | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Extended AMF Name | O |  | 9.3.3.51 |  | YES | ignore |

NEXT CHANGE

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

UNCHANGED PART OMITTED

-- **************************************************************

--

###### -- NG REMOVAL RESPONSE

--

-- **************************************************************

NGRemovalResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGRemovalResponseIEs} },

...

}

NGRemovalResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMFName 					CRITICALITY reject	TYPE AMFName					PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		}|

{ ID id-Extended-AMFName			CRITICALITY ignore	TYPE Extended-AMFName			PRESENCE optional		},

...

}

UNCHANGED PART OMITTED

END OF CHANGES