3GPP TSG-RAN WG4 #116	R4-2509034

Bengaluru, India, August 25-29, 2025

Agenda item:	7.23.3.1		On-demand SSB SCell operation

Source:	Qualcomm Incorporated

Title:	Additional comments on core requirements for on-demand SSB SCell operation

Document for:	Discussion

# Introduction

In RAN4 #112, RAN4 work on Rel-19 NES was started and the work plan has been agreed in [1]. Several agreements have been made for on-demand SSB, and additional open issues have been identified in [2].

In this paper we provide our view for the open issues identified in the way forward in [2] in RAN4 #115.

# Discussion

Good progress was made in RAN4 #115 on several aspects of OD-SSB [2]. In the following subsections we address the open topics.

## OD-SSB based deactivated SCell L3 Measurements

### Case #1: OD-SSB only

One of the open issues that could not be closed in RAN4 #115 is the collision scenario of OD-SSB in the fast measurement window with measurement gaps.

Issue 1-1-1: OD-SSB based deactivated SCell measurement – requirement in FMW

Sub-issue 2: OD-SSB in FMW collided with MG

- Option 1: UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion.

- Option 2: UE to prioritize the OD-SSB carrier/layer within FWM

In our view, OD-SSB based measurements should not have higher priority than measurement gaps. Hence, we prefer option 1. The measurement period for OD-SSB should be extended accordingly.

Proposal 1: RAN4 should decide that UE is allowed to skip the OD-SSB measurements if FMW collides with MG occasion.

Another open issue is the UE behaviour when OD-SSB is activated between two FMWs, i.e., before time T2 (Issue 1-1-2).

Issue 1-1-2: OD-SSB based deactivated SCell measurement – Procedure

Sub-issue 4 – UE’s behaviour after T2 when OD-SSB further activation/reconfiguration occurs within T2 between two FMWs

The issue can be further discussed based on contribution driven, and the discussion will not impact the WI completion.

- Option 1: When OD-SSB further activation/reconfiguration occurs within T2, the UE should perform ‘fast measurement’ immediately after T2.

- Option 2: when OD-SSB further activation/reconfiguration occurs within T2, UE should follow legacy deactivated SCell measurement after T2.

- Other option is not precluded.

It remained open whether the UE should follow fast or legacy measurement requirements after the time T2. Note that the activation of OD-SSB happens before T2 and the UE follows therefore legacy requirements. We do not see a reason why the UE should switch to fast measurements after T2 but rather believe that too early activation of OD-SSB can be seen as network misbehaviour. In our view the UE should continue to measure following the legacy requirements.

Observation 1: If the network activates OD-SSB before the time T2, the UE follows the legacy requirements based on measCycleSCell.

Proposal 2: When OD-SSB further activation/reconfiguration occurs within T2, the UE should continue to follow legacy deactivated SCell measurements after T2 (Option 2).

In case of event triggered reporting and the event condition is not met the UE can be required to measure in fast measurement window for a very long time since no report is sent to the network. In this case it makes sense to limit the time of a fast measurement window to avoid significant increase in UE power consumption. This limitation has already been at least implicitly agreed in RAN4 #114 by the following [3]:

Sub-issue 1-1 – The length of FMW

Agreement:

- The time window is defined with Tuncertain plus actual measurement duration.

  - Tuncertain equals Tmin(RAN1) plus additional processing/preparation time, if it allows.

  - The actual measurement duration equals min(Treport , Tidentify) from the end of uncertain time.

    - Treport is the first OD-SSB based L3 measurement reporting

    - Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow.

      - A detected cell: OD-SSB based measurement period

      - A newly detectable cell: OD-SSB based cell identification period

No OD-SSB measurement requirement applies in fast window if NW deactivates the OD-SSB earlier than the end of min(Treport, Tidentify).

The statement that the actual measurement duration equals min(Treport, Tidentify) means that after this time the fast measurement window ends. If in event triggered case no report has been sent after this time, the UE continues with legacy measurement. In case that the cell has already been detected, Tidentify is the measurement period based on OD-SSB periodicity and is given by for FR1:

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |

That means after this time, the UE stops fast measurements even if no report has been sent to the network.

Observation 2: In case of event triggered reporting and the event condition is not met within the time Tidentify, the UE stops doing fast measurements.

Proposal 3: RAN4 should confirm that the UE can stop doing fast measurements after time Tidentify even if no measurement report has been sent to the network.

### Case #2: Always-on SSB and OD-SSB

Another open issue in RAN4 #115 was whether to define requirements for Alt Time-C2. Alt-Time as defined in [4] results in a non-periodic time domain pattern of always-on SSB and OD-SSB.

Issue 1-2-2: Requirement for RAN1 Alt Time-C2

Agreement

- Option 1: RAN4 to define the delay requirement only based on the OD-SSB periodicity.

- Option 2: RAN4 to define the delay requirement only based on the OD-SSB periodicity only if

  - The periodicity of AO-SSB and OD-SSB are same

- It doesn’t impact the WI’s completion.

It has been agreed in RAN4 #115 that requirements for this scenario will be defined, but we still fail to see a use case for this scenario. It can result in an effective SSB burst periodicity less than 5 ms if no additional restrictions are applied. This is not a periodicity supported by SSB in the 5G NR specification. Hence, we prefer to keep the scenario as simple as possible if requirements have to be defined.

Proposal 4: RAN4 to define the delay requirement only based on the OD-SSB periodicity only if the periodicity of AO-SSB and OD-SSB are same.

RAN2 sent a LS to RAN4 for the case when always-on SSB and OD-SSB have the same center frequency [8].

For the case where AO-SSB and OD-SSB have the same center frequency, RAN2 had made the following agreement:

- The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured.

This RAN2 agreement holds for activated SCell but for deactivated SCell, RAN2 did not agree to introduce a new OD-SSB specific SMTC. So far, RAN4 has the understanding that neighbour measurements follow legacy requirements and no change is needed. For deactivated SCell this continues to hold.

Observation 3: If the the SCell is deactivated, neighbour cell measurements follow legacy requirements based on measCycleSCell.

For activated SCell, RAN4 should further discuss whether the RAN2 LS applies to serving cell or neighbour cell measurement. Following the RAN2 discussions, it is our understanding that the RAN2 LS only applies to serving cell measurements but is not targeting neighbour cell measurements. In case the RAN2 agreement is meant for neighbour cell, it contradicts previous RAN4 agreements that neighbour cell measurement follow legacy requirements.

Observation 4: For activated SCell, our understanding of the RAN2 LS is that applies for serving cell only but not for neighbour cell measurements.

Proposal 5: For activated SCell, RAN4 should further discuss whether the RAN2 LS applies to serving cell only or both to serving cell and neighbour cell measurements.

As can be seen in the LS to RAN4 [4], RAN1 agreed that always-on SSB and OD-SSB can be location on different non-overlapping frequencies within the same BWP. This is the case when always-on SSB is a CD-SSB on the synchronization raster. In this case, OD-SSB is not on the sync-raster. RAN1 further agreed in RAN1 #120 that this scenario is subject to a separate UE capability and the UE is not required to measure both always-on SSB and OD-SSB.

Issue 1-2-3: Requirement for scenario 3 (OD-SSB and AO-SSB are within different frequencies)

Sub issue 2: Requirement for scenario 3

Agreement:

RAN4 will further discuss whether and how to define the requirements for scenario 3, and meanwhile the discussion will not impact the WI completion.

In RAN4 #115 it remained open whether requirements should be defined for that case (Issue 1-2-3). In our understanding the use case of this scenario is not very clear, and it will become rather complicated to define requirements in the remaining time of Rel-19 since no discussion has taken place so far in RAN4. Hence, we propose postponing this scenario and do not define requirements in Rel-19.

Observation 5: In our understanding the use case of this scenario is not very clear, and it will become rather complicated to define requirements in the remaining time of Rel-19 since no discussion has taken place so far in RAN4.

Proposal 6: Hence, we propose postponing scenario 3 (OD-SSB and AO-SSB are within different frequencies) and do not define requirements in Rel-19.

## Remaining Timeline Aspects

Regarding the timeline for OD-SSB parameter update there are still two options in Issue 1-4-4, i.e., either whether a UE capability of 2ms and 5 ms is introduced or whether no capability is defined and the timeline is fixed to 5 ms on top of the MAC CE timeline.

Issue 1-4-4: Additional Processing time for OD-SSB parameter update

Agreement

- Option 1: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates.

  - The additional processing time is 5ms or 2ms based on UE capability reporting.

  - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated].

- Option 2: When NW updates the OD-SSB parameters for an active Scell by MAC-CE, additional 5ms processing time is applied on top of THARQ+3ms after UE receiving the MAC-CE.

  - The additional processing time is 5ms

  - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated].

  - Note 2: No capability is needed

Generally, there is now difference in the processing timeline between OD-SSB activation and OD-SSB parameter update. Hence, the same timeline should be applied. Since already a UE capability was introduced for OD-SSB activation, the same capability should also be applied for updating the parameters of OD-SSB.

Observation 6: There is no difference in the processing timeline between OD-SSB activation and OD-SSB parameter update.

Proposal 7: Since already a UE capability was introduced for OD-SSB activation, the same UE capability with 2ms and 5 ms should also be applied for updating the parameters of OD-SSB.

OD-SSB parameter update applies to scenario #3B when the SCell is already activated. OD-SSB can also be applied for SCell activation. Here two different MAC CE for SCell activation and OD-SSB activation will be sent in the same time instance. In this scenario the OD-SSB is used for updating AGC and time/frequency tracking and not for RSRP measurements as discussed so far. However, it does not have any impact to the timeline. Instead, the timeline for MAC CE based OD-SSB activation should also be applied for joint SCell activation and OD-SSB activation.

Observation 7: The timeline for OD-SSB based processing is identical independent whether OD-SSB is used for RSRP measurements, AGC updates or time/frequency tracking refinements.

Proposal 8: For the case of joint SCell activation and OD-SSB activation, a UE capability for OD-SSB processing should be introduced. The UE capability should have the same values as for MAC CE based OD-SSB activation for the deactivated SCell, i.e., 2 ms and 5ms.

We understand that there is a great interest to apply OD-SSB also for fast SCell activation in the same way as aperiodic TRS in Rel-17 fast SCell activation feature. In Rel-17 fast SCell activation, A-TRS can come directly after the MAC CE processing time of m + 3Nslotsubframe+1 and companies want to have the same timeline for OD-SSB. Hence, we propose to add an additional value of 0 ms to the UE capability for OD-SSB processing for joint SCell activation and OD-SSB activation.

Proposal 9: An additional value of 0 ms should be added to the UE capability for OD-SSB processing in case of joint SCell activation and OD-SSB processing.

In RAN4 #115 a LS from RAN1 was received regarding the time instance B for SCell operation [5]. RAN4 answered in [6] by confirming the RAN1 understanding from a network transmission perspective. RAN4 also replied that there are still discussions whether additional UE processing time is needed for OD-SSB deactivation.

- RAN4 to confirm that T_min equals to m+3Nslotsubframe,μ+1 from network transmission perspective.

- RAN4 is discussing whether the additional UE processing time for RAN4 RRM requirement due to OD-SSB deactivation is needed.

The discussion whether additional processing time is needed in RAN4 for OD-SSB deactivation is reflected in Issue 1.4.5 in option 1 and option 2.

Issue 1-4-5: RAN1 LS - OD-SSB deactivation timeline

Agreement

- RAN4 to confirm RAN1’s working assumption about OD-SSB deactivation processing time. Further discuss the wording in the reply LS.

- Further discuss in RAN4 the UE additional processing time for OD-SSB deactivation.

  - Option 1: No UE additional processing time for OD-SSB deactivation

  - Option 2: Follow the same values as UE additional processing time for OD-SSB activation

The point to discuss is whether the UE can be fully aware in time instance B’ that no OD-SSB is transmitted in this time instance by the network. As for MAC CE based activation and MAC CE based parameter updates, the MAC CE based deactivation needs to cover the worst case of all possible scenarios. In case that the network stops transmission, but UE has not stopped OD-SSB based measurement quickly enough, the UE will measure an OD-SSB time location that does not carry OD-SSB anymore. This would mess up filtering in case of L3 measurements and L1 measurements.

Observation 8: In case that network stops transmission and UE has not stopped OD-SSB based measurement quickly enough, the UE will measure an OD-SSB time location that does not carry OD-SSB anymore. This will mess measurements.

Certainly, MAC CE deactivation timeline will be shorter in most cases than MAC CE activation timeline but in some cases, it will be in the same order. However, to keep the timeline specification simple, we suggest that RAN4 defines the same timeline for activating and deactivating OD-SSB based MAC on MAC CE.

Proposal 10: To keep the specification simple, RAN4 should define the same timeline for activating and deactivating OD-SSB based on MAC CE.

Obviously, first the OD-SSB deactivation timeline needs to be defined by RAN4 before the wording of the reply LS can be discussed.

Proposal 11: The wording for the reply LS to RAN1 should be defined after the agreement for the OD-SSB deactivation timeline has been reached in RAN4.

It should be noted that the discussion for the timeline considered only OD-SSB activation and deactivation for a single carrier. It still needs to be analysed whether the processing timeline can also be applied for OD-SSB activation of multiple carriers.

Observation 9: The discussion for the timeline considered only OD-SSB activation and deactivation for a single carrier. It still needs to be analysed whether the processing timeline can also be applied for OD-SSB activation of multiple carriers.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #115. Our observations and proposals are summarized as follows:

Proposal 1: RAN4 should decide that UE is allowed to skip the OD-SSB measurements if FMW collides with MG occasion.

Observation 1: If the network activates OD-SSB before the time T2, the UE follows the legacy requirements based on measCycleSCell.

Proposal 2: When OD-SSB further activation/reconfiguration occurs within T2, the UE should continue to follow legacy deactivated SCell measurements after T2 (Option 2).

Observation 2: In case of event triggered reporting and the event condition is not met within the time Tidentify, the UE stops doing fast measurements.

Proposal 3: RAN4 should confirm that the UE can stop doing fast measurements after time Tidentify even if no measurement report has been sent to the network.

Proposal 4: RAN4 to define the delay requirement only based on the OD-SSB periodicity only if the periodicity of AO-SSB and OD-SSB are same.

Observation 3: If the the SCell is deactivated, neighbour cell measurements follow legacy requirements based on measCycleSCell.

Observation 4: For activated SCell, our understanding of the RAN2 LS is that applies for serving cell only but not for neighbour cell measurements.

Proposal 5: For activated SCell, RAN4 should further discuss whether the RAN2 LS applies to serving cell only or both to serving cell and neighbour cell measurements.

Observation 5: In our understanding the use case of this scenario is not very clear, and it will become rather complicated to define requirements in the remaining time of Rel-19 since no discussion has taken place so far in RAN4.

Proposal 6: Hence, we propose postponing scenario 3 (OD-SSB and AO-SSB are within different frequencies) and do not define requirements in Rel-19.

Observation 6: There is no difference in the processing timeline between OD-SSB activation and OD-SSB parameter update.

Proposal 7: Since already a UE capability was introduced for OD-SSB activation, the same UE capability with 2ms and 5 ms should also be applied for updating the parameters of OD-SSB.

Observation 7: The timeline for OD-SSB based processing is identical independent whether OD-SSB is used for RSRP measurements, AGC updates or time/frequency tracking refinements.

Proposal 8: For the case of joint SCell activation and OD-SSB activation, a UE capability for OD-SSB processing should be introduced. The UE capability should have the same values as for MAC CE based OD-SSB activation for the deactivated SCell, i.e., 2 ms and 5ms.

Proposal 9: An additional value of 0 ms should be added to the UE capability for OD-SSB processing in case of joint SCell activation and OD-SSB processing.

Observation 8: In case that network stops transmission and UE has not stopped OD-SSB based measurement quickly enough, the UE will measure an OD-SSB time location that does not carry OD-SSB anymore. This will mess measurements.

Proposal 10: To keep the specification simple, RAN4 should define the same timeline for activating and deactivating OD-SSB based on MAC CE.

Proposal 11: The wording for the reply LS to RAN1 should be defined after the agreement for the OD-SSB deactivation timeline has been reached in RAN4.

Observation 9: The discussion for the timeline considered only OD-SSB activation and deactivation for a single carrier. It still needs to be analysed whether the processing timeline can also be applied for OD-SSB activation of multiple carriers.

# References

- R4-2112508, Work plan for R19 NES, RAN4 #112, August 2024

- R4-2508277, WF on RRM requirements for Netw_Energy_NR_enh_Part1, RAN4 #115, May 2025

- R4-2502598, WF on RRM requirements for Netw_Energy_NR_enh_Part1, RAN4 #114, February 2025

- R1-2501633, LS on time location of on-demand SSB for SCell, RAN1 #120, February 2025

- R1-2503108, LS to RAN4 on time instance B for on-demand SSB SCell operation, RAN1 #120bis, April 2025

- R4-2508331, Reply LS on time instance B for on-demand SSB SCell operation, RAN4 #115, May 2025

- R4-2508440, Reply LS on servingCellMO of OD-SSB, RAN4 #115, May 2025

- R2-2504981, LS on L3 measurements of OD-SSB, May 2025