3GPP TSG-RAN WG4 Meeting #116										                    R4-2509119

Bangalore, India, August 25th – 29th, 2025

Source:	Murata

Title:	LP WUR UE RX ACS

Agenda item:		7.24.3.2

Document for:	Approval

# 1.	Introduction

In this contribution, we will discuss the LP-WUR ACS.

# 2.	Discussion

We discuss and propose LP-WUR ACS, and associated test parameters based on the agreed WF [1] and using a similar objective taken in [2]. In section 2.1, we formulate the low power ACS specification and in section 2.2, we justify the specification to all NR bands.

## Low Power Wake Up Radio ACS Formulation

In this section, we derive the low power ACS in terms of the main radio ACS given the agreed RX parameters. Using the same interference level for both the main radio and LP-WUR for selectivity determination was the main agreement. Questions remain as to whether the ACS test parameter or wanted level of LP-WUS needs to be larger, which reduces the UEs burden to maintain the expected selectivity under a low power condition in the presence of interference.

Figure 2.1-1 shows the LP-WUR radio as well as the main radio interference scenario. The interference levels remain the same for both scenarios, except for the LP-WUR RX level. The objective is to find a suitable LP-WUR ACS requirement in terms of the main radio ACS, then apply or adjust the test parameters if required upon completion of the updated coexistence studies.

Figure 2.1-1: LP-WUR in the presence of Main Radio interference

The RX selectivity is the ratio of the interference level to the radio’s co-channel noise in the wanted signal’s bandwidth. Using figure 2.1-1 as a reference:

ACSM,BW =(∆M,BW-14)+SNRM+IM

ACSLP=(PrefM+∆M,BW)-(PrefLP+∆LP)+SNRLP+IM

∆M,BW=(Pinterferer-REFSENS) is from Table 7.5-3 in 38.101-1. ACSM,BW is from Table 7.5-1 in 38.101-1

ACSLP can be derived in terms of ACSM,BW as follows:

ACSLP=ACSM,BW+14-∆LP+SNRLP-SNRM-∆RLP,BW

For this situation, ∆RLP,BW, is simply the REFSENS difference between the 2 radios and is a parameter that defines the low power radio REFSENS in terms of the main radio REFSENS for any given UE channel BW. Specifically:

∆RLP,BW=PrefLP-PrefLM,BW

∆RLP,BW=10log10BWMBWLP+NFM-NFLP+SNRLP-SNRM+DG

A quick observation shows that ACSLP remains essentially constant Vs UE channel BW because the frequency variation of ACSM,BW  and  ∆RLP,BW counteract each other. In other words, the reduction of the main radio ACS Vs frequency due to the higher cutoff frequency of the channel filter is compensated by the increase in the difference between the REFSENS of the 2 radios Vs UE channel BW. Furthermore, the LP-WUR channel filter is fixed, so the selectivity must not be relaxed for the same WUR transmission BW for any given UW channel BW. Only if the SCS of the LP-WUS increases will the ACS reduce with a corresponding increase in the wanted level because the LP-WUR channel filter BW has increased for the same LP PRBs. Lastly, some consideration is necessary of the wanted signal delta test parameter, which is defined as ∆LP in figure 2.1-1. The summary is shown in Table 2.1-1 along with the existing main radio requirement. Intuitively, and as shown in the figure, increasing ∆LP to overcome low power radio impairments such as phase noise (test case 1 and 2) and linearity (test case 2), would also mean lowering ACSLP for the required SNR. The results of the coexistence simulations completed in the study item were inconclusive. In [3], in the study item phase, we showed the degradation of the in-band co-channel noise using various phase noise models with varying PLL bandwidths for example. Phase noise was not used in the coexistence study item for ACS determination. Hence for the OOK based RX, the ∆LP = [14] dB and ACS is in square brackets (Table 2.1-1) pending further studies.

Using the agreed parameters for SNR and NF, ACSM,BW  and ACSLP are plotted and shown in Figure 2.1-2 using the formula derivation.

SNRLP= -4.5dB

SNRM= -1.0dB

DG= 3dB

NFM= 9.0dB

NFLP= 13.5dB for OFDM based RX

NFLP= 18.0dB for OOK based RX

Figure 2.1-2a: ACSLP Vs UE channel bandwidth compared to Main radio using the formula. OOK-RX (left) and OFDM (right). ∆LP=14dB. SCSLP=15KHz.

Figure 2.1-2b: ACSLP Vs UE channel bandwidth compared to Main radio using the formula. OOK-RX (left) and OFDM (right). ∆LP=14dB. SCSLP=30KHz

Table 2.1-1: Low Power ACS Summary for all SCS and UE channel BW

Observation 1: LP-WUR ACS is essentially constant Vs the UE channel BW.

Observation 2: LP-WUR ACS will decrease for increased LP-WUS bandwidth (larger SCS for the same NRB) and increased LP-WUR RX NF (OOK based Vs OFDM based). The 3dB ACS decrease for larger SCS is reflected by the 3dB increased LP REFSENS.

Observation 3: The LP-WUR ACS requirement will decrease 1dB for every 1dB increase in the wanted low power wake-up signal test parameter. Coexistence studies with all RF impairments and agreed SNR for 1% MDR and 1% FAR should be completed to determine further ACS relaxation for OOK based RX and the elimination of the square brackets.

## ACS for all NR bands

ACS requirements in the current specification differ for bands >3300MHz and bands < 2700MHz. Adding 2 sets of ACS requirements for the 2 different band groups unnecessarily complicates the specification. Figure 2.2-1 shows the derived low power ACS requirement in terms of the main radio ACS requirement for NR bands >3300MHz using the methodology in section 2.1. From the plot, it can be seen, that the low power selectivity requirement increases for increasing channel BW due to maintaining the same interference level for all BWs but using an interference bandwidth the same as the UE BW and spreading the power over the interference BW. However, it can be argued that such a wide bandwidth requirement does not reflect any more of a worst-case ACS test-point than measuring using a narrow band interference with higher PSD.

For example, the lower band group scenario for IBB case 1 and IBB case 2 already equivalently provide a larger selectivity requirement at the larger interferer frequency offset. So, concentrating the power in a smaller interference bandwidth at a close offset provides an equally alternative measure of radio selectivity than spreading the interference over larger bandwidth. Additionally, the LP-WUR channel filter is much smaller than the larger NR BWs and will attenuate the interference power more so than the UE channel BW filter. So, for these reasons, it is preferable to maintain a single LP-WUS ACS requirement for all NR bands.

Figure 2.2-1: Projected ACSLP Vs UE channel bandwidth for 2 NR band groups.

Observation 4: Concentrating the power in a smaller interference bandwidth provides an equally alternative measure of radio selectivity than spreading the interference over a wider bandwidth.

Proposal 1: Use the following ACS values and test case parameters in the following table for all NR bands.

| Case | Parameter | LP-WUR Type |  |
| --- | --- | --- | --- |
|  |  | OOK RX based | OFDM RX based |
|  | ACS, dB | 25-10log10(2μ-1)⁡ | 29.5-10log10(2μ-1) |
| Test Case1 | Power in LP-WUS transmission BW, dBm | 14+REFSENSLP | 14+REFSENSLP |
|  | Pinterferer, dBm | REFSENSM+∆M,BW | REFSENSM+∆M,BW |
| Test Case2 | Power in LP-WUS transmission BW, dBm | [-52]+10log10(2μ-1)⁡ | -56.5+10log10(2μ-1) |
|  | Pinterferer, dBm | -25 | -25 |
| ∆M,BW=(Pinterferer-REFSENS) from Table 7.5-3 in 38.101-1, and μ=SCSLP15KHz BWinterferer=5MHz ±Finterfereroffset=LP-WUSBW2+2.5MHz [1] guard RB for OOK and 0 guard RB for OFDM. |  |  |  |

# Conclusion

Observation 1: LP-WUR ACS is essentially constant Vs the UE channel BW.

Observation 2: LP-WUR ACS will decrease for increased LP-WUS bandwidth (larger SCS for the same NRB) and increased LP-WUR RX NF (OOK based Vs OFDM based). The 3dB ACS decrease for larger SCS is reflected by the 3dB increased LP REFSENS.

Observation 3: The LP-WUR ACS requirement will decrease 1dB for every 1dB increase in the wanted low power wake-up signal test parameter. Coexistence studies with all RF impairments and agreed SNR for 1% MDR and 1% FAR should be completed to determine further ACS relaxation for OOK based RX and the elimination of the square brackets.

Observation 4: Concentrating the power in a smaller interference bandwidth provides an equally alternative measure of radio selectivity than spreading the interference over a wider bandwidth.

Proposal 1: Use ACS values and test case parameters in the following table for all NR bands.

| Case | Parameter | LP-WUR Type |  |
| --- | --- | --- | --- |
|  |  | OOK RX based | OFDM RX based |
|  | ACS, dB | 25-10log10(2μ-1)⁡ | 29.5-10log10(2μ-1) |
| Test Case1 | Power in LP-WUS transmission BW, dBm | 14+REFSENSLP | 14+REFSENSLP |
|  | Pinterferer, dBm | REFSENSM+∆M,BW | REFSENSM+∆M,BW |
| Test Case2 | Power in LP-WUS transmission BW, dBm | [-52]+10log10(2μ-1)⁡ | -56.5+10log10(2μ-1) |
|  | Pinterferer, dBm | -25 | -25 |
| ∆M,BW=(Pinterferer-REFSENS) from Table 7.5-3 in 38.101-1, and μ=SCSLP15KHz BWinterferer=5MHz ±Finterfereroffset=LP-WUSBW2+2.5MHz [1] guard RB for OOK and 0 guard RB for OFDM. |  |  |  |

# 4.  References:

- R4-2507936, “WF on NR_LPWUS_UERF”, Vivo, RAN4#115, Malta, May 19th  – 23rd, 2025.

- R4-2505368, “On UE RF requirements for the LP-WUR”, Qualcomm, RAN4#115, Malta, May 19th  – 23rd, 2025.

- R4-2320817, “LP WUR ACS with phase noise”, Murata, RAN4#109, Chicago, USA, November 13th  – 17th, 2023.

# Appendix