3GPP TSG-RAN4 #116	R4-2509133

Bengaluru, India, 25th Aug. – 29th Aug. 2025

Title:	Feasibility of reduction in PMI report and scheduled PDSCH separation for AI/ML CSI prediction

Source:			Anritsu Corporation

Agenda Item:			7.17.3

Document for:			Discussion

# 1.	Introduction

We continued a discussion on the AI/ML CSI prediction at RAN4 #115 meeting in Malta and the issue 1-2 in the following topic summary [1] was created as an action to be confirmed by TE vendors.

- Sub-topic 1-2 PMI report and scheduled PDSCH separation The time distance between the report with the predicted PMI and the actual scheduled PDSCH should be discussed Issue 1-2: PMI Report and Scheduled PDSCH separation - Proposals - Option 1: Reduce the time separation between PMI report and scheduled PDSCH relative to the values assumed in Rel-18 prediction tests - RAN4 considers assuming the following in FDD tests: “This reported PMI cannot be applied at the gNB downlink before slot#(n+3)” - Option 2: Reuse the same values as in Rel-18 prediction tests - Option 3: others - Recommended WF - To be discussed

In this contribution, we share our findings on the feasibility of time separation reduction between PMI report and scheduled PDSCH by test equipment.

# 2.	Discussion

# 2.1 Feasibility of time separation reduction between PMI report and scheduled PDSCH by test equipment

As shown at issue 1-2 in the topic summary [1] above, we discussed a reduction of time separation between PMI report and scheduled PDSCH. And there was a question to TE vendors on a feasibility during the session. Thus we checked the feasibility of time separation reduction between PMI report and scheduled PDSCH by our test equipment.

According to the proposed option 1, the target reduction is from “before slot#(n+4)” to “before slot#(n+3)”. And based on the test equipment implementation, we confirmed that it is possible to reflect the change on PDSCH at slot#(n+2) after receiving the PMI report. Further reduction of the time separation depends on the timing when the TE receives a CSI/PMI report within a slot.

Observation 1: It is possible to schedule PDSCH at slot#(n+2) based on the received PMI report from TE point of view.

Therefore, it is possible to reduce the time separation between PMI report and scheduled PDSCH and change from “before slot#(n+4)” to “before slot#(n+3)”.

Observation 2: It is possible to reduce time separation between PMI report and scheduled PDSCH and change from “before slot#(n+4)” to “before slot#(n+3)” from test equipment viewpoint.

- Conclusion

In this contribution, we confirmed the feasibility of time separation reduction between PMI report and scheduled PDSCH by test equipment.

Observation 1: It is possible to schedule PDSCH at slot#(n+2) based on the received PMI report from TE point of view.

Observation 2: It is possible to reduce time separation between PMI report and scheduled PDSCH and change from “before slot#(n+4)” to “before slot#(n+3)” from test equipment viewpoint.

# 4.	References

[1] R4-2507570, “Topic summary for [115][131] NR_AIML_air_part1”, Moderator (Qualcomm), RAN4 #114-bis, Wuhan, China

[2] R4-2505508, “CSI Prediction: Testing Framework”, Qualcomm Inc., RAN4 #115, St. Julian’s, Malta