3GPP TSG RAN WG4 Meeting #116                  	R4-2509146

Bengaluru, IN, 25th - 29th Aug, 2025

Title:	RRM requirements on XR enhancements

Source:	China Telecom

Agenda item:			7.26.3

Document for:	Discussion

- Introduction

In the RAN4 #115 meeting, the RRM requirements for XR enhancements were discussed and the WF [1] has been approved. In this paper, we present our views on some open issues.

- Discussion

Topic #3: XR skipping requirements impact

Issue 3-1: Formulas for measurement delay extension without DRX

The following options are listed in the WF:

- RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Option 1: Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)),

  - Option 2: Max(200 ms, ( 8 + Nskip ) × Max(MGRP , SMTC period) ) × CSSFinter

  - Option 3:

    - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms

    - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms

    - where Tskip = Nskip ´ Max(MGRP , SMTC period)

  - Other options are not precluded

  - where Nskip is the number of skipped MG occasions in the measurement period.

  - FFS: CSSFinter

In our understanding, option 2 is the simple extension of the existing formula using the NR-U approach. Thus option 2 should be the baseline for defining requirements, and option 1 can be further discussed and adopted if consensus can be reached.

Proposal 1: Use option 2 as baseline of the formula for measurement delay extension, and option 1 can be further discussed and adopted if consensus can be reached.

- Option 2: Max(200 ms, ( 8 + Nskip ) × Max(MGRP , SMTC period) ) × CSSFinter

Issue 3-2: Formulas for measurement delay extension with DRX

The following options are listed in the WF:

- Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - DRX cycle ≤ 320 ms

    - Max(200 ms  CSSFinter, Ceil(8 1.5)  Max(MGRP, SMTC period, DRX cycle)  CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) )

  - DRX cycle > 320 ms

    - 8  DRX cycle  CSSFinter + Nskip  DRX cycle

- Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

  - DRX cycle ≤ 320 ms

    - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5)  Max(MGRP, SMTC period, DRX cycle))  CSSFinter

  - DRX cycle > 320 ms

    - Ceil(8+XPSS/SSS,gaps)  DRX cycle  CSSFinter

- Option 3: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms

  - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms

  - Where

    - DRX cycle ≤ 320 ms, Tskip = Nskip  Max(MGRP , SMTC period, DRX cycle)

    - DRX cycle > 320 ms, Tskip = Nskip  DRX cycle

- Other options are not precluded

Option 2 is also preferred in order to keep the UE behaviour aligned between DRX and non-DRX cases.

Proposal 2: For the formula for measurement delay extension with DRX, option 2 is also preferred.

- Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

  - DRX cycle ≤ 320 ms

    - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5)  Max(MGRP, SMTC period, DRX cycle))  CSSFinter

  - DRX cycle > 320 ms

    - Ceil(8+XPSS/SSS,gaps)  DRX cycle  CSSFinter

Issue 3-5: Conditions for measurement skipping

The following options are listed in the WF:

- Option 1: No conditions are defined

- Option 2: Define the maximum number of gaps skipped during the measurement period.

- Option 3: Define a maximum separation between two non-skipped gaps for measurement

In our view, the conditions in option 2 and option 3 will not only restrict network scheduling, but also bring extra issues for RAN4 discussion. To make this feature not too complicated, and also complete the RAN4 work on time, option 1 is supported.

Proposal 3: No additional conditions are to be defined for measurement skipping.

- Summary

In this paper, we provide our views on RRM requirements for XR enhancements. From this discussion we have the following proposals:

Proposal 1: Use option 2 as baseline of the formula for measurement delay extension, and option 1 can be further discussed and adopted if consensus can be reached.

- Option 2: Max(200 ms, ( 8 + Nskip ) × Max(MGRP , SMTC period) ) × CSSFinter

Proposal 2: For the formula for measurement delay extension with DRX, option 2 is also preferred.

- Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

  - DRX cycle ≤ 320 ms

    - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5)  Max(MGRP, SMTC period, DRX cycle))  CSSFinter

  - DRX cycle > 320 ms

    - Ceil(8+XPSS/SSS,gaps)  DRX cycle  CSSFinter

Proposal 3: No additional conditions are to be defined for measurement skipping.

- References

[1] R4-2508285, WF on RRM requirements for XR_Ph3, Nokia