3GPP TSG-RAN WG4 Meeting #116	R4-2509279

# Bengaluru , IN, 25th – 29th Aug, 2025

Title: 	Discussion on enhancements for Rel-19 ATG RRM performance requirements

Source: 	CATT

Agenda item:	7.8.6

Document for:	Discussion

- Introduction

In last RAN4 meeting, RAN4 has reached good progress in defining the perf RRM requirements and draft CRs has been endorsed [1]. In this contribution, we will continue to discuss and provide our views on the performance requirements based on the agreed core requirements.

- Discussion

- Test setups - TDD pattern for SCell

In the last meeting, the proposals on TDD pattern for SCell for R19 ATG CA are copied as below [1]:

- Proposal 1: Reuse the legacy TDD pattern (CATT, LGE, CMCC, ZTE) - Proposal 1-1: (CATT) - For intra-band CA, TDD pattern for SCell need to be the same with PCell. - For inter-band CA, the pattern used in R18 ATG test cases can also be reused. - Proposal 1-2: (ZTE) - For TDD case, the TDD pattern for SCell could be same as PCell (ZTE) - Proposal 2: RAN4 to discuss whether or not to define a new TDD pattern for ATG (HW) - Proposal 3: Define all ‘D’ slot TDD pattern for ATG (Ericsson)

For TDD pattern, RAN4 agreed that the legacy TDD pattern can be used for PCell, but there was still FFS for SCell. In our view, for intra-band CA, n79 is TDD band, so aggregated TDD cell need to have the same TDD configuration, which is to ensure HARQ timing synchronization, avoid conflicts in timing alignment, and ensure cross carrier scheduling. For inter-band CA, the example band combination is n3_n39, which is FDD and TDD band, so if SCell is operating in TDD band, then the TDD pattern or SCell can be independently configured and the pattern used in R18 ATG can also be reused.

Observation 1: For intra-band CA, the aggregated TDD cells need to have the same TDD configuration, which is to ensure HARQ timing synchronization, avoid conflicts in timing alignment, and ensure cross carrier scheduling.

Proposal 1: For intra-band CA, TDD pattern for SCell need to be the same with PCell.

Proposal 2: For inter-band CA, if SCell is operating in TDD band, then the TDD pattern for SCell can be independently configured and the pattern used in R18 ATG test cases can also be reused.

- Test scope

For the test scope, acoording to the agreement in core part, we use the following table from the WF to present our specific viewpoints [1], which is:

Proposal 3: At least the following test cases can be defined for R19 ATG CA:

| Test category | Test cases |
| --- | --- |
| Interruption | Interruptions during measurements on deactivated NR SCC in FR1 |
|  | Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot |
|  | Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot |
|  | Interruption at NR SRS antenna port switching |
| SCell Activation and deactivation | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle |
|  | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle |
|  | SCell Activation and deactivation of unknown SCell in FR1 in non-DRX |
|  | Direct SCell activation at SCell addition of known SCell in FR1 |
|  | Direct SCell activation at handover with known SCell in FR1 |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle |
|  | SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle |
|  | TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX |
| Active BWP Switch | Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs |
|  | Simultaneous RRC-based Active BWP Switch on multiple CCs |
| SCell dormancy switch | SCell dormancy switch |
| deactivated SCell measurement | Event triggered reporting on SCC with deactivated SCell test under non-DRX |
| Pre-configured measurement gap | Test case for Pre-configured measurement gap requirement |

- Conclusion

This document discussed the issues for R19 ATG RRM performance requirements and presented following observation and proposals:

Observation 1: For intra-band CA, the aggregated TDD cells need to have the same TDD configuration, which is to ensure HARQ timing synchronization, avoid conflicts in timing alignment, and ensure cross carrier scheduling.

Proposal 1: For intra-band CA, TDD pattern for SCell need to be the same with PCell.

Proposal 2: For inter-band CA, if SCell is operating in TDD band, then the TDD pattern for SCell can be independently configured and the pattern used in R18 ATG test cases can also be reused.

Proposal 3: At least the following test cases can be defined for R19 ATG CA:

| Test category | Test cases |
| --- | --- |
| Interruption | Interruptions during measurements on deactivated NR SCC in FR1 |
|  | Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot |
|  | Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot |
|  | Interruption at NR SRS antenna port switching |
| SCell Activation and deactivation | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle |
|  | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle |
|  | SCell Activation and deactivation of unknown SCell in FR1 in non-DRX |
|  | Direct SCell activation at SCell addition of known SCell in FR1 |
|  | Direct SCell activation at handover with known SCell in FR1 |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle |
|  | SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle |
|  | TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX |
| Active BWP Switch | Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs |
|  | Simultaneous RRC-based Active BWP Switch on multiple CCs |
| SCell dormancy switch | SCell dormancy switch |
| deactivated SCell measurement | Event triggered reporting on SCC with deactivated SCell test under non-DRX |
| Pre-configured measurement gap | Test case for Pre-configured measurement gap requirement |

- Reference

[1]	R4-2508263, WF on RRM requirements for NR_ATG_enh, CMCC