3GPP TSG-RAN WG4 Meeting #116	R4-2509289

# Bengaluru , IN, 25th – 29th Aug, 2025

Title: 	Discussion on CSI-RS based L1 measurement for Rel-19 LTM enhancements

Source: 	CATT

Agenda item:	7.25.3.2

Document for:	Discussion

- Introduction

In the last meeting, the WF on RRM requirements for NR_Mob_Ph4_Part1 was approved [1]. This document will discuss the RRM issues about CSI-RS based L1 measurement for LTM, and give our understanding and proposals.

- Discussion

- Issue 1-1: whether step 2 is skipped for serving cell?

For the whole CSI-RS based L1 measurement procedure, RAN4 has agreed the entire framework in RAN4#113 meeting and RAN4 agreed that whether step 2 can be skipped for neighboring cell or not will based on new UE capability reporting, but there was still FFS for whether step 2 can be skipped for serving cell or not. The agreement and candidate options are copied as below [1]:

RAN4#113: <Agreement> - Agree the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements and reports at least before LTM configuration. - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether Step 2 can be skipped under certain conditions, or based on UE capability or based on network RRC configuration. FFS the details. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Depending on whether step 2 is skipped, the requirements in step 3 may be different. - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.] RAN4#114-bis: Agreement: - Whether step 2 can be skipped for neighboring cell or not based on new UE capability reporting. - Further discuss whether step 2 can be skipped for serving cell or not. RAN4#115: Candidate options: - Option A: RAN4 requirements regarding ‘skipping step 2’ do not apply for the case CSI-RS based L1 measurement for LTM candidate cell is configured in legacy CSI-RS based L1 measurement in serving cell in FR2. (QC, HW) - Option A1: Requirement regarding ‘skipping step 2’ is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to ‘off’, is one of the FR2 serving cells. (QC, OPPO) - Option A2: The CSI-RS based L1-RSRP measurement requirement is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to 'off', is one of the FR2 serving cells (QC). - Option B: keep the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM. (MTK, Apple, E///)

The controversy on the above issue in the last meeting lies in whether step 2 can be skipped for serving cell or not in FR2. As per RAN4#114bis agreement, whether step 2 can be skipped for neighboring cell or not based on new UE capability reporting has been a compromise. Considering that the candidate cell may also be a serving cell for LTM and skipping step 2 will introduce a new measurement behavior for serving cell, we also agree that using two different measurement procedures for the same cell is too complex. Therefore, keeping the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM is acceptable for us, which means ‘skipping step 2’ do not apply for serving cell.

Observation 1: Using two different measurement procedures for the same cell is too complex.

Proposal 1: Keeping the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM is acceptable for us, which means ‘skipping step 2’ do not apply for serving cell.

- Issue 1-2: Sharing factor for resource collision between serving and neighbour cell

In the last meeting, one issue was about how to handle a collision in time domain of CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell, and the candidate options are copied as below [1]:

Candidate options: - For FR2 CSI-RS L1 measurement requirement, - if the CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell are colliding in time domain,. - Option 1: sharing factor shall be introduced. (HW, QC, E///) - Option 2: introduce measurement restriction as legacy. (MTK, vivo)

Since R18 LTM has defined measurement restriction for the case of collision in time domain of CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell in FR2. When step 2 is skipped, RAN4 can also introduce measurement restriction as legacy.

Proposal 2: If the CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell are colliding in time domain, RAN4 to introduce measurement restriction as legacy.

- Issue 1-3: Measurement restriction

In the last meeting, one undecided issue was about measurement restriction, and the candidate options are copied as below [1]:

Candidate options: - Option 1: Apple, ZTE, Ericsson - In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5ms. - Option 2: vivo - When CSI-RS based L1 measurements cause scheduling restriction, all CSI-RS resources on one intra-frequency layer are configured within up to two separate windows where each window is up to 5 ms and the two windows, if configured, are separated by 20ms.

For CSI-RS based L3 measurement, the configuration of CSI-RS for mobility is used and the maximum periodicity is 40 ms, whereas for CSI-RS for L1-RSRP, the configuration of CSI-RS for beam management and the maximum periodicity is 640 slots. Therefore, the configuration restriction needs to be clarified. We believe that legacy restriction for CSI-RS based L3 measurement can be reused, that is, when CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5ms.

Proposal 3: When CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5ms.

- Issue 1-4: Early CSI acquisition

In the last meeting, the issue for early CSI acquisition hasn’t been discussed, and the candidate options are copied as below [1]:

Candidate options: - Option 1: Define timeline for baseline early CSI acquisition. (vivo, Nokia, E///) - Option 2: Define scheduling/measurement restriction requirements for optional early CSI acquisition. (QC) - Option 3: not define RAN4 requirements for early CSI acquisition. (MTK, HW, Apple)

As we know, RAN1 has supported early CSI acquisition, and CSI reporting is performed after MAC CE of LTM Cell switch command. However, RRM has not previously defined any requirements for CSI related procedure before, and considering this meeting will be the last meeting for core part, so we prefer not to define RAN4 requirements for early CSI acquisition.

Proposal 4: Prefer not to define RAN4 requirements for early CSI acquisition.

- Conclusion

This document discussed the issues for CSI-RS based L1 measurement for LTM in Rel-19 and the following proposals and observations are presented:

Observation 1: Using two different measurement procedures for the same cell is too complex.

Proposal 1: Keeping the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM is acceptable for us, which means ‘skipping step 2’ do not apply for serving cell.

Proposal 2: If the CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell are colliding in time domain, RAN4 to introduce measurement restriction as legacy.

Proposal 3: When CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5ms.

Proposal 4: Prefer not to define RAN4 requirements for early CSI acquisition.

- Reference

[1]        R4-2508282, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple