3GPP RAN WG4 Meeting #116	R4-2509303

Bengaluru, India, August 25th – August 29th, 2025

Title: 	Discussion on impacts of AIML measurement event prediction on RRM requirements

Source: 	CATT

Agenda item:	7.19.3.2

Document for:	Discussion

Introduction

In RAN4#115 meeting, RAN4 discussed the RRM impacts of AI/ML measurement event prediction and the agreements were captured in the WF [1]. In this contribution, we will further discuss the open issues in [2] and their potential impacts and present our views.

Discussion

Performance metric for event prediction use case

Measurement event prediction use case

According to RAN2 discussions, there are two types of measurement event prediction: direct and indirect prediction, differences of which are copied below [3]:

RAN2#127bis - For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighboring cells. The final output is the expected occurrence time of a certain measurement event (ex. event A3). - For direct measurement event prediction, the model output is the probability of event occurrence within a time window.

Furthermore, RAN2 has following agreements in RAN2#128 meeting, which further categorizes indirect measurement event prediction into four cases [4]:

Agreements on event prediction 1 Agree to listed 4 definitions of indirect measurement event prediction Indirect measurement event prediction for temporal domain case A: In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for temporal domain case B: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case B at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for frequency domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in frequency domain at first. Afterwards, predicted and optional actual historical measurement result(s) of serving cell are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for spatial domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in spatial domain at first. Afterwards, predicted measurement result(s) and actual measurement results are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model 2 The input of model for RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction. Additional input(s) is also allowed. 3 For intra-frequency temporal domain case B indirect measurement event prediction is taken as baseline. Direct prediction is optional.

Based on above RAN2 agreements, we will define measurement event prediction from direct and indirect prediction.

Direct measurement event prediction

For direct measurement event prediction, RAN2 has agreed that the input of model for RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction. Therefore, from RAN4 perspective, the corresponding achievements on model input discussion can be reused as well as the test configurations.

Regarding the model output, since what model outputs is the probability of event occurrence within a time window, a candidate test metric is a pre-defined successful prediction rate, e.g., 90%.

Observation 1: Based on RAN2 agreements, RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction.

Proposal 1: RAN4 to reuse the accuracy requirements defined for RRM measurement prediction for the inputs of measurement event prediction.

Proposal 2: For the outputs of measurement event prediction, RAN4 consider to use a pre-defined successful prediction rate, e.g., 90%, as the performance metric.

Indirect measurement event prediction

For indirect measurement event prediction, related issues were discussed and the following agreements are reached:

Agreement: - Potential RRM requirement impact for event prediction use case - for indirect case: - RAN4 to study the requirement for the predicted event triggered reporting - RAN4 to study the requirement for [delay and accuracy] - RAN4 to discuss the metric for the event prediction - Additionally, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], - Candidate options: - Option 1: RAN4 will define requirement for the predicted RSRPs - Option 2: RAN4 will not define requirement for the predicted RSRPs RAN4#115 Agreement: In case of indirect measurement event prediction use case, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], RAN4 will define requirement for the absolute and/or relative accuracy of predicted RSRP.

Firstly, RAN4 need to define delay requirement for predicted event triggered reporting, the non-AI conditional handover requirements can be used as a starting point, and RAN4 can further discuss the details in WI phase.

Secondly, regarding whether to define accuracy requirement, this is related to the second part in the above agreement and depends on the contents of reporting. In RAN4#115 meeting, RAN4 has agreed to define requirements for the absolute and/or relative accuracy of predicted RSRP if the report includes the predicted RSRPs corresponding to the predicted event occurrence time. When the report does not include the predicted RSRPs, RAN4 need to further discuss whether to define accuracy requirement, where the report contains only the final result of indirect measurement event prediction, i.e., expected occurrence time of a certain measurement event (ex. event A3). In such case, we do not think it is necessary to define accuracy requirement.

Proposal 3: RAN4 to define delay requirement for indirect case and discuss the details in WI phase.

Proposal 4: RAN4 do not define accuracy requirements when the final result report contains the expected occurrence time of a measurement event.

Impact on core requirements for event prediction use case

In addition, measurement event prediction also impacts the conditional handover delay. In RAN2#129 meeting [5], RAN2 provides objectives for some measurement event predictions:

Objective in Case A (FR2): Predict A3 events to enhance HO execution via earlier HO Prep upon A3 event prediction (as shown in Option 2 or Option 3), i.e., HO performance enhancement. Objective in Case B (FR1): Predict A3 events in the presence of skipped measurements, i.e., overhead reduction while maintaining similar HO performance.

According to both objectives, some components in current conditional handover delay requirements can be updated or removed, e.g., Tprocessing and/or Tmeasure. Details can be further discussed in WI phase.

Proposal 5: With AI/ML based measurement event prediction, some components in conditional handover delay requirements can be updated or removed, e.g., Tprocessing and Tmeasure. FFS details in WI phase.

Conclusions

This contribution discussed the impacts of AI/ML measurement event prediction on RRM requirements, and some observations and proposals are provided:

Direct measurement event prediction

Observation 1: Based on RAN2 agreements, RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction.

Proposal 1: RAN4 to reuse the accuracy requirements defined for RRM measurement prediction for the inputs of measurement event prediction.

Proposal 2: For the outputs of measurement event prediction, RAN4 consider to use a pre-defined successful prediction rate, e.g., 90%, as the performance metric.

Indirect measurement event prediction

Proposal 3: RAN4 to define delay requirement for indirect case and discuss the details in WI phase.

Proposal 4: RAN4 do not define accuracy requirements when the final result report contains the expected occurrence time of a measurement event.

Impact on core requirements

Proposal 5: With AI/ML based measurement event prediction, some components in conditional handover delay requirements can be updated or removed, e.g., Tprocessing and Tmeasure. FFS details in WI phase.

References

[1] R4-2508268, WF on AI/ML Mobility Part 1, Nokia, RAN4#115.

[2] R4-2505564, Topic summary for [115][215] FS_NR_AIML_Mob_Part1, Nokia, RAN4#115.

[3] R2-2409502, Report of 3GPP TSG RAN WG2 meeting #127bis, RAN2#127bis.

[4] Report of 3GPP TSG RAN WG2 meeting #128, RAN2#128.

[5] Report of 3GPP TSG RAN WG2 meeting #129, RAN2#129.