3GPP TSG-RAN WG4 Meeting #116				                                                R4-2509310

Bengaluru, India, August 25th – August 29th, 2025

Title:	Discussion on FR2-1 L3 measurement delay reduction by optimizing Rx beam sweeping factor

Source:	CATT

Agenda Item:	7.14.3.1

Document for:	Discussion

1 Introduction

In last RAN4 meeting, the enhancement for FR2-1 SSB based measurement delay reduction was discussed. The conclusions were captured in the approved WF [1] and the left open issues can be found in the topic summary [2]. In this paper, we will continue the discussion on FR2-1 SSB based measurement delay reduction by optimizing Rx beam sweeping factor (BSF) and present our views.

2 Discussion

In last meeting, RAN4 discussed the quitting condition of FBS and reached the following agreement:

RAN4#115 Agreement: For the quitting condition of FBS: - Introduce a new UAI indicating “overheating” issue: - RAN4 will not discuss the UE behaviour after reporting UAI - Introduce a timer-based solution: - UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition - Further discuss the candidate numbers for T1. Way forward: For re-evaluation mechanism for scenario 1/2/3/4: - Check whether the following mechanism is agreeable: - UE re-evaluates the activation condition after the timer expires, where the timer starts after UE quits from Multi-Rx mode.

RAN4 agreed that UE can fall back to normal measurement behaviour when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode, but the time duration of T1 needs further discussion. According to the time duration required by UE to finish SSB-based L3 measurements in the legacy test cases, usually the UE needs a few seconds to complete the measurements. And considering that this FBS feature will be applied for different scenarios, such as SSB-based L3 intra-frequency/inter-frequency measurements with and without measurement gap, therefore we suggest specifying several candidate values, which are in unit of second, for T1 which can be chosen and configured to UE by network. Based on the discussion, we think the minimum value is 10 s. For largest value, few minutes, e.g., 5 minutes, are reasonable to exploit the benefits of this feature. The full set of candidate values for T1 can be {10, 20, 30, 60, 150, 300} second(s), which can be used as a starting point.

Proposal 1: The minimum value for T1 is 10 s and the maximum value for T1 is 300 s.

Proposal 2: The candidate values {10, 20, 30, 60, 150, 300} seconds can be used as a starting point.

Another issue is to define the re-evaluation mechanism after UE falls back to normal measurements. Based on companies’ proposals, a recommended way forward is provided in the summary, i.e., UE re-evaluates the activation condition after a new timer expires, where this new timer starts after UE quits from Multi-Rx mode. Considering that the quit condition of FBS is a timer-based solution, it is straightforward to apply the same approach for re-evaluation mechanism. Regarding the values for this new timer, the agreed values for T1 can be used as a starting point.

Proposal 3: UE re-evaluates the activation condition after a new timer expires, where this timer starts after UE quits from Multi-Rx mode. The values of this new timer use the values for T1 as a starting point.

Regarding the UE capability for FBS, the following agreement is reached:

RAN4#115 Agreement: For FBS capability design: - UE capability is per-band and optional - TDD only - FR2-1 only - FFS: in the same FG, whether to define separate UE capability for (intra-frequency) and (intra-frequency + inter-frequency)

One open issue is whether to define separate UE capability for (intra-frequency) and (intra-frequency + inter-frequency). Considering that UE may need extra implementation for supporting FBS in inter-frequency measurements, we are okay to define separate UE capability for different scenarios in the same FG.

Proposal 4: RAN4 to define separate UE capability for (intra-frequency) and (intra-frequency + inter-frequency) in the same FG.

3 Conclusion

In this paper, we further discussed on FR2-1 L3 measurement delay reduction by optimizing Rx beam sweeping factor, and the following proposals are given:

Proposal 1: The minimum value for T1 is 10 s and the maximum value for T1 is 300 s.

Proposal 2: The candidate values {10, 20, 30, 60, 150, 300} seconds can be used as a starting point.

Proposal 3: UE re-evaluates the activation condition after a new timer expires, where this timer starts after UE quits from Multi-Rx mode. The values of this new timer use the values for T1 as a starting point.

Proposal 4: RAN4 to define separate UE capability for (intra-frequency) and (intra-frequency + inter-frequency) in the same FG.

4 References

- R4-2508266, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT, RAN4#115.

- R4-2505562, Topic summary for [115][213] NR_RRM_Ph5_Part2, CATT, RAN4#115.