3GPP TSG-RAN WG4 Meeting #116				                                                R4-2509319

Bengaluru, India, August 25th – August 29th, 2025

Title:	Discussion on the impacts of A-IoT on RRM performance requirements

Source:	CATT

Agenda Item:	7.22.7

Document for:	Discussion

1 Introduction

RAN4 will discuss performance requirements of ambient IoT from RAN4#116 meeting. In this contribution, we will present our views on performance requirements of A-IoT device 1 deployed in scenario 1 with topology 1 (D1T1-B).

2 Discussion

  - Test cases

In core part, RAN4 defined the requirements for UL transmit timing procedure only. Therefore RAN4 do not need to touch any measurement accuracy requirements and can directly discuss the test cases for UL transmit timing procedure. Regarding the specific scenarios, we prefer to define test cases for the following scenarios:

- Msg1 transmission (X=2)

- Msg3 transmission

- Other D2R transmissions

Proposal 1: Define test cases for the following scenarios to verify the UL transmit timing requirements:

- Msg1 transmission (X=2)

- Msg3 transmission

- Other D2R transmissions

On detailed test configurations, RAN4 need to consider the parameters for both CW2D and D2R transmissions in D1T1-B case. For CW, transmitting power and CW2D distance may need to be set up and candidate values are provided below according to link budget captured in TR 38.769 [1]:

- Transmitting power: [33] dBm.

- CW2D distance: [5, 10, 20] m.

Regarding the supported SCS and R2D transmission bandwidth, the following values are agreed in RAN1 and RF:

- SCS = 15 kHz only.

- R2D transmission bandwidth configuration: 1 PRB, 2 PRB, 3 PRB, 4 PRB.

RAN4 can use these values as a starting point for the test configurations.

Proposal 2: RAN4 to discuss the following test parameters as a starting point:

- CW transmitting power: [33] dBm.

- CW2D distance: [5, 10, 20] m.

- R2D SCS = 15 kHz only.

- R2D transmission bandwidth configuration: [1 PRB, 2 PRB, 3 PRB, 4 PRB].

3 Conclusion

In this paper, we discussed the RRM performance requirements A-IoT and the following proposals are given:

Proposal 1: Define test cases for the following scenarios to verify the UL transmit timing requirements:

- Msg1 transmission (X=2)

- Msg3 transmission

- Other D2R transmissions

Proposal 2: RAN4 to discuss the following test parameters as a starting point:

- CW transmitting power: [33] dBm.

- CW2D distance: [5, 10, 20] m.

- R2D SCS = 15 kHz only.

- R2D transmission bandwidth configuration: [1 PRB, 2 PRB, 3 PRB, 4 PRB].

- References

- TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR, V19.0.0.