3GPP TSG-RAN WG4 Meeting #116           	R4-2509400

Bengaluru, IN, 25th-29th, Aug, 2025

Agenda item:	7.22.8

Source:	Samsung

Title:	View on demodulation requirements for Ambient-IoT device 1

Document for:	Discussion

- Introduction

In the RAN#106 meeting, the new WI related with solutions for Ambient IoT (Internet of Things) in NR was endorsed in [1] with the following objectives

- General scope - The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only - Device (un)availability via Direction 1 in TR 38.769 only - RAN1 scope - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission. - RAN2 scope - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. - Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included - RAN3 scope - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including - Inventory and command operations - Device location reporting at reader ID granularity - Note: The above A-IoT functions are supported over the existing NG interface, based on architectures defined by RAN3/SA2 - RAN4 scope - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system - Use band n8 as an example band - Objective of Performance part WI - Specify the necessary conformance testing for Ambient-IoT BS - Specify the necessary RRM performance requirements for device 1 - Specify the necessary demodulation performance requirements for device 1 and Ambient-IoT BS

As stated in the WID, from RAN4 perspective, RAN4 needs to specify the necessayr demodualtion requirements for device 1 and Ambient-IoT BS.

According to meeting agenda, this is the 1st meeting to dicuss the performance part of Ambient-IoT device 1 and BS. In this contribution, the overall view on the test scope of demodulation requirement for Ambient-IoT device is provided.

# 2	General Scope

In this section, the view on general scope of demodulation requirement for Ambient-IoT Device is provided.

Connectivity topologies

Regarding the connectivity, there are four topologies to support A-IoT operation and two deployment scenarios supported.

Figure 1: Topology 1: BS – Ambient IoT device

According to WID, the D1TI was included, as the deployment scenario 1 with connectivity topology 1. With Ambient IoT device indoors and base station indoors, the deployment scenario 1 is characterized according to the following table

Table 1: Characteristic of deployment scenario 1

| Applicable representative use cases | Characteristics | Description (NOTE 1) |
| --- | --- | --- |
| Indoor inventory Indoor sensor Indoor positioning Indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | Micro- or pico-cell |
|  | Connectivity topology | Topology (1), (2), (3) |
|  | Spectrum | Licensed FDD, licensed TDD, unlicensed |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | DT and DO |
|  | Device characteristic | Device A or Device B or Device C |

Proposal 1: RAN4 focus on D1T1-B scenario for specifying the A-IoT Device demodulation requirement

PRDCH

Based on RAN1 design for A-IoT, only one physical channel is available for R2D link, as PRDCH (Physical Reader to device channel (PDRCH)), which carriers the data and control information from the reader to device. A dedicated broadcast (e.g., 5G PBCH like channel) and a dedicated control channel (e.g., 5G PDCCH like channel) are not supported for R2D. Physical signals such as DMRS, CSI-RS, PTRS are supported in R2D.

For PDRCH transmission, RAN1 has designed the new waveform, modulation, coding scheme to support A-IoT. The following is the processing procedure which includes CRC attachment, coding, OOK modulation, and waveform generation

Figure 2: PRDCH transmission structure

In general, considering it is a new physical layer design, RAN4 should define the related demodulation requirement for PRDCH transmission.

Different with NR devices, since there is no ACK/NACK feedback for R2D transmission through the dedicated PUCCH channel, it is difficult to have the statistics for throughput. TE has no idea about whether the A-IoT device correctly decoded the PRDCH or not, there, the device performance may be not verified properly. Therefore, before agreed to introduce the PRDCH requirement, RAN4 should further discuss how to test the PRDCH demodulation requirement for Ambient-IoT devices.

Observation 1:  There is no ACK/NACK feedback for R2D transmission, it is difficult to have the statistics for throughput. TE has no idea about whether the A-IoT device correctly decoded the PRDCH or not

Proposal 2: RAN4 should further discuss how to test the PRDCH demodulation requirement for Ambient-IoT devices.

R2D timing

Due to the asynchronous nature of A-IoT communication, the starting and ending of the PRDCH transmission need to be specified to device

Each R2D transmission consists of the following 3 parts

Figure 3: R2D transmission

For R2D time acquisition signal, it is used to indicate the starting time of the following PRDCH. It immediately precedes the PRDCH and also carries chip length information. And the R2D postamble is used to indicate the end of the PRDCH.  For R2D time acquisition signal, it consists of two components, the start indicator part and clock acquisition part.

Observation 2: RAN1 has designed the preamble-based R2D transmission with R-TAS signal to indicate the starting point of the PRDCH transmission, and designed postamble transmission to indicate the end of the PRDCH transmission

In general, based on the detection of R-TAS and R2D postamle, the starting point and end of PRDCH transmission can be obtained, then, the length or TBS of PRDCH transmission can be obtained. Based on that, the A-IoT device will proceed the PRDCH detection. Different with NR system, since there is no dedicated PDCCH channel to indicate UE the TBS of data signal, the A IoT device only relies the preamble detection to obtain the related TBS information of PRDCH. Therefore, we think the preamble detection should be included when defining the demodulation requirement for PRDCH.

Proposal 3: RAN4 should include the R2D preamble detection when defining demodulation requirement for PRDCH

# 3	Test Setup

In this section, the view on the test setup for PRDCH demodulation requirement.

In-band or standalone operation

For A-IoT device, both the standalone and in-band operations can be supported. For in-band operation, the starting position of a A-IoT R2D transmission is aligned in time with the starting position of an NR OFDM. Considering the design of R2D time acquisition signal is used to indicate the starting time of the following PRDCH, and it was included to specify the PRDCH requirement, we think only focusing on the standalone operation when defining the PRDCH requirement should be enough. We are also open to further discuss whether the in-band operation should be considered when defining the PRDCH requirement.

Proposal 4: Consider the standalone operation when defining the PRDCH requirement, FFS on considering the in-band operation when defining the PRDCH requirement.

Waveform

Based on RAN1 design, the R2D transmission is a DFT-s-OFDM based OOK-4 waveform, and for OOK-4 modulation for M-chip per OFDM symbols.

Nomology

For R2D transmission, only subcarrier spacing of 15 kHz is supported, and no other SCS are further considered. And for R2D chip duration, the CP handling that retains sub-carrier orthogonality, the length of a R2D chip duration in one OFDM symbol is defined as

- C= 1/(SCS*M)

- Note: the total length of M OOK chips is equal to 1/SCS

Proposal 5: RAN4 to define PRDCH requirement with DFT-s-OFDM with OOK-4 waveform for 15KHz SCS.

Regarding the value of M, the following is agreed in the last meeting

Agreement - M = {2,6,12,24} are adopted for CAP and same M value is used for CAP and PRDCH in an R2D transmission. R2D postamble is specified with 4 ON chips corresponding to M value of the PRDCH - R2D postamble is added immediately after the PRDCH - R2D postamble has always 4 ON chips - Note: For M=24, 2 ON chips at the end of OFDM symbol for CP handling are in addition to R2D postamble, and are not part of the R2D postamble - R2D padding duration is determined after R2D postamble insertion TBS information for R2D is supported via higher layer R2D control signalling. - Send LS to RAN2 asking to include R2D TBS information (excluding CRC length) in higher layer signaling, at least for messages with variable size. Note: Exact method for determining the end of PRDCH at the device is not specified. Agreement For R2D, the minimum Btx,R2D # of PRBs associated to each agreed M value (i.e. M = 2, 6, 12 and 24) is specified as per TR38.769. Agreement For M=24 on CP handling, update the below agreement as follows Agreement For further down-selection among CP handing which retains subcarrier orthogonality, at least for PRDCH, at least Method Type 1 is supported - For supported M values <= 12 - RAN1 will not further pursue additional CP handling design - For supported M values > 12 - RAN1 will further down-select one from the followings - Option 1: Candidate 3 of M2-1-1 (as per agreements from RAN1#120) - Insert padding chips only at the end OOK chips of OFDM symbol - The last 2 out of M OOK chips at the end of an OFDM symbol are always ‘ON’ - For the OFDM symbol contains CAP - The last 2 out of M OOK chips at the end of the OFDM symbol are always ‘ON’ - Option 3: RAN1 will not further pursue additional CP handling design

The value of M will impact on the length of R2D chip duration, also R2D postamble related operation, CP handling and the minimum Btx,R2D # of PRBs. Therefore, RAN4 should discuss the proper value of M when defining PRDCH requirement

Observation 3: The value of M will impact on the length of R2D chip duration, R2D postamble related operation, CP handling and minimum Btx,R2D # of PRBs

Since RAN1 will not further pursue additional CP handling in case the supported M values is small than 12, and specify the solution for M value is large than 12. Based on agreed M values, M as 24 can be considered as starting point when defining PRDCH requirement.

Proposal 6: RAN4 should discuss the proper value of M when defining PRDCH requirement, M as 24 can be considered as starting point.

Repetition

For the R2D transmission, the repetition is not supported in Rel-19 Ambient IoT. Therefore, there is no repetition when defining PRDCH requirement

Proposal 7: No repetition when defining PRDCH requirement

Chanel coding and CRC

For the R2D transmission, FEC is not supported in RAN1. Regarding the CRC bit. RAN1 concluded that the generator polynomials in TS 38.212 for 6-bit CRC and 16-bit CRC are reused for Ambient IoT. Pending on the number of information bits, the number of CRC bit is selected differently as following

Agreement - When CRC is attached to a PRDCH or PDRCH transmission, when the number of information bits is ≤ 24 bits, CRC-6 is used. Otherwise, when the number of information bits is > 24 bits, CRC-16 is used. - For attaching the CRC, the parity bits are appended to the end of the input bits, according to the order in TS 38.212

Observation 4:  Both 6-bit CRC and 16-bit CRC can be supported for PRDCH transmission

As proposed, we prefer to consider M as 24 for starting point, we think we can select the number of information bits is larger than 24 bits, the related 16-bis CRC bit can be considered.

Proposal 8:  Consider the number of 16-bits CRC as starting point when defining PRDCH requirement.

R2D line coding

For the R2D transmission, the Manchester encoding was applied, where the bit-to-chip mapping as: bit 0 to chips {0, 1}, RAN4 will follow the R2D line coding to define the related PRDCH requirement.

Proposal 9:  RAN4 will follow the R2D line coding to define the corresponding PRDCH requirement.

Maximum TBS

For the R2D transmission, the maximum number of TBS is equal to the maximum message size of 1000 bits for A-IoT device. Pending on configurated bandwidth and M, RAN4 should further discuss the number of TBS used for defining PRDCH demodulation requirement.

Proposal 10: RAN4 should further discuss the number of TBS used for defining PRDCH demodulation requirement

Channel Bandwidth

For the R2D transmission, the following is the minimum of PRB and associated with M value as indicated in the TR 38.769

Table 2: Mapping of minimum of PRB and associated with M value

| M | Minimum Btx,R2D # of PRBs |
| --- | --- |
| 1 | 1 |
| 2 | 1 |
| 4 | 1 |
| 6 | 1 |
| 8 | 2 |
| 12 | 2 |
| 16 | 2 |
| 24 | 3 |
| 32 | 4 |

As agreed in RAN1

Agreement - For R2D, Bocc,R2D is not specified in RAN1.This does not preclude RAN4 to discuss Bocc,R2D themselves when defining requirement(s), if needed. Bocc,D2R =Btx,D2R - For R2D, the minimum Btx,R2D # of PRBs associated to each agreed M value (i.e. M = 2, 6, 12 and 24) is specified as per TR38.769. - For R2D, whether to specify the maximum Btx,R2D # of PRBs is up to RAN4

And furthermore, the following related with R2D transmission bandwidth was agreed in RAN4#114bis and RAN4#115 meeting

Agreement in RAN4#114bis - For RAN4 requirements, only define R2D transmission bandwidth configuration as 1PRB, 2PRB,3PRB, 4PRB with 15KHz SCS - Define R2D channel bandwidth with assumption of 90% spectrum utilization Agreement in RAN4#115 - Define symmetric guard band as 10kHz, 20kHz, 30kHz and 40kHz for 1PRB, 2PRB, 3PRB and 4PRB channel bandwidth configuration

Considering we propose M as 24, based on the mapping of minimum PRB and M value, we can consider the number of PRB as 3PRB as starting point. We are also open to further discuss whether other transmission bandwidth will be considered.

Proposal 11: Considering the number of PRB as 3 for starting point when defining PRDCH requirement. FFS on other R2D transmission bandwidth configuration

Test metric

Regarding the test metric, the BLER with 1% or 10% was selected for RAN1 A-IoT evaluation. In existing NR demodulation requirement, the SNR with 70% of maximum throughput was applied. RAN4 should further discuss the proper test metric for RAN4 A-IoT device demodulation requirement.

Proposal 12: RAN4 should further discuss the proper test metric for defining the A-IoT device demodulation requirement

Channel Model

For A-IoT device, the main deployment should be targeting for indoor scenario with low speed, similar as eMTC and NB-IoT. Therefore, the channel varying is slow. Based on RAN1 evaluation, AWGN and TDL-A channel were selected for feasibility evaluation. We propose to use the AWGN and TDL-A channel as starting point

Proposal 13: RAN4 should further discuss the proper channel used for defining PRDCH requirement

R2D multiplexing

For R2D transmission, FDMA and CDMA are not supported, only TDMA is supported. From RAN4 demodulation perspective, since there is no need considering the multiple user scenario, RAN4 focuses on the single device for defining PRDCH requirement.

Proposal 13: RAN4 focuses on single device to define PRDCH requirement.

# 4	Conclusion

In this contribution, the overview on performance part of A-IoT devices was provided.

Connectivity topologies

Proposal 1: RAN4 focuses on D1T1-B scenario for specifying the A-IoT Device demodulation requirement

PRDC

Observation 1:  There is no ACK/NACK feedback for R2D transmission, it is difficult to have the statistics for throughput. TE has no idea about whether the A-IoT device correctly decoded the PRDCH or not

Proposal 2: RAN4 should further discuss how to test the PRDCH demodulation requirement for Ambient-IoT devices.

R2D timing

Observation 2: RAN1 has designed the preamble-based R2D transmission with R-TAS signal to indicate the starting point of the PRDCH transmission, and designed postamble transmission to indicate the end of the PRDCH transmission

Proposal 3: RAN4 should further discuss whether to include the R2D preamble detection when defining demodulation requirement for PRDCH

In-band or standalone operation

Proposal 4: Consider the standalone operation when defining the PRDCH requirement, FFS on considering the in-band operation

Waveform & Nomology

Proposal 5: RAN4 to define PRDCH requirement with DFT-s-OFDM with OOK-4 waveform for 15KHz SCS.

Observation 3: The value of M will impact on the length of R2D chip duration, R2D postamble related operation, CP handling and minimum Btx,R2D # of PRBs

Proposal 6: RAN4 should discuss the proper value of M when defining PRDCH requirement, M as 24 can be considered as starting point.

Repetition

Proposal 7: No repetition when defining PRDCH requirement

Chanel coding and CRC

Observation 4:  Both 6-bit CRC and 16-bit CRC can be supported for PRDCH transmission

Proposal 8:  Consider the number of 16-bits CRC as starting point when defining PRDCH requirement.

R2D line coding

Proposal 9:  RAN4 will follow the R2D line coding to define the corresponding PRDCH requirement.

Maximum TBS

Proposal 10: RAN4 should further discuss the number of TBS used for defining PRDCH demodulation requirement

Channel Bandwidth

Proposal 11: Considering the number of PRB as 3 for starting point when defining PRDCH requirement. FFS on other R2D transmission bandwidth configuration

Test metric

Proposal 12: RAN4 should further discuss the proper test metric for defining the A-IoT device demodulation requirement

Channel Model

Proposal 13: RAN4 should further discuss the proper channel used for defining the number of TBS used for defining PDRCH requirement

R2D multiplexing

Proposal 14: RAN4 focuses on single device to define PRDCH requirement.

# References

RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR,