3GPP TSG-RAN WG4 Meeting #116          	R4-2509401

Bengaluru, IN, 25th-29th, Aug, 2025

Agenda item:	7.21.6

Source:	Samsung

Title:	View on demodulation requirement for SBFD

Document for:	Discussion

- Introduction

In the RAN#104 meeting, the WI was endorsed to support the evolution of NR duplex operation SBFD, which can be categorized as following

- Objective 1: semi-static indication of time location of SBFD subbands

- Objective 2: semi-static indication of frequency domain location of SBFD subbands mode

- Objective 3: SBFD operation to support random access in SBFD symbols

- Objective 4: UE transmission, reception and measurement behavior and procedures in SBFD symbols

- Objective 5: Enhancements for CLI handling

In the last RAN4 meeting, the test scope for UE and BS demodualtion requirements with supporting SBFD feature was discussed,  the related agreement was captured into WF in [1] as following

- UE demodulation requirement - General Scope - whether to introduce UE demodulation requirement related with inter-UE CLI for Rel-19 SBFD aware UEs - Not to introduce UE demodulation test related with inter-UE CLI for Rel-19 SBFD-aware UEs - whether to introduce UE CSI reporting requirement for Rel-19 SBFD aware UEs - Not to introduce UE CSI reporting requirement for Rel-19 SBFD-aware UEs - whether to introduce UE demodulation with SBFD carrier (with DL sub-band and UL sub-suband configured) - Option 1: Define limited number of additinal SBFD cases - Other options are not precluded - whether to introduce UE demodulation with repetition transmission - Do not introduce UE demodulation requirement with repetition transmission cases for SBFD-aware UE - BS demodulation requirement - Introduce PUSCH demodulation requirement with UL resource muting configured with interference modelling - FFS on which type(s) of interference will be considered to specify the PUSCH requirement - gNB self-interference - co-site inter-sector interference - inter-site gNB-gNB - FFS on how to model interference - gNB self-interference - Option 1: modelling as additive white guassian noise with fixed INR =-6dB targeting 1dB desense as starting pointing - Other options are not precluded - co-site inter-sector interference - Option 1: modelling as additive white gaussian noise with fixed INR = - X dB based on assumption of co-site isolation - Other options are not precluded - inter-site gNB-gNB co-channel inter-subband CLI - whether to introduce new PUCCH requirement to support SBFD operation - No new PUCCH requirement introduced. The legacy PUCCH test cases will be applied for SBFD capable-gNB - whether to introduce new PRACH requirement to support SBFD operation - No new PRACH requirement introduced. The legacy PRACH test cases will be applied for SBFD capable-gNB - Test setup for demodulation requirement if introduced - Frequency Range - Option 1: FR1 and FR2-1 - Other options are not precluded - Channel Bandwidth - Option 1 - prioritize the minimum bandwidth sizes of SBFD for both FR1 (40 MHz) and FR2-1 (100 MHz) NR TDD bands - if additional bandwidth is required, 100MHz for FR1 - Option 2 - FR1 with 30KHz, 40MHz DL sub-band + 20MHz UL sub-band for 30kHz SCS - FR2-1 with 120KH, 100MHz DL sub-band + 100MHz UL sub-band for 120kHz SCS - Other options are not precluded - TDD pattern - Option 1 - 30KHz SCS: XXXXXXXXUU - 120khZ SCS: XXXXU - Other options are not precluded - Waveform - Option 1 - Both DFT-s-OFDM and CP-OFDM - Other options are not precluded - Inclusion in Specification - Specification Impact - Option 1: New sub-clause for demodulation requirements under same chapter with RF requirements can be considered. If RAN4 agree Not to introduce new BS demodulation requirements for SBFD, only applicability rule can be captured. - Other options are not precluded

In this contribution, the view on test scope and test setup for UE and BS demodulation requirement with supporting SBFD operation is provided.

# 2	Overall RAN4 demodulation impact

2.1	UE demodulation requirement

According the objective of NR duplex operation SBFD as following

- Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Followings are assumed based on TR 38.858 - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators

This is major related to demodulation requirements from RAN4 perspective. Since both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs, from UE perspective, there is no impact for demodulation requirement, considering there is no special enhancement for UE side. Therefore, we are fine not to introduce the UE demodulation requirement SBFD carrier (with DL sub-band and UL sub-band configured)

Proposal 1: Do not introduce the UE demodulation requirement SBFD carrier (with DL sub-band and UL sub-suband configured)

2.2	BS demodulation requirement

Interference modeling

In the last meeting, RAN4 agreed to introduce PUSCH demodulation requirement with UL resource muting configured with interference modelling.

Regarding the interference modelling for SBFD gNB, from gNB perspective, the interference scenario can be identified as 3 categories that

- gNB Self-Interference (SI)

- co-site inter-sector (or intra-site) co-channel inter-subband CLI

- gNB-to-gNB inter-site interference

From Link level simulation perspective, RAN1 considers them in the TDD system with considering the following modeling method in TR

- Option 1 - For SBFD slot, additive white Gaussian noise with variance of NSBFD is generated, where NSBFD=ISI+Ico-site+IgNB-gNB+IUE-gNB+N0 - ISI, Ico-site, IgNB-gNB, IUE-gNB are self-interference, co-site inter-sector interference, inter-site gNB-gNB co-channel inter-subband CLI and UE-gNB interference (in linear scale), respectively - Based on the modelling method, the following high-level evaluation method can be used as an example for coverage performance evaluation: - Step 2: For SBFD system with frame structure XXXXU, assume the SNR in UL only slot is SNRSBFD-UL=SNTDD and the SNR in SBFD slot is SNRSBFD-X=SNSBFD. Perform LLS to get the required SNR (SNRSBFD-UL) with which UE can achieve a certain bit rate in UL for a given SBFD coverage enhancement scheme (e.g., SBFD with PUSCH repetition type A, etc.) - Option 2 - The UE-gNB interference and inter-site gNB-gNB co-channel inter-subband CLI in LLS coverage evaluation are explicitly modelled based on a given topology of aggressor UEs and gNBs. The UE-gNB and gNB-gNB fast fading channels are explicitly modelled in LLS. The signal model is as follows - Y=HkSk +n≠kHnSn +HgSg+HCLI,mWmSm+Iself+N - Y is the received signal vector at the victim gNB Hn, Sn, n≠k, are the channel matrix and transmitted signal of the UE in the same cell as the target user - Hg and Sg are the channel matrix and transmitted signal of the UEs in the adjacent cell - HCLI,m, Wm and Sm are the channel matrix, the precoding matrix, and leakage CLI signal from aggressor gNB m to the victim gNB. - The power of the signal and interference is included in the channel matrix respectively - Iself and N are the self-interference vector of the co-site sectors and the thermal noise signal vector on the receiving antennas - Based on the above modelling, the following high-level evaluation method can be used as an example for coverage performance evaluation: - Step 2: For SBFD system with frame structure XXXXU, perform LLS to get the required SNR (SNRSBFD-UL=SN0) with which UE can achieve a certain bit rate in UL for a given SBFD coverage enhancement scheme (e.g., SBFD with PUSCH repetition type A, etc.)

According the TR 38.858, for above two options, self-interference can be modeled as additive white gaussian noise with fixed INR=-6dB targeting 1 dB densene similar as SLS, and co-site inter-sector interference can be modelled as additive white gaussian noise with fixed INR=-XdB based on the assumption of co-site isolation αco-site.

Regarding the gNB-gNB interference, for option1, the additive white Gaussian noise is modeling, the value is derived based on a certain assumption of the topology of gNBs and UEs, and the interference is pre-receiver interference.

For option 2, the inter-site gNB-gNB interference is explicitly modelled with including the fading channel based on a given topology of aggressor gNBs.

In general, for option 2, the interference profile requires the system level simulation with proper test setup. It requires to define a commonly agreed leakage model. Meanwhile, for inter-site gNB-gNB interference, we think the existing RAN4 specification has already defined the corresponding test cases with MMSE-IRC for inter-tell interference. Therefore, there is no necessary to consider the inter-site gNB-gNB interference when defining PUSCH requirement.

Based on RAN4 co-existence simulation assumptions for RF requirement study, we see that the overall of inter-sector gNB-gNB interference including the gNB-gNB self-interference can be modelled as additive white gaussian noise with fixed INR =-6dB targeting 1dB UL receiver sensitivity degradation

Observation 1: Self-interference can be modeled as additive white gaussian noise with fixed INR=-6dB targeting 1dB desense similar to SLS, and co-site inter-sector interference can be modelled as additive white gaussian noise with fixed INR=-X dB based on the assumption of co-site isolation  αco-site.

Observation 2: Either option 1 and option 2 for gNB-gNB interference modeling, the interference level is derived based on a certain assumption of the topology of gNBs and UEs.

Proposal 2: RAN4 introduce PUSCH demodulation with interference modeling only including the gNB self-interference and co-site inter-sector interference

Proposal 3: The overall of inter-sector gNB-gNB interference including the gNB-gNB self-interference can be modelled as additive white gaussian noise with fixed INR =-6dB targeting 1 dB UL receiver sensitivity degradation

# 3	Test Setup

In this contribution, the view on the demodulation scope for SBFD gNB is provided

Subband Configuration

According RAN1 design, the following SBFD subband configuration are considered for SBFD evaluation.

- SBFD subband configuration #1 with {DUD} pattern, which means one SBFD slot consists of one UL subband at the center of the channel bandwidth and two DL subbands at two sides of the channel bandwidth

- SBFD subband configuration # 2 with {DU} pattern, which means one SBFD slot consists of one UL subband at one side of the channel bandwidth and one DL subband at the other side of the channel bandwidth

Compared with two subband configurations, for UL transmission, from demodulation perspective, only different is the location of UL subband. We can consider subband configuration{DU} as starting point. While, we are also open to discuss whether additional subband configuration {DUD} pattern can be considered

Proposal 4: Consider subband configuration with {DU} for specifying SBFD PUSCH demodulation requirement, FFS on additional subband configuration {DUD} will be considered

TDD pattern

For SBFD subband configuration, for performance evaluation and comparison between baseline and legacy TDD operation, several options are considered in RAN1 study in TR 38.858 as following

- Al1 (No SBFD DL subband in slots/symbols that correspond to UL slots/symbols in legacy TDD) - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] - SBFD: Frame structure#1 (DXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about 20% of the channel bandwidth. - Alt 2 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD): - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about 20% of the channel bandwidth. - Alt 3 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD): - Legacy TDD: Static TDD UL/DL configuration with {DDSUU}, where S=[12D:2G:0U] - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about 25% of the channel bandwidth. - Note: Alt 3 is deprioritized. - Alt 4 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD): - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] - SBFD: Frame structure#3 (XXXXX), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about 20% of the channel bandwidth.

In legacy demodulation test, TDD pattern with {7D1S2U} for 30KHz SCS, and {3D1S1U} for 120KHz SCS are applied. To compare with legacy cases, we propose to use XXXXXXXXUU for 30KHz SCS and XXXXU for 120KHz SCS for RAN4 SBFD PUSCH demodulation requirement definition.

Proposal 5: Consider the SBFD slot configuration XXXXXXXXUU for 30KHz SCS and XXXXU for 120 KHz SCS for SBFD PUSCH demodulation requirement definition

Bandwidth

According to RAN4 agreement, the channel bandwidth for SBFD operation shall be no less then 40MHz and no less than 20MHz for band 39 in FR1 bands, and shall be no less than 100MHz in FR2-1 bands.

Based on RAN1 study, 100MHz channel bandwidth for 30KHz SCS is baseline for FR1, and 200MHz channel bandwidth for 120KHz SCS is baseline for FR2-1.

Since 40MHz and 100MHz are the typical channel bandwidth and applied for existing demodulation requirement, and considering the large channel bandwidth is more proper for SBFD operation, then, we prefer to consider 40MHz and 100MHz for FR1, and 200MHz for FR2-1 for specifying SFBD PUSCH demodulation requirement. Regarding the bandwidth for UL sub-band, we can further discuss pending on the agreed subband pattern with {DUD} or {DU} configuration.

Proposal 6: Consider 40MHz and 100MHz for FR1, and 200MHz for FR2-1 for specifying SFBD PUSCH demodulation requirement

SBFD slots

For UL subband, it can be located at the one side of the carrier or can be allocated at the middle part of carrier. In time domain, UL symbol is defined per symbol granularity where the SBFD symbols are configured in consecutive manner within a TDD UL-D pattern period and extract start and end symbols are defined. The SBFD slot can consist of SBFD symbols and non-SBFD symbols.  Since the purpose of SFBD operation is to transmit the UL signal in the DL slot for improving UL capacity, for simplicity, we propose to configure SBFD UL subband spans all the symbols a SBFD slot. We are also open to further discuss whether non SBFD symbols will be considered in a SBFD slot.

Proposal 7: Configure SBFD UL subband spans all the symbols a SBFD slot.  FFS to consider non SBFD symbols in a SBFD slot.

Waveform

For SBFD operation, there is no limitation for waveform, Since the configuration and UE behavior will be different for different waveforms, our preference is to cover both CP-OFDM and DFT-s-OFDM

Proposal 8: Configure one OFDM symbol for UL resource muting. The UL resource muting is applied in SBFD symbols

Number of Symbols Muting

The position in the slot of the up to two OFDM symbols, since the purpose of UL resource muting resource is for gNB-to-gNB CLI measurement and/ or interference covariance matrix measurement. The UE applies the muting to the whole PUSCH bandwidth and with 3 dB power boosting over the active REs within the UL muting symbol(s). In our understanding configuring 1 symbol or 2 symbols can meet the test purpose. For simplicity, we prefer to configure 1 OFDM symbol as starting point.

For UL resource muting, if UE configured with SBFD symbols, it can be configured the muting in both SBFD symbols and non-SBFD symbols. Since the DMRS symbols within the non-SBFD symbols can be used for interference covariance matrix measurement, we prefer the UL resource muting is only applied in the SBFD symbols. As our preference, only considering non-SBFD symbols in a SBFD slot, then, the UL resource muting is applied in SBFD symbols in a SBFD slot.

Proposal 9: Configure one OFDM symbol for UL resource muting. The UL resource muting is applied in SBFD symbols in a SBFD slot

Configuration of UL muting

The position of symbol and comb offsets relative to the lowest indexed resource element of the PUSCH allocation, are given by the high-layer parameters symbolPos and combOffset. And the muting resource configuration and expected UE behavior is not expected to be configured with overlap with RE for PUSCH as following

- The UE is not expected to be configured with a muting resource within which resource elements overlap in time and frequency with a resource element used for PUSCH PT-RS when transform precoding is not enabled. - The UE shall ignore any resource elements of a muting resource that overlaps in time with an OFDM symbol used for any of - PUSCH DM-RS - PT-RS when transform precoding is enabled

Based on existing RAN4 test case for PUSCH, 2 DMRS symbols will be configured. Based the DMRS and PTRS pattern, the UL source muting configuration can be considered as staring point in Fig 1, Fig 2 and Fig.3, repectively.

Figure 1: PUSCH mapping type A, symbolPos=13 and combOffset=1 for UL resource muting

Figure 2: PUSCH mapping type B, symbolPos=13 and combOffset=1 for UL resource muting

Figure 3: PUSCH mapping type B, symbolPos=10 and combOffset=1 for UL resource muting, with PT-RS

Proposal 10: Consider the following UL resource muting as starting point for defining SBFD PUSCH demodulation requirement

- FR1, CP-OFDM and DFT-s-OFDM, PUSCH mapping type A, symbolPos=13 and combOffset=1

- FR1, CP-OFDM and DFT-s-OFDM, PUSCH mapping type B, symbolPos=13 and combOffset=1

- FR2, CP-OFDM and DFT-s-OFMM, PUSCH mapping type B, symbolPos=10 and combOffset=1, with and without PTRS

Mapping type

Similar as legacy PUSCH demodulation requirement, both mapping type A and type B are specified for FR1, mapping type B is specified for FR2-1. We proposal the same configuration can be used for defining SBFD PUSCH demodulation requirement

Proposal 11: RAN4 considers both mapping type A and type B for SBFD PUSCH demodulation requirement in FR1, and mapping type B for SBFD PUSCH demodulation requirement in FR2-1.

Number of DMRS symbols

For number of DMRS symbols, since the SBFD is not targeting for high-speed scenario, we think 2 DMRS symbols configured should be enough.

Proposal 12: RAN4 considers 2 DMRS symbols for defining SBFD PUSCH demodulation requirement in FR2-1.

Channel Model

For MCS, we think the same configuration can be reused as TDLB100-400, TDLC300-100 and TDLA30-10 for defining the SBFD PUSCH demodulation requirement

Proposal 13: RAN4 considers TDLB100-400, TDLC300-100 and TDLA30-10 for defining SBFD PUSCH demodulation requirement in FR1 and FR2-1.

MCS

For MCS, we think the same configuration can be reused as MCS 2, MCS 16 and MCS 20 for defining the SBFD PUSCH demodulation requirement

Proposal 14: Reuse the MCS assumption including MCS 2, MCS 16 and MCS 20 for specifying SBFD PUSCH demodulation requirement in FR1 and FR2-1.

Antenna Configuration

For antenna configuration, we think the same configuration can be reused as 1Tx/2Rx/4Rx/8Rx in FR1 and 1Tx/2Rx in FR2-1 for defining the SBFD PUSCH demodulation requirement

Proposal 15: Reuse the same antenna configuration assumption for specifying SBFD PUSCH demodulation requirement as 1Tx/2Rx/4Rx/8Rx in FR1 and 1Tx/2Rx in FR2-1

Others

For other parameters, the same assumption can be used for specifying SBFD PUSCH demodulation requirement in FR1 and FR2-1 as referred in Table: 8.2.1.1-1/ Table: 8.2.2.1-1 and Table 11.2.2.1.1-1/ Table 11.2.2.2.1-1, respectively.

Proposal 16: Reuse the same assumption for specifying SBFD PUSCH demodulation requirement in FR1 and FR2-1 as referred in Table: 8.2.1.1-1/ Table: 8.2.2.1-1 and Table 11.2.2.1.1-1/ Table 11.2.2.2.1-1, respectively.

# 4	Conclusion

In this contribution, the view on the demodulation scope for SBFD is provided.

Proposal 1: Do not introduce the UE demodulation requirement SBFD carrier (with DL sub-band and UL sub-suband configured)

Observation 1: Self-interference can be modeled as additive white gaussian noise with fixed INR=-6dB targeting 1dB desense similar to SLS, and co-site inter-sector interference can be modelled as additive white gaussian noise with fixed INR=-X dB based on the assumption of co-site isolation  αco-site.

Observation 2: Either option 1 and option 2 for gNB-gNB interference modeling, the interference level is derived based on a certain assumption of the topology of gNBs and UEs.

Proposal 2: RAN4 introduce PUSCH demodulation with interference modeling only including the gNB self-interference and co-site inter-sector interference

Proposal 3: The overall of inter-sector gNB-gNB interference including the gNB-gNB self-interference can be modelled as additive white gaussian noise with fixed INR =-6dB targeting 1 dB UL receiver sensitivity degradation

Proposal 4: Consider subband configuration with {DU} for specifying SBFD PUSCH demodulation requirement, FFS on additional subband configuration {DUD} will be considered

Proposal 5: Consider the SBFD slot configuration XXXXXXXXUU for 30KHz SCS and XXXXU for 120 KHz SCS for SBFD PUSCH demodulation requirement definition

Proposal 6: Consider 40MHz and 100MHz for FR1, and 200MHz for FR2-1 for specifying SFBD PUSCH demodulation requirement

Proposal 7: Configure SBFD UL subband spans all the symbols a SBFD slot.  FFS to consider non SBFD symbols in a SBFD slot.

Proposal 8: Configure one OFDM symbol for UL resource muting. The UL resource muting is applied in SBFD symbols

Proposal 9: Configure one OFDM symbol for UL resource muting. The UL resource muting is applied in SBFD symbols in a SBFD slot

Proposal 10: Consider the following UL resource muting as starting point for defining SBFD PUSCH demodulation requirement

- FR1, CP-OFDM and DFT-s-OFDM, PUSCH mapping type A, symbolPos=13 and combOffset=1

- FR1, CP-OFDM and DFT-s-OFDM, PUSCH mapping type B, symbolPos=13 and combOffset=1

- FR2, CP-OFDM and DFT-s-OFMM, PUSCH mapping type B, symbolPos=10 and combOffset=1, with and without PTRS

Proposal 11: RAN4 considers both mapping type A and type B for SBFD PUSCH demodulation requirement in FR1, and mapping type B for SBFD PUSCH demodulation requirement in FR2-1.

Proposal 12: RAN4 considers 2 DMRS symbols for defining SBFD PUSCH demodulation requirement in FR2-1.

Proposal 13: RAN4 considers TDLB100-400, TDLC300-100 and TDLA30-10 for defining SBFD PUSCH demodulation requirement in FR1 and FR2-1.

Proposal 14: Reuse the MCS assumption including MCS 2, MCS 16 and MCS 20 for specifying SBFD PUSCH demodulation requirement in FR1 and FR2-1.

Proposal 15: Reuse the same antenna configuration assumption for specifying SBFD PUSCH demodulation requirement as 1Tx/2Rx/4Rx/8Rx in FR1 and 1Tx/2Rx in FR2-1

Proposal 16: Reuse the same assumption for specifying SBFD PUSCH demodulation requirement in FR1 and FR2-1 as referred in Table: 8.2.1.1-1/ Table: 8.2.2.1-1 and Table 11.2.2.1.1-1/ Table 11.2.2.2.1-1, respectively.

# References

RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, HiSilicon

R4-2508723, Way Forward for [115][330] NR_duplex_evo_demod, Samsung