3GPP TSG-RAN WG4 Meeting #116	R4-2509425

# Bengaluru, India, August 25th – 29th, 2025

Source:	Apple

Title:	RRM core requirements and testing framework Beam Management

Agenda item:	7.17.4

Document for:	Discussion

Introduction

The discussions on AI/ML enabled BM use case were continued in RAN4#114bis in the scope of Rel-19 WI NR_AIML_air ‎[1] based on the objectives agreed in RAN4#107. Agreements achieved during RAN4#114bis meeting are summarized in the WF ‎[2].

Some of the issues related to AI/ML enabled BM-Case1 and BM-Case2 require further discussion. In this paper, we provide some additional views on issues related to RRM core requirements, as listed below:

- Measurement delay requirements

- Impacts on requirements related to candidate beam detection

- LCM and performance monitoring related requirements

- Delay requirements for applicable functionality reporting

- Testing framework based beam management

Discussion

Measurement delay requirements

We are discussing if legacy 5G NR L1-RSRP measurement-delay requirements (TS 38.133 §9.5.4.2) can or can’t be reused when AI/ML-based beam-management (BM) prediction is introduced in RAN4 work. In particular, we are focusing on the values M,N,P defined in legacy measurement delay requirements.

Firstly, let’s have a conceptual recap of the values M, N, P:

- M (temporal-freshness / averaging factor)
Counts how many consecutive CSI-RS periods (after we have already made measurements with every Rx-beam once) the UE must keep gathering fresh samples before it can hand an L1-RSRP value to higher layers. The spec sets M = 1 when a higher-layer timeRestrictionForChannelMeasurement is present or for aperiodic CSI-RS; otherwise M = 3 for periodic / semi-persistent CSI-RS.

- N (spatial-sweep factor)
With repetition = ON the same Tx beam-set is sent in several CSI-RS symbols/ SSB bursts; the UE measures its Rx beam each time.  If repetition = ON for periodic CSI-RS in that resource set N = ceil( maxNumberRxBeam / N_res_per_set ), where: maxNumberRxBeam is a UE capability / handled-beam upper bound signalled and N_res_per_set is the number of CSI-RS resources in the configured resource set (i.e., how many distinct spatial “shots” we already get per period).  In many FR2 RRM test configurations and lab discussions, UEs are assumed to support up to 8 Rx beams (a practical test-capability level reflected in UE capability signalling work and used in various RAN4 RRM setups). When: maxNumberRxBeam = 8, and the network configures one CSI-RS resource in the resource set (N_res_per_set = 1), and repetition = ON (meaning the UE must get enough CSI-RS opportunities to cover all beams), then the formula gives N = ceil(8 / 1) = 8. That’s the scenario that led to  “legacy N = 8”.  Therefore,  “N = 8” is a configuration-derived value, not a spec constant.

- P (availability-loss factor)
Inflates the measurement window when some CSI-RS occasions collide with SMTC windows or measurement gaps.  P = 1 when there is no overlap; it rises according to the overlap formulas in §9.5.4.2 when collisions occur.

N tells us how many CSI-RS periods/SSB bursts in total the UE must span to complete its Rx-beam sweep.

- If N = 1, the UE measures what it needs in the very next CSI-RS occasion and we pay only one period of delay.

- If N = 4, it means we wait through four CSI-RS periods before every Rx-beam has been measured once.

When the CSI-RS resource-set is configured with repetition = OFF, each CSI-RS resource may carry a different Tx beam, but the UE only measures one Rx-beam per resource instance so by definition it never needs to cycle through its other Rx-beams to cover spatial directions. In that case the spatial-sweep factor N in the 38.133 delay formula collapses to 1.

When repetition = ON, we re trading off how many CSI-RS periods the UE spends sweeping its Rx-beams (that’s N) against how many CSI-RS symbols we load into each period (that’s N₍res_per_set₎).  Here are the tradeoffs involved:

Pros of a smaller N

- Lower latency: Fewer full CSI-RS periods means the first complete Rx-beam sweep, and thus the multi-beam measurement vector arrives more quickly, shaving milliseconds off the end-to-end prediction.

- Reduced UE power draw: The RF front end and digital beam-steering logic only turns on for a handful of symbols before reporting, which conserves battery.

- Faster beam-failure recovery: Should we need to fall back to measured beams, having a small-N sweep means we can detect link degradation sooner.

Cons of a smaller N

- Higher DL overhead: To hit a small N we must pack more CSI-RS symbols per period (larger N₍res_per_set₎), which could consume more time-frequency resources on PDSCH or special CSI-RS slots.

- Scheduler pressure: Denser CSI-RS placements can collide with data or other measurements, forcing tighter resource-planning and potentially more PDCCH/DCI juggling.

Pros of a larger N

- Lower CSI-RS footprint: With fewer symbols per period, the network can serve more UEs or carry more data, since CSI-RS “steals” less air-time.

- Simpler scheduling: Sparsely spaced CSI-RS patterns could ease the burden on scheduling

Cons of a larger N

- Increased measurement delay: Every extra period we wait, it multiplies the sweep delay by T_CSI-RS ot T_SBB risking stale inputs to your AI/ML model.

- Higher UE energy cost: The RF and beam-steering logic keep running across many periods, drawing more power and potentially delaying sleep in DRX.

- Slower reaction to channel changes: In fast-moving or deep-fade scenarios, a large N can let the serving beam degrade significantly before the next full measurement arrives.

Basee on the above we have the following proposal:

- (BM-Case 1, spatial prediction) RAN4 should not alter the existing TS 38.133 measurement‐delay formula for BM-Case 1, instead, networks should be encouraged to optimize the spatial‐sweep factor N by sizing their CSI-RS resource‐sets to match exactly the number of Rx-beam measurements required for AI/ML inference. By keeping M=1 and P=1 and simply adjusting the resource-set count, the gNB can directly trade CSI-RS overhead against measurement latency. This approach preserves full compatibility with legacy and gives operators freedom to hit a desired delay-overhead point for single-snapshot inference

BM-Case 2 (temporal-domain prediction, observation window)
The AI/ML predictor needs K successive time-domain measurement snapshots to learn speed, direction, fading trends, etc. Let denote K_obs be that window length of measurements

The implications on the measurement delay requirements we discussed above are:

  - Impact on M: Must be re-interpreted as M = K_obs (e.g. 5–20), because each fresh sweep contributes another point to the ML feature vector. This is not covered by today’s “1 or 3” rule; a new higher-layer parameter is needed.

  - Impact on N: Same spatial-sweep logic applies; however, because M grows, the product M × N can easily explode. To bound latency the network should again enlarge N_res per set and/or shorten T_CSI-RS.

  - Impact on P: Still best kept at 1; if unavoidable overlaps exist, the ML scheduler must accept a stretched window, or interpolate missing points.

Using M = K_obs writes the required time-series observation window explicitly into the requirement, ensuring that every UE delivers a uniform-length history to the inference engine without making the gNB guess how many measurement samples the device is caching.

We are proposing a spec-enhanced path for BM-Case 2 and describe the key points below:

  - Introduce a new RRC parameter observationWindowLength that maps 1-for-1 to the M in the measurement-period formula (range 1…16).

  - Retain the existing N equation but allow N_res per set up to 8 so latency can scale down as M scales up.

  - NW network should schedule CSI-RS to avoid overlaps, else the UE must flag which samples are missing so the AI/ML engine can perform a gap-fill (through interpolation for example). If the network arranges its CSI-RS occasions so they never collide with SMTC slots or measurement gaps, then every scheduled CSI-RS burst is usable. In spec terms this means keeping the availability factor P = 1, so the delay-formula remains a clean M × N × T_CSI-RS with no hidden inflation.  But if absolute alignment isn’t possible because of DRX, multi-cell coordination, or tight scheduler loads then some CSI-RS bursts will be skipped in those scenarios. In these cases the UE cannot silently pretend they arrived. Instead, the UE must annotate its L1-RSRP (or beam-prediction) report with a simple bitmap or index list that marks which of the M × N expected measurement instants actually occurred.  Also the UE model resamples or interpolate exactly where data is missing so the temporal features stay meaningful;

- (BM-Case 2, temporal/spatial prediction) RAN4 should make the temporal depth M a configurable RRC parameter with a defined range, retain the existing spatial-sweep rule N = ⌈maxNumberRxBeam / N_res_per_set⌉ while allowing limited-size resource sets to keep latency bounded as M grows, and mandate P = 1 by default via CSI-RS scheduling that avoids SMTC/DRX collisions; when perfect alignment isn’t possible, require the UE to add a small per-report “gap” flag indicating which of the expected measurement instants were actually captured/used, so the network can judge prediction quality.

How to define requirements

Usually, when we define accuracy requirements in RAN4, the requirements defined is scenario agnostic. Take L1-RSRP measurement accuracy as an example. The requirements are applicable to all the channel models, all UE speeds, all BS radiation patterns and etc. But for AI/ML BM models, it could be the case where it seems not possible to define a scenario-agnostic performance requirements.

As mentioned above, for each BM sub-case, there are different scenarios. If it is not possible to define scenario-agnostic performance requirements, we shall discuss which scenarios to define requirements and test cases for. Take BM case 1 as an example. There are several candidate scenarios:

- Scenario 1: Set A and Set B are different (Set B is NOT a subset of Set A). E.g., use SSB beams to predict CSI-RS beams. This corresponds to wide to narrow prediction.

  - Scenario 1a: AI model inputs are the RSRP with best Rx beam for each Tx beam in set B (repetition ON)

  - Scenario 1b: AI model inputs are the RSRP with specific Rx beam for each Tx beam in set B

- Scenario 2: Set B is a subset of Set A. This corresponds to narrow to narrow prediction or wide to wide prediction

  - Scenario 2-1 (SSB) : use part of SSB beams to predict all the SSB beams.

    - Scenario 2-1a: AI model inputs are the RSRP with best Rx beam for each Tx beam in set B (repetition ON)

    - Scenario 2-1b: AI model inputs are the RSRP with specific Rx beam for each Tx beam in set B

  - Scenario 2-2 (CSI-RS): Use part of CSI-RS beams with repetition off to predict more CSI-RS beams with repetition off

According to the simulation assumptions agreed for AI/ML BM in R4-2505152, only scenario 2-1a and 2-2 are included..

The use of beam-pair prediction models is an internal UE design choice. Since beam-pair prediction is out of scope for the current WI, we propose no separate requirements or conformance test cases be defined in this release; instead, capture it as a non-normative note for future study and revisit the topic of Rx beam prediction for RRM measurement reduction in a 6G/next 3GPP release.

- RAN4 should first decide whether truly scenario-agnostic performance requirements are achievable; if not, we propose agreeing a scenario list to anchor requirements and conformance tests, starting with BM-Case 1. Concretely, define: (i) Scenario 1 (Set A ≠ Set B; wide→narrow), e.g., use SSB beams to predict CSI-RS beams, with two input options: 1a “best-Rx-beam RSRP per Tx in Set B” (repetition ON) and 1b “specific-Rx-beam RSRP per Tx in Set B”; and (ii) Scenario 2 (Set B ⊂ Set A; narrow→narrow or wide→wide), comprising 2-1 (SSB), predict all SSB beams from a subset, with 2-1a best-Rx input (repetition ON) and 2-1b specific-Rx input, and 2-2 (CSI-RS), predict additional CSI-RS beams from a subset with repetition OFF. The use of beam-pair prediction remains a UE implementation choice and is out of scope for this WI; we propose no separate requirements or test cases for beam-pair prediction in this release and instead record it as an item for future study (e.g., Rx-beam prediction for RRM measurement reduction) in a next 3GPP/6G release

Impacts on requirements related to candidate beam detection

In current RAN4 requirements, as mentioned in Clause 8.5.6.2 of TS 38.133, the UE is supposed to evaluate during a certain time period the L1-RSRP measured on the configured CSI-RS resource in the candidate beam set (called q1) containing CSI-RS resources configured for a serving cell. The objective is to verify if the L1-RSRP measured on the configured CSI-RS resource has not been degraded too much and is still better than a certain threshold (Qin_LR). The certain period used to evaluate L1-RSRP is known as TEvaluate_CBD_CSI-RS

Clause 8.5.6.2 of TS 38 133 tells every UE to monitor the quality of each configured candidate Tx-beam by averaging its L1-RSRP over an evaluation window TEvaluate_CBD_CSI-RS  = ceil( MCBD ⋅P⋅N⋅PCBD )  TCSI-RS

In Release-16/17 networks this worked because every “use-worthy” beam had a short, uniform CSI-RS period (TCSI-RS ≈ 5 – 10 ms), so the window stayed well under a beam-coherence time and beam recovery was prompt.

However, in the AI/ML design the gNB often transmits CSI-RS on Set A beams far less frequently say every 40 ms instead of every 5 ms because it now relies on the UE’s model to predict which of those beams will be good. If the network simply takes those same, slowed-down CSI-RS bursts and also designates them as the candidate-beam resources that feed Clause 8.5.6.2, then the evaluation formula must use that long ​ TCSI-RS value. In other words, the “health-check” beams inherit the longer period that was chosen purely to save downlink overhead, rather than a period optimised for fast degradation detection. Because the evaluation window equals “(fixed burst count) × TCSI-RS​,” putting a long period into the formula stretches the window proportionally and delays beam-failure detection. During that longer window a bad beam can fade or be blocked long before the UE declares it unhealthy, raising the probability of beam failures and negating AI/ML’s expected robustness gains.

- RAN4 is invited to study under realistic Set A/Set B CSI-RS patterns the impact of elongated TEvaluate_CBD_CSI-RS  ​ on beam-failure rate, and to evaluate mitigation options such as (i) defining a separate, short-period “health-check” TCSI-RS for candidate beams, (ii) capping TEvaluate_CBD_CSI-RS  ​ to a fixed upper limit (e.g. 40 ms or one DRX cycle), or (iii) scaling the freshness factor MCBD​ downward when the serving beam is AI/ML-predicted;

- The goal would be to recommend in Rel-19 either a revised formula or complementary scheduling guidelines that keep beam-recovery latency within the pre-AI/ML envelope while preserving the downlink-overhead benefits of sparse CSI-RS transmission

Known Condition for TCI state

The known condition for TCI state is defined based on the period between TCI state switch and the last transmission of RS or RS QCLed with target TCI state. If the target TCI state is a predicted beam, and the RS was not transmitted to, measured and reported by UE, then we need to introduce a new definition for known condition. We also need to differentiate between UE side prediction and NW side prediction. For the case when target TCI state is a RS based on beam prediction -

UE side model: The UE predicts and reports the top K beams based on measurements on set B. Since the UE is predicting the beam(s) reported, it is known to the UE if a TCI state is received within a certain period. The known condition can be determined based on last transmission of set-B.

NW side model: The UE reports the measurements for Set B and target TCI state is based on NW side prediction, the TCI state is unknown to the UE. In case the RS of target TCI state is transmitted to the UE, measured and reported prior to the TCI state switch, then it is known to the UE.

Figure 1: Known condition for UE side prediction

- For UE side model known TCI state is determined based on Set B transmission and UE report of top-K.

- For NW side model: (1) TCI state is unknown if RS of target beam is not transmitted to UE prior to TCI state switch (2) If RS of target beam is transmitted to the UE and reported, legacy definition of known TCI state is applicable

For UE side Rx beam assumption, there are also different assumptions:

Alt 1: UE always uses the best Rx beam to measure set B beams and set A beams and the corresponding L1-RSRP

This imply beam sweeping in the data collaboration procedure at the UE side for set A and set B measurement.

In training data collection for NW side model based on MDT framework, the UE report the gNB beam with corresponding L1-RSRP with best UE Rx beam.

During inference, UE report the L1-RSRP and corresponding NW beam index(es) (CRI/SSBRI) assuming the best UE Rx beam.

How best UE Rx beam is determined requires advanced UE side Rx beam management implementation. For example,

UE record the best Rx beam in global coordinates during training,

During inference, based on sensor information and UE orientation, to figure out the best Rx beam to use for the set A.

During performance monitoring, best Rx beam for set A and set B respectively is used.

Alt 2: UE always use the best Rx beam for set B measurement, and UE used the best Rx beam of set B for set A measurement during training and inferencing.

This imply that UE beam sweeping for set B measurement during training and inference.

In addition, using data collection for training procedure, the RS configuration in gNB needs to indicate the root RS of set A configuration with respect to set B RS.

There can be proprietary information disclosures concern at the NW vendor. There is no agreement in RAN1 (no related proposals either in this case).

In training data collection for NW side model based on MDT framework, the UE report the gNB beam with corresponding L1-RSRP with best UE Rx beam for set B, and set A L1-RSRP with best UE Rx beam QCLed to set B beam.

During inference, UE report the set B L1-RSRP and corresponding NW beam index (CRI/SSBRI, can be more than 4) assuming the best UE Rx beam for set B.

The same QCL type D root RS to set B should be signalled by the NW, as the training phase.

- NW side data collection, QCL type D is signalled between set A and set B RS.

- For NW side model, TCI state is known if QCL source is signalled in both data collection and inference (to ensure UE Rx consistency), it is known. Otherwise, it is unknown

- (for Alt 1):  Define UE capability for Alt 1 of UE implementation. When UE does not support this capability, the TCI state is unknown. Unknown TCI state switching delay is applied

- (for Alt 2): When gNB signal the QCL relationship between set A and set B, in training RS configuration, inference RS configuration and performance monitoring RS configuration (example of Alt 2 NW beam implementation of set A and set B),

The TCI state is known when the following is met:

Target TCI state is based on RS from set A beams with QCL relationship configured to a set B beam

The time between TCI state switch command and transmission of the QCLed RS from set B shall not exceed X ms

X = 1280 ms (legacy)

UE shall send measurement report for the QCLed RS from Set B between transmission and TCI state switch

TCI state and SSB of TCI state remain detectable during TCI state switching period

Otherwise, it is unknown.

In the past few meetings, RAN4 discussed whether Rx beam information is known for predicted beams.

Issue 3-2: UE Rx beam knowledge Agreement: - Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS - RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not. - Other way to know UE Rx beams is not precluded - UE capability is one of the solutions

Based on our discussion above we would like to make the following proposal:

- The ability to identify the Rx beam associated with an unmeasured predicted Tx beam may rely on the UE’s supported features.

LCM and performance monitoring related requirements

Agreement (RAN4#114bis)

- RAN4 has the following understanding and need further check whether the understanding is aligned with RAN1 and RAN2:

- AI/ML functionality activation refers to the process of enabling an applicable functionality to perform inference

- functionality deactivation refers to the process of network deactivating an active functionality

Agreement (RAN4#113)

- Whether transition from AI to non-AI would be UE initiated or network initiated

  - This will be a decision for other groups

  - RAN4 to assess what RRM requirements are needed for each case.

Agreements from RAN4#113, RAN4#114bis, agreements from RAN2#129 meetings and agreements from RAN1#119

RAN2#129 Agreements

- If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

- Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework:

  - Semi-persistent reporting, activated by MAC CE/DCI

  - Aperiodic CSI reporting, activated by DCI

RAN1#119 Conclusion

For the CSI-ReportConfig for inference configuration provided in Step 5,

- aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete.

- periodic CSI Report is considered as activated after RRCReconfigurationComplete.

- Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig

  - Any specification impact is a separate discussion

In “classic” NR the chain from configuration to usable CSI reports is predictable. A full RRC Reconfiguration configures or rewrites every parameter the UE needs; once the UE returns RRCReconfigurationComplete the gNB can assume the report engine will start after a deterministic warm-up that equals one CSI-RS period (plus, at most, the fixed freshness factor in TS 38.133). Operators rarely have to worry about the small difference between the moment the RRC message finishes and the instant the first report appears.

AI/ML beam prediction breaks that comfort zone because the content of a CSI-report now matters far more than the mere presence of a report:

#### Issue 1: Model-handling latency

A freshly signalled CSI-ReportConfig can reference an AI model that is not yet loaded on the device. The UE may need to fetch weights from flash, verify a signature, allocate accelerator memory, or even download a new version.
This load time (tens of ms on mid-tier devices) is invisible to the gNB, so it can schedule a PUSCH grant long before the model is ready, wasting UL resources or triggering a “missing-report” fault.
This issue could be solved by adding a UE-capability field like U_proc_max (“worst-case model-handling time”) and let the network include that value in a new latency budget timer T_LCM so the scheduler knows the latest safe instant to expect the first AI/ML report from UE.

#### Issue 2:  Observation-window latency (BM-Case 2)

Sequence-based predictors need K_obs consecutive CSI-RS snapshots before a single beam prediction is possible. This could be an issue because legacy signalling never encodes that window length, so the gNB again has no clue when the first valid prediction can appear.
We could address this issue by introducing an RRC parameter observationWindowLength = K_obs that maps 1-for-1 onto M in the measurement-delay formula. The network then computes the earliest report time as U_proc_max + K_obs × N × T_CSI-RS.

#### Issue 3:  Frequent mode toggling

AI/ML control loops could toggle between “predicted” and “legacy” beams far more often than classic mobility events ever triggered RRC re-configs. AI/ML beam prediction operates in a closed loop: the network continually monitors how well the predicted beams perform and via life-cycle-management (LCM) signalling enables AI/ML when accuracy is high and falls back to legacy measured beams the moment confidence drops.

In mmWave and upper-mid bands, blockage or fast geometry changes can push the prediction hit-rate below threshold within a few hundred milliseconds; once the UE flags that “inapplicability,” then the gNB de-activates the model and reverts to traditional Set A measurements.

If conditions improve e.g., the UE rotates back into line-of-sight or the model parameters are refreshed the network can quickly re-activates AI/ML to regain overhead savings. Because channel quality at these frequencies can fluctuate on sub-second time-scales, this enable → disable → re-enable loop can cycle far more frequently than the macroscopic events that used to justify an occasional RRC Reconfiguration, thus creating a new class of rapid-fire LCM operations that legacy requirements never had to time.
This could be an issue because each toggle re-invokes activation or deactivation timing, so any uncertainty in those latencies now could appear very frequently, thus compounding wasted UL grants and false beam-failure alarms.
Therefore, we could envision a lighweight Fast LCM that toggles AI/ML beam prediction on or off via a single MAC/DCI flag instead of a full RRC reconfiguration.

#### Issue 4:  Two activation paths with largely different timing

#### It seems there could be two paths with largely different time scales:

Slow path: A brand-new RRC message installs a fresh CSI-ReportConfig and a model file with full latency = SRB transfer + U_proc_max + measurement warm-up.
Fast path: An already-signalled report is toggled ON via DCI or MAC CE; the model is cached, so latency = one PDCCH round-trip + measurement warm-up.

This could be an issue because the difference (for example ∼40 ms vs 5 ms) is big enough to confuse the scheduler unless the chosen path is explicit. There are two activation paths because the network sometimes has to install a brand-new AI/ML configuration and other times only needs to toggle an already-installed one. When a fresh CSI-ReportConfig or a new model file must be delivered, the gNB has no choice but to send a full RRC Reconfiguration In order to address the issues from the existence  of two paths with largely different time scales we propose: adding a one-bit LCM-Path flag in the enable message: 0 = slow (RRC), 1 = fast (DCI/MAC).  This can be combined with U_proc_max and K_obs so the network can deterministically time the first UL grant.

- Because AI-driven beam prediction introduces timing uncertainty, adopt a lightweight and flexible LCM framework that (i) exposes the UE’s worst-case preparation time, (ii) signals the required measurement-history depth, and (iii) flags whether activation uses a fast toggle or a full reconfiguration. With these inputs, the scheduler can predict the first valid report, handle rapid on/off cycles cleanly, and place uplink grants accurately for both quick toggles and full reconfigs.

    - Performance monitoring related requirements

Agreement (RAN 1 #120bis)

- For UE-sided model monitoring Type 1 option 2, regarding the type of resource for the set for monitoring, support at least periodic CSI-RS, semi-persistent CSI-RS and SSB

Agreement (RAN1 #119)

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling:

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

  - FFS on whether to support all the combination on time domain behavior of the reportConfigType for inference report and the reportConfigType for monitoring report

  - FFS on the timing related issues

  - UE measures the dedicated resource set(s) for monitoring.

Agreement (RAN4#114bis)

- Delay requirements for performance monitoring reporting from the UE (e.g.., Type 1 Option2, UE-assisted monitoring for BM) in case-by-case manner

- FFS on which use cases the requirement will be defined for

- FFS on the details for each use case in WID

Following agreement was reached in RAN4#114bis meeting on the delay requirements for reporting/monitoring in general aspect.

Working Assumption (RAN 1 #120b)

For BM-Case 2, at least support to report one beam prediction accuracy for one configured time instance, configured by one CSI-ReportConfig for monitoring,

- only one resource set is configured in the CSI-ReportConfig

- the one configured time instance (i.e. f-th time instance of the time instance in one inference report) for metric calculation is configured in the CSI-ReportConfig for monitoring

- FFS on whether to configure more than one time instance

- the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource with linked time instance, no later than CSI reference resource corresponding to the CSI report for monitoring

- N (N>=1) is configured in the CSI-ReportConfig

- FFS on additional rule for counting N linked pair

- measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring.

  - Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS/SSB resources for monitoring, [FFS on the f-th time instances] are no later than the CSI reference resource corresponding to the CSI report for monitoring

  - FFS: whether to introduce a threshold X, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance

Working Assumption (RAN 1 #120bis)

At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring,

- for BM-Case 1, measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring.

  - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring

  - FFS: whether to introduce a threshold X for the minimal slot offset, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report.

Agreement (RAN 1 #120b)

For UE-sided model monitoring Type 1 option 2, support the following combination for inference report type and monitoring report type:

| Monitoring report type Inference report type | P report | SP report | AP report |
| --- | --- | --- | --- |
| AP report | Not support | Not support | Support |
| SP report | Not support | Support | Support |
| P report | Support | Support | Support |

- With a UE-side AI/ML model, the UE must first gather N valid monitoring instances inside a window, compute a KPI/event, and then uplink it. Legacy RAN4 delay rules only covered “one measurement → one report”; they never bounded (a) the window duration itself, (b) UE processing to compute the KPI, or (c) the UL scheduling wait. So RAN4 now has to cap the total time from window start (or Nth valid sample) to KPI/event arrival at the gNB.
For UE-side performance monitoring, delay is the elapsed time between a clearly defined start event and the instant the gNB actually receives the KPI/event report. Two natural start points exist:

- Window-start based: when the network (or UE) arms the monitoring window and the first valid sample is taken.

- Window-end based: when the UE has just collected the N-th valid sample (i.e., the metric is computable).

From that start point to gNB reception, the total delay is the sum of:

- Window accumulation time: time to gather N valid instances.

- UE processing time: compute the metric / evaluate the event condition.

- UL scheduling time: wait for the next PUCCH/PUSCH granted by the configured CSI-ReportConfig.

- Legacy RAN4 requirements never had to care about this processing chain, because the legacy reports were following the  “measure once → report once”  format with a fixed, specified measurement delay. In AI/ML, the UE must first build a window of N valid predictions/measures, then do extra processing to compute a metric or check a threshold, and only then send this information to the uplink. None of those durations (window length, UE compute, UL grant wait) are currently bounded or even transparent to the network. As a result, the gNB can’t predict when the KPI will arrive, can’t schedule uplink efficiently, and doesn’t know how long to tolerate silence before declaring the AI/ML function unreliable. That uncertainty is new and specific to UE-assisted AI/ML monitoring.

- We are discussing the following options to address the missing end-to-end delay bounds for UE-side AI/ML performance monitoring (window → compute → uplink).
Each option gives the gNB just enough visibility to schedule intelligently and trigger fallback without altering the core CSI framework:

Option 1:  One explicit “Performance-Monitoring Report Latency” timer
Define a single end-to-end timer (start = window armed or Nth valid sample, end = KPI/event received at gNB). The gNB signals the window length (or N) and a small UL-scheduling budget; the UE advertises a worst-case compute time. The UE must then meet the summed bound. This makes the first-KPI arrival deterministic without changing CSI procedures.

Option 2 (two-step “progress + result” reporting):
As soon as the UE progresses to collecting N valid samples, it sends a tiny Progress flag on the next control opportunity (PUCCH UCI bit, MAC CE, etc.). This immediately tells the gNB that the metric is being finalized, so the network no longer guesses whether the UE is still accumulating samples or stuck processing. After computing the metric or evaluating the event, the UE transmits the full KPI/event using the configured CSI-ReportConfig.

To make this two-step reporting system robust and standardized, RAN4 would need to define three key elements: (1) the trigger condition for sending the progress bit, such as when the UE has collected a sufficient number of valid samples (e.g., N CSI-RS measurements); (2) a maximum allowed time gap between the transmission of the progress bit and the final KPI or event report, ensuring predictable latency and enabling the gNB to take timely action; and (3) how to reuse existing signalling mechanisms, such as spare bits in PUCCH or MAC CE, for transmitting the progress indication, thereby minimizing overhead and avoiding the need to introduce new signalling formats.

Option 3 (“fast event path”)

Give the UE a panic button for performance monitoring: the moment a configured metric crosses a threshold (e.g., prediction-hit rate drops below X %), the UE can bypass the normal window/periodic schedule and send an immediate, lightweight aperiodic report. Instead of waiting until the full monitoring window closes or the next periodic KPI slot arrives, the UE uses the next available PUCCH/UCI bit, MAC CE, or a gNB-triggered aperiodic CSI-report to indicate “event tripped AI/ML is unreliable now.” As soon as the gNB gets the “bad metric” event, it can drop trust in the UE’s predictions and immediately flip back to the safe, measured path: activate (or unmute) the CSI-RS/CSI reports for Set B beams, stop using Set A predictions, and if desired push a fresh model or parameters so the UE can employ the AI/ML function later. In practice this is just a fast MAC/DCI or short RRC command sequence that swaps configurations and restarts the monitoring loop without waiting for the next periodic KPI slot

- RAN4 should address the lack of end-to-end delay visibility for UE-side AI/ML performance monitoring (window → compute → uplink) by adopting: (1) a single “performance-monitoring latency” budget that bounds the whole chain; (2) a two-step “progress + result” scheme where the UE first flags the window is ready and later sends the full KPI/event; and (3) a “fast event” path allowing the UE to immediately signal threshold breaches via a lightweight aperiodic report so the gNB can fall back.

Delay requirements for applicable AI/ML functionality

The agreements following RAN2#127bis and RAN2#129bis related to applicable AI/ML functionality for BM use-case are as follows:

Agreement (RAN2#127bis)

Agreement (RAN2#129bis)

Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

For the purpose of discussion of AI/ML BM LCM operations, existing procedures and terminologies from the CSI Framework should be used, including those defined for aperiodic, semipersistent on PUCCH, semipersistent on PUSCH, and periodic reporting configurations (as/if defined in RAN1 pending response LS from RAN1).

RAN2 agreed as working assumption on five steps procedure, intended to cover the main signalling steps from UE capability enquire up to NW provisioning and activation of the ML functionality.

RAN2 has already agreed to describe Life-Cycle-Management (LCM) and Applicability procedures for this feature using exactly those CSI-framework terms and a five-step capability-enquiry → provision → activation chain.
A new problem now arises for RAN4: what happens, and how quickly must it happen, when the UE discovers that the configuration has become “inapplicable” (e.g. wrong model version or missing prerequisite feature)?

In such a scenario,  the UE and/or gNB must, before activation and continuously afterwards, verify that every inference prerequisite remains valid: model version, CSI-RS mapping, observation-window length, feature availability, or additional conditions etc. Should any prerequisite fail, the configuration becomes inapplicable even though the Layer-3 parameters themselves are unchanged.

The UE does not autonomously deactivate the AI/ML function. It raises a flag typically a UAI message to inform the network that its beam predictions are now unreliable, yet it continues to transmit the already configured periodic prediction reports. RAN2’s working assumption is that the gNB must respond: deactivate the AI/ML function and/or reconfigure the UE to report Set A (directly measured) beams instead.

While the UE is still sending unreliable Set A predictions, the actual serving beam could potentially degrade. If the network keeps the same long CSI-RS periodicity it originally chose for Set A beams, the timer that guards beam-failure recovery extends in time. A late reaction increases the risk of a real link failure.

RAN4 therefore needs to set delay bounds that cover the entire sequence:

- Detection-to-flag delay from the UE detecting inapplicability until the gNB receives the UAI flag.

- Network reaction delay  time for the gNB to decide and signal de-activation or fallback.

- Fallback measurement delay time for the UE to start reporting reliable Set A measurements under the new configuration.

Only if this end-to-end latency is bounded can the system guarantee beam-failure recovery within an acceptable limit once AI/ML output becomes invalid.

Let  TAI-Inapplicable denote the maximum end-to-end time from the moment the UE first flags its AI/ML beam prediction as inapplicable to the moment the UE starts sending reliable fallback measurement covering the UE’s uplink flag, gNB processing and signalling, and the first valid fallback report. We need to establish an upper bound, because the “inapplicable → fallback” sequence can stretch arbitrarily long, and that could create real, testable problems:

- (1) Link robustness: Every extra millisecond the network trusts a bad prediction increases the chance of beam failure or RLF before recovery starts.

- (2) Deterministic scheduling: The gNB must know the latest moment a valid fallback report will show up to time UL grants, handover/beam-switch, and KPI checks; an unbounded delay could make the NW to over-provision or guess.

By anchoring the bound to an integer multiple of existing periods (the slower of T_CSI-RS), the gNB can compute it with known timers and labs can verify it.

- RAN4 should set one end-to-end timer, TAI-Inapplicable, so that from the instant the UE flags inapplicability to the first reliable fallback report, everything (UL flag, NW reaction, signalling, new report) finishes within K × T_CSI-RS_max, the slower of the relevant CSI-RS periods. This single cap prevents drawn-out beam recovery yet relies only on existing UAI signalling and CSI-report reconfiguration.

Testing framework for AI/ML based Beam Management

The testability issues for AI/ML based BM use case were highlighted by multiple companies during previous few RAN4 meetings but the discussions continued without any concrete conclusion on the topic. Below are the agreements/summary of the progress on this topic during the past RAN4 meetings which are documented in the corresponding WFs.

Agreements from RAN4#110 meeting Issue 2-3: Test setup feasibility for FR2 Companies are invited to provide further analysis on what the test setup should enable in terms of test environment Agreements from RAN4#113 meeting 2.3.2 Agreements in main session: Companies are invited to bring further analysis on the test setups with single AoA and multiple AoAs. Agreements from RAN4#114 meeting 2.4 Testing for one sided models Issue 4-1: Baseline test setup Agreement: - Discuss both single AoA based and multiple AoA based set up - Continue to study the two multiple AoA candidate setups - One option to continue the discussion proposed as following: - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs Issue 4-2: Channel model Agreement: - Use CDL-based channel model as starting point Agreements from RAN4#114-bis meeting Issue 2-6: Test system setup Companies are invited to bring further analysis on the test system setup. At least the following aspects to be considered in the analysis: - Whether the test can/should reflect performance in the field and whether field data should be usable in the test - What channel model should be used - UE prediction model input (e.g. whether any spatial information like UE Rx beam, UE beam pattern, etc is used) - Pros and cons of proposed single AoA and multi AoA setups For the multi AoA test setups, companies are invited to bring further analysis on the following aspects: - Define candidate test setups which are already feasible (e.g. based on eIFF, MPAC etc.) - Propose simplified channels suitable for the defined candidate test setup(s) - Validate the proposed simplified channels through AIML BM system level simulations, especially by comparing the performance with the non-simplified (theoretical) channels. - Assess the implementation of the simplified channels on the candidate systems, after the probe layout and the channel model have been clarified.

Recent RAN4 agreements show that there is a need for a solution for the conformance testing of AI/ML based BM use case which not only be based on the existing test solutions but should also be AI/ML BM performance centric.

As there was not much progress on finalising the testing setup in the previous RAN4#114bis meeting, we reiterate some of our previous views along with providing our further views and solutions for the conformance testing of AI/ML based BM use case.

    - Test setup based on single or multiple AoA(s)

- For a while, RAN4 has been actively discussing whether the set up for the conformance testing of AI/ML based beam management use case should be based on single AoA or multiple AoAs. Undoubtedly, each of these test setups have their own pros and cons, which were discussed during the past RAN4 meetings. The main challenge is the emulation of Set A beams during the conformance testing to extract the ground truth which depends on the UE architecture and implementation. Also, RAN4 has agreed that Set A should have 32 beams for the conformance testing. So, the question is how to generate 32 beams inside the OTA test chamber during the conformance testing?

- Ideally, all the beams in Set A and/or Set B should be generated without the UE rotation as the similar scenario will be observed in the real deployments. However, based on the TE vendors’ input, only a limited number of beams with different AoAs (RX-PAS profiles) can be emulated inside the test chamber due to which multiple beams in Set A and/or Set B will arrive at the DUT from the same/similar direction(s).

We below provide two alternatives to emulate the Set A and Set B beams during the conformance testing, one of which is based on the concept of UE rotation while the other solution does not assume any kind of UE rotation.

      - Multiple AoA based testing setup

To verify the accuracy of the L1-RSRP prediction for BM case 1 and case 2, we will need to introduce enough randomness in the spatial and temporal domain.  As it is shown in Fig 2, the propagation environment characterized by the different AoAs and AoDs, effectively sample the radiation beam patterns across the mainlobe and sidelobes of each of the beams in set B/set A at Tx and Rx antenna arrays. Fig 2 shows the radiation pattern of one beam inside set B or set A. The spatial mapping from set B to set A is learnt through all possible samplings of set B Tx beam patterns according to the AoDs along with samplings of the Rx beams across AoAs. AI/ML can learn the mapping from set B samples to set A samples by exploiting the correlations of the radiation patterns. For this learning to be complete we should sample the radation patterns at many points in spatial domain. This could be achieved by introducing randomness in AoA/AoD domain since the channel response is determined by the Tx beam response at the direction of the AoD.

In the presence of multiple AoAs and AoDs, the resulting L1-RSRP is the superposition of these spatial beam samples at both Rx and Tx. The spatial mapping of set B L1-RSRP to set A L1-RSRP is learnt by proving enough random data  to capture the spatial correlations across different angles of departures and arrivals as well as the different superpositions of those angles. The correlations inferred will depend on the specific mainlobe as well as the sidelobe shapes of the setA/B patterns which will have to configured in the test and emulated by the test equipment. Therefore, the AI/ML model should be trained and tested with enough randomness.

The testing environment for Beam Management (BM) case 1 and BM case 2 should replicate conditions that sufficiently capture the correlations of the transmit (Tx) and receive (Rx) beam patterns across the entire spectrum of propagation conditions in both spatial (angles of arrival and departure - AoAs and AoDs) and temporal domains.

The following conditions will introduce randomness and variations in propagation conditions across both time and spatial domains for the computation of L1-RSRP.

- Different AoDs with respect to the Tx antenna array

- Different AoAs with respect to the Rx antenna array

- Different superpositions of {AoA,AoD} pairs

- Fading/Variation in time domain ( different {AoA,AoD} pairs per resolvable delay bin path)

- UE movement (including rotation)

It is expected that during deployment in the field, all the aforementioned randomness and variations will be inherent in the real propagation environment. Therefore, the AI/ML model should be tested under similar conditions, otherwise, the UE could easily pass the test and fail during real deployment scenarios.

During real-world deployment, UE will encounter random radio propagations characterized by variations in both spatial and temporal domains (fading). Testing UE under similar conditions is important to reflect the realities of deployment accurately

Fig. 3: Beam pattern spatial domain sampling across AoA/AoD

The baseline measurement setup for RRM characteristics and SNR reference point for RRM testing procedure based  on TR 38.810 [4] for the study on test methods specifically for FR2 UE beam management is shown in Fig 3.

Fig. 3:  UE RRM test methodology for FR2 Beam Management

For the testing goals of BM AI/ML use case as described in the paragraph above, there are two main limitations with the existing test methods:

- Limitation on the number of simultaneously active AoAs (=2)

- Limitation of the relative angular separation between AoA pairs: 30°, 60°, 90°, 120° and 150°.

There are questions regarding how to meet the testing goals and RAN4 requirements using the FR2 OTA chamber, particularly with the necessity of sweeping beam patterns across Set-A and Set-B, but with the limitations of only two Angle of Arrivals (AoAs) and coarse angular separation. Moreover, for BM-Case2, to evaluate temporal DL Tx beam prediction, there should be some variation in beam direction over time, including UE rotation.

The following questions need to be answered to evaluate the feasibility of the FR2 OTA-based test procedure:

- How can we generate multiple beams from the Set-B and Set-A Tx beams given the limitation of two AoAs?

- What assumption is made regarding Rx beam sweeping? Does the UE utilize a fixed Rx beam, or does it sweep to find the optimal Rx beam?

- How can we simultaneously emulate different AoAs (Rx beam) and AoD (Tx beams)?

- How can we achieve dynamic variation in the AoD domain (Tx beam sweeping) for BM case 2 prediction?

It was concluded that we would need for the current 3D MPAC to approx. >100 probes i.e. also  >200 RF channels (to count for dual polarized probes) for emulating CDL-C for AI/ML based BM testing. This would be simply unfeasible. Thus, further simplifications of the test scenarios are necessary. For feasible implementation of an OTA test setup for AI/ML based beam management, further simplifications are inevitable.

For beam management, the primary goal is not to develop an exact or near-perfect representation of the resulting CDL channel model in static scenarios (such as FR2 MIMO OTA analyzed in earlier sections). Instead, the focus is on emulating a time-varying power environment with limited resolution, thereby avoiding the need for physical simulation of angular spread at both the cluster and overall levels

- Multi-AoA based Testing Setup for BM: Evaluate the testability requirements for simulating time-varying input power to a sparse probe layout based on a simplified channel CDL model.  The key aspects to be considered are:

- How to determine the minimum number of clusters to be emulated without introducing bias in the results. Investigate a quantitative “goodness” criterion to selecting the number of probes/clusters.

- Consider incorporating UE rotation for the test

- How many different CDL channels we need to support (considering generalization purposes as well)

For RAN 4 testing we should have the following goals:

(1) Provide confidence that if a DUT passes the test it will also perform well in the field

(2) Reliability of using synthetic channels for test data in evaluating models trained on real data.

For RAN4 testing we should aim at satisfying the following goals: (1) Provide confidence that if a DUT passes the test it will also perform well in the field (2) Ensure reliability of using synthetic channels for test data in evaluating models trained on real data

Based on the above goals we have the following:

- To establish a criterion for assessing the feasibility of a simplified sparse probe layout for beam management (BM) testing, the following procedure can be considered:

- Train a model using the reference CDL channel and test it with the simplified sparse layout-based CDL channel.

- Train a model using the simplified CDL channel and test it with the reference CDL channel model.

- Train and test a model using the reference CDL channel.

A similarity metric should be defined to compare the RSRP accuracies across these test cases. This metric would indicate the effectiveness of testing with the simplified CDL model. The similarity metric can be derived through simulations.

Fig. 4:  Channel Emulator Based Spatial channel for testing BM

Below we provide some details on how to emulate a CDL channel with a time varying power across the AoA probes.

During RAN4#110-bis a spatial model was proposed to be implemented by the channel emulator as shown in Fig 5.

Fig. 5   Multipath based Testing Setup with a simplified CDL channel and probe layout

The following parameters that can be programmed:

Tx codebook and antenna pattern. The Tx gain is known given the codebook and antenna pattern (across 2 polarizations) The AoD angles from the Tx side. We assume that the AoA angles are fixed and they are part of the probes in the MIMO OTA testing setup. The coupling factors that couple the AoDs clusters to the AoAs clusters Gc  AoDs represent the first stage of clusters and AoAs represent the last stage of clusters. Any intermediate clusters and interactions can be abstracted by the coupling factors Gc,ij(AoAi,AoDj). The different AoDs that couple to AoA_i are denoted as AoDij.  A Tx beam (from set A or set B) is denoted as wk. A UE RX beam is denoted as um. The RX codebook is denoted as U.

By taking into consideration the testing set-up shown in Fig 5, the RSRP for a particular Tx beam is derived as shown below:

Fig. 6:  Proposed BM-Case 1 spatial prediction testing setup ( multiple static channels)

For testing the RSRP accuracy for BM-Case1 spatial prediction we can configure multiple static configuration tests, where each static scenario corresponds to a set of parameters for generating the channel realizations. For each test we configure UE to perform measurements on the set A and we treat those measurements as the genie measurements. The whole procedure is shown in Fig 6.

- For BM-Case 1 spatial prediction and for verifying RSRP accuracy we propose to use the testing setup and channel emulator functionality as described in Fig 6. (multiple static configurations

For testing the RSRP accuracy for BM-Case2 temporal prediction we can configure a non-static configuration test, where the set of parameters for generating the channel realizations vary over time.Therefore, the channel realization vary due to inherent propagation conditions that should alter the optimum Tx beam over time and introduce some randomnsee and correlation over time to be captured by the AI/ML. Similar to BM-Case 1 testing procedure, we configure UE to perform measurements on the set A and we treat those measurments as the genie measurements. The whole procedure is shown in Fig 7.

Fig. 7:  Proposed BM-Case 2 temporal prediction testing setup ( non- static channels)

- For testing the beam prediction/RSRP accuracy for BM-Case 2 temporal prediction we propose the framework described in this section and visualized in Fig 7, through a non static configuration of the channel emulator

Single AoA based testing setup

For DL Tx beam prediction, the model inputs consist of the L1-RSRP values of Tx beams in Set B, along with their corresponding Tx beam ID information, either explicitly provided or implicitly indicated. Essentially, only the final L1-RSRP value is critical, as all spatial characteristics are inherently captured in the RSRP measurements. Rather than simulating CDL channels and [64-128] Tx beams using multiple angles of arrival (AoAs) and departure (AoDs), the spatial-selective and time-varying propagation conditions can be effectively represented by adjusting the Tx power at the test equipment (TE) for each beam at different times. This approach ensures that the resulting RSRP values received at the test UE reflect the desired conditions. Notably, this method is already widely adopted in legacy RAN4 RRM testing.

Rather than simulating CDL channels and [64-128] Tx beams using multiple angles of arrival (AoAs) and departure (AoDs), the spatial-selective and time-varying propagation conditions can be effectively represented by adjusting the Tx power at the test equipment (TE) for each beam at different times

To simplify the testing setup, we can design it under the assumption of near-LOS conditions, which can be emulated using a single cluster and a single AoA probe. This is shown in Fig 7. Note that the multiple AoD links are not active simultenously. Only one link is active at a time, the purpose is to emulate the gains across different angles of the Tx antenna pattern to have multiple samples across the AoD domain and help making different Tx beams distinguishable. We also have one AoA/probe.

Fig. 8:  Single path/cluster (LOS condition) based Testing Setup

For each AoA (spatial sample) we collect RSRP inputs across setB beams (x,y) and we predict the RSRP at the same AoD for beams in set A (z). Let’s assume we consider the angle AoDj.  The received power at UE by using beam x is given by:

RSRPx,AoDj=PtGtx(x,AoDj)×Gpath(AoDj,AoA)×Grx(AoA)

Similarly the received power at UE by using beam y is given by:

RSRPy,AoDj=PtGtx(y,AoDj)×Gpath(AoDj,AoA)×Grx(AoA)

The AI/ML model will receive as inputs the RSRPs at set B beams and make a prediction as beam z (in set A):

RSRPz,AoDj=fAI/MLRSRPx,AoDj,RSRPx,AoDj

During test, we can generate multiple realizations of gains by varying the AoD as well as the gain Gpath(AoDj,AoA) to collect multiple RSRP samples across the set B beams and test the prediction against the RSRP of set A beams. The gain Gpath(AoDj,AoA) should remain constant during one realization of testing (sweeping of set B, set A beams).

In contribution [5] it was discussed that the multi-AoA test environment could be converted to a single AoA setup: The multi-AoA environment can be transformed into a single‑AoA test for AI/ML BM testing, by including the UE Rx beam/antenna pattern in the combination of BS Tx beams with the CDL channel model to be emulated, without impacting the number of BS Tx beams or the parameters of the CDL channel model.

The following options were identified:

We can differentiate 4 options how for AI/ML BM, a multi-AoA test can be transformed into a single-AoA test:

- Option 1: RAN4 defines a Reference Fixed UE Rx Beam/Antenna pattern. In this case, since the testing becomes primarily a “baseband test”, a second test would be required to address the UE spatial/directional performance.

- Option 2: RAN4 defines a Reference Complex UE Rx Beam/Antenna pattern (Codebook). Also in this case, testing becomes primarily a “baseband test”, thus a second test would be required to address the UE spatial/directional performance.

- Option 3: UE manufacturer provides the Complex UE Rx Beam/Antenna pattern (Codebook) to the TE prior to testing. Assuming that the pattern is realistic, the UE spatial/directional performance is covered in the test.

- Option 4: TE measures the Complex UE Rx Beam/Antenna pattern (Codebook) before starting test. This approach would rely on the basics of Radiated Two Stage Method (RTS) defined in [7]. In this case, the UE spatial/directional performance is covered in the test.

Similar concepts have been discussed in [6]. According to that contribution and for the setup B the following Pros and Cons were discussed:

Setup B) 1AoA + Fading simulator with wireless (virtual) cable connection

Pros

Though the number of test probe is physically one and DL signals are transmitted from one direction, it is possible to test with CDL channel models by using a fading simulator.

There are no limits to channel models (AoDs, AoAs, clusters, UE positions, etc.), and even possible to use the actual field data for the tests.

Possible to reuse the existing 1AoA OTA test system. i.e. Simpler test system compared to Setup A.

Cons

There is a need to include UE Rx beam pattern to a fading simulator, either standardized one or need to obtain actual UE beam patterns.

If the actual UE Rx beam patterns are necessary, there is a need to measure at least one beam pattern before the test to include it in the fading simulator. The number of necessary beam patterns are FFS. i.e. Longer test time is expected in a case the actual Rx beam pattern is necessary.

In a case the UE autonomously aims the Rx beam and then measures the RSRP and inputs it to the AI model in the field, it may not be possible to simulate the behaviour of the UE.

The measurements are made while beam-locked by the UBF (UE Beamlock Function) command, so they do not necessarily reproduce the UE in operation on the real network.

By considering the AI/ML spatial prediction as a purely power (RSRP) prediction problem and by considering that these power signal levels can be emulated by the fading channel emulator, we have the following proposal for a single AoA testing setup as it is shown in the flowchart in Figure 9.

Fig. 9:  Proposal for single AoA BM testing setup (flowchart)

The number of channel realizations (=640K) in the figure only serves as an example and for motivating the discussion on the tradeoff between testing complexity and sufficient number of channels to derive a test metric (top-K beam accuracy)

- RAN4 to consider the set of procedures/steps shown in the flowchart for adopting a single AoA BM testing setup

- RAN4 to evaluate the tradeoff between testing complexity (number of different channel realizations for power measurements, different number of parameters that need to be emulated in the fading simulator) and establishing accuracy on the test metric (top-K) beam prediction accuracy

- RAN4 to evaluate if a power measurement should be performed with a single Rx beam or the optimum Rx beam should be determined by sweeping Rx beams from an Rx codebook to measure the power of set B Tx beams

- RAN4 to discuss Option2 (a Reference Complex UE Rx Beam/Antenna pattern) for emulation of Rx patterns in the fading simulator

The equations described for the received power are based on powers (CDL power-based channel model). However, the channel model is described in the complex I-Q domain.

- Distinguish between the descriptions of power based CDL channel model and the original complex domain CDL channel fading simulator

- RAN4 to evaluate the prediction performance difference between a reference complex UE Rx beam/antenna pattern and its own complex pattern to ensure that testing will provide confidence that UE will perform well in the field if it passes the test

Conclusion

In the paper, the following Observations and Proposals were made:

- If we could truly guarantee that the predicted RSRP accuracy in AI/ML is the same as the measured RSRP accuracy in legacy, for every predicted beam the UE reports, then in principle, a separate beam prediction accuracy metric would not be strictly necessary.

The testing environment for Beam Management (BM) case 1 and BM case 2 should replicate conditions that sufficiently capture the correlations of the transmit (Tx) and receive (Rx) beam patterns across the entire spectrum of propagation conditions in both spatial (angles of arrival and departure - AoAs and AoDs) and temporal domains.

The following conditions will introduce randomness and variations in propagation conditions across both time and spatial domains for the computation of L1-RSRP.

- Different AoDs with respect to the Tx antenna array

- Different AoAs with respect to the Rx antenna array

- Different superpositions of {AoA,AoD} pairs

- Fading/Variation in time domain ( different {AoA,AoD} pairs per resolvable delay bin path)

- UE movement (including rotation)

During real-world deployment, UE will encounter random radio propagations characterized by variations in both spatial and temporal domains (fading). Testing UE under similar conditions is important to reflect the realities of deployment accurately

The following questions need to be answered to evaluate the feasibility of the FR2 OTA-based test procedure:

- How can we generate multiple beams from the Set-B and Set-A Tx beams given the limitation of two AoAs?

- What assumption is made regarding Rx beam sweeping? Does the UE utilize a fixed Rx beam, or does it sweep to find the optimal Rx beam?

- How can we simultaneously emulate different AoAs (Rx beam) and AoD (Tx beams)?

- How can we achieve dynamic variation in the AoD domain (Tx beam sweeping) for BM case 2 prediction?

For RAN4 testing we should aim at satisfying the following goals: (1) Provide confidence that if a DUT passes the test it will also perform well in the field (2) Ensure reliability of using synthetic channels for test data in evaluating models trained on real data

Rather than simulating CDL channels and [64-128] Tx beams using multiple angles of arrival (AoAs) and departure (AoDs), the spatial-selective and time-varying propagation conditions can be effectively represented by adjusting the Tx power at the test equipment (TE) for each beam at different times

- (BM-Case 1, spatial prediction) RAN4 should not alter the existing TS 38.133 measurement‐delay formula for BM-Case 1, instead, networks should be encouraged to optimize the spatial‐sweep factor N by sizing their CSI-RS resource‐sets to match exactly the number of Rx-beam measurements required for AI/ML inference. By keeping M=1 and P=1 and simply adjusting the resource-set count, the gNB can directly trade CSI-RS overhead against measurement latency. This approach preserves full compatibility with legacy and gives operators freedom to hit a desired delay-overhead point for single-snapshot inference

- (BM-Case 2, temporal/spatial prediction) RAN4 should make the temporal depth M a configurable RRC parameter with a defined range, retain the existing spatial-sweep rule N = ⌈maxNumberRxBeam / N_res_per_set⌉ while allowing limited-size resource sets to keep latency bounded as M grows, and mandate P = 1 by default via CSI-RS scheduling that avoids SMTC/DRX collisions; when perfect alignment isn’t possible, require the UE to add a small per-report “gap” flag indicating which of the expected measurement instants were actually captured/used, so the network can judge prediction quality.

- RAN4 should first decide whether truly scenario-agnostic performance requirements are achievable; if not, we propose agreeing a scenario list to anchor requirements and conformance tests, starting with BM-Case 1. Concretely, define: (i) Scenario 1 (Set A ≠ Set B; wide→narrow), e.g., use SSB beams to predict CSI-RS beams, with two input options: 1a “best-Rx-beam RSRP per Tx in Set B” (repetition ON) and 1b “specific-Rx-beam RSRP per Tx in Set B”; and (ii) Scenario 2 (Set B ⊂ Set A; narrow→narrow or wide→wide), comprising 2-1 (SSB), predict all SSB beams from a subset, with 2-1a best-Rx input (repetition ON) and 2-1b specific-Rx input, and 2-2 (CSI-RS), predict additional CSI-RS beams from a subset with repetition OFF. The use of beam-pair prediction remains a UE implementation choice and is out of scope for this WI; we propose no separate requirements or test cases for beam-pair prediction in this release and instead record it as an item for future study (e.g., Rx-beam prediction for RRM measurement reduction) in a next 3GPP/6G release

- RAN4 is invited to study under realistic Set A/Set B CSI-RS patterns the impact of elongated TEvaluate_CBD_CSI-RS  ​ on beam-failure rate, and to evaluate mitigation options such as (i) defining a separate, short-period “health-check” TCSI-RS for candidate beams, (ii) capping TEvaluate_CBD_CSI-RS  ​ to a fixed upper limit (e.g. 40 ms or one DRX cycle), or (iii) scaling the freshness factor MCBD​ downward when the serving beam is AI/ML-predicted

- For UE side model known TCI state is determined based on Set B transmission and UE report of top-K.

- For NW side model: (1) TCI state is unknown if RS of target beam is not transmitted to UE prior to TCI state switch (2) If RS of target beam is transmitted to the UE and reported, legacy definition of known TCI state is applicable

- NW side data collection, QCL type D is signalled between set A and set B RS.

- For NW side model, TCI state is known if QCL source is signalled in both data collection and inference (to ensure UE Rx consistency), it is known. Otherwise, it is unknown

- (for Alt 1):  Define UE capability for Alt 1 of UE implementation. When UE does not support this capability, the TCI state is unknown. Unknown TCI state switching delay is applied

- (for Alt 2): When gNB signal the QCL relationship between set A and set B, in training RS configuration, inference RS configuration and performance monitoring RS configuration (example of Alt 2 NW beam implementation of set A and set B),

The TCI state is known when the following is met:

Target TCI state is based on RS from set A beams with QCL relationship configured to a set B beam

The time between TCI state switch command and transmission of the QCLed RS from set B shall not exceed X ms

X = 1280 ms (legacy)

UE shall send measurement report for the QCLed RS from Set B between transmission and TCI state switch

TCI state and SSB of TCI state remain detectable during TCI state switching period

Otherwise, it is unknown.

- The ability to identify the Rx beam associated with an unmeasured predicted Tx beam may rely on the UE’s supported features.

- Because AI-driven beam prediction introduces timing uncertainty, adopt a lightweight and flexible LCM framework that (i) exposes the UE’s worst-case preparation time, (ii) signals the required measurement-history depth, and (iii) flags whether activation uses a fast toggle or a full reconfiguration. With these inputs, the scheduler can predict the first valid report, handle rapid on/off cycles cleanly, and place uplink grants accurately for both quick toggles and full reconfigs.

- RAN4 should address the lack of end-to-end delay visibility for UE-side AI/ML performance monitoring (window → compute → uplink) by adopting: (1) a single “performance-monitoring latency” budget that bounds the whole chain; (2) a two-step “progress + result” scheme where the UE first flags the window is ready and later sends the full KPI/event; and (3) a “fast event” path allowing the UE to immediately signal threshold breaches via a lightweight aperiodic report so the gNB can fall back.

- RAN4 should set one end-to-end timer, TAI-Inapplicable, so that from the instant the UE flags inapplicability to the first reliable fallback report, everything (UL flag, NW reaction, signalling, new report) finishes within K × T_CSI-RS_max, the slower of the relevant CSI-RS periods. This single cap prevents drawn-out beam recovery yet relies only on existing UAI signalling and CSI-report reconfiguration.

- Multi-AoA based Testing Setup for BM: Evaluate the testability requirements for simulating time-varying input power to a sparse probe layout based on a simplified channel CDL model.  The key aspects to be considered are:

- How to determine the minimum number of clusters to be emulated without introducing bias in the results. Investigate a quantitative “goodness” criterion to selecting the number of probes/clusters.

- Consider incorporating UE rotation for the test

- How many different CDL channels we need to support (considering generalization purposes as well)

- To establish a criterion for assessing the feasibility of a simplified sparse probe layout for beam management (BM) testing, the following procedure can be considered:

- Train a model using the reference CDL channel and test it with the simplified sparse layout-based CDL channel.

- Train a model using the simplified CDL channel and test it with the reference CDL channel model.

- Train and test a model using the reference CDL channel.

A similarity metric should be defined to compare the RSRP accuracies across these test cases. This metric would indicate the effectiveness of testing with the simplified CDL model. The similarity metric can be derived through simulations.

- Train a model using the re For BM-Case 1 spatial prediction and for verifying RSRP accuracy we propose to use the testing setup and channel emulator functionality as described in Fig 6. (multiple static configurations

- For testing the beam prediction/RSRP accuracy for BM-Case 2 temporal prediction we propose the framework described in this section and visualized in Fig 7, through a non static configuration of the channel emulator

Fig. 9:  Proposal for single AoA BM testing setup (flowchart)

- RAN4 to consider the set of procedures/steps shown in the flowchart for adopting a single AoA BM testing setup

- RAN4 to evaluate the tradeoff between testing complexity (number of different channel realizations for power measurements, different number of parameters that need to be emulated in the fading simulator) and establishing accuracy on the test metric (top-K) beam prediction accuracy

- RAN4 to evaluate if a power measurement should be performed with a single Rx beam or the optimum Rx beam should be determined by sweeping Rx beams from an Rx codebook to measure the power of set B Tx beams

- RAN4 to discuss Option2 (a Reference Complex UE Rx Beam/Antenna pattern) for emulation of Rx patterns in the fading simulator

- Distinguish between the descriptions of power based CDL channel model and the original complex domain CDL channel fading simulator

- RAN4 to evaluate the prediction performance difference between a reference complex UE Rx beam/antenna pattern and its own complex pattern to ensure that testing will provide confidence that UE will perform well in the field if it passes the test

- References

- RP-250792, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm Incorporated, RAN#107 meeting, 12 – 14 March 2025, Incheon, S. Korea.

- R4-2505105, “WF on the requirements for AI/ML air interface”, Qualcomm Incorporated, RAN4#114bis meeting, 7th – 11th April 2025, Wuhan, China.