3GPP TSG-RAN WG4 Meeting #116	R4-2509516

# Bangalore, India,  25th - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.863 | CR | 0031 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_solutions-Core) Correction of the NR NTN band n256 REFSENS values |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2025-07-08 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Initial Rel-17 versions of TS 38.101-5 and TR 38.863 had aligned REFSENS values for the NTN band n256. Later, NTN band n256 REFSENS values were corrected in TS 38.101-5, but the corresponding changes were not introduced into this TR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In sub-clause 7.4.3.2.2, NTN band n256 REFSENS values are aligned with the values from the normaltive specification (refer to TS 38.101-5, sub-clause 7.3.2). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be a misalignment between TS 38.101-5 and TR 38.863 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 7.4.3	UE Receiver characteristics for satellite access

#### 7.4.3.1	General

Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE. For UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s). UE with an integral antenna(s) may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For UEs with more than one receiver antenna connector, identical interfering signals shall be applied to each receiver antenna port if more than one of these is used (diversity).

#### 7.4.3.2	Conducted receiver characteristics

##### 7.4.3.2.1	General and diversity characteristics

Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE for satellite access.

The handheld UE for satellite access is required to be equipped with a minimum of two Rx antenna ports in all NTN satellite operating bands in FR1.

##### 7.4.3.2.2	Reference sensitivity

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel. For below 6GHz, the REFSENS level can be calculated by the equation below in TR 38.817-01[32]:

Sensitivity = -174dBm(kT) + 10*log(RX BW) + NF + SNR +IM – diversity gain

It is noted that the Rx BW is identical to the transmission bandwidth configuration, which is determined by the spectrum utilization. The RB values in the analysis of this contribution are based on the agreed SU for NR.

SNR in Nagoya meeting was agreed as -1dB for NR.

The REFSENS for E-UTRA assumes that the receiver is equipped with two Rx port as a baseline. If 2Rx is considered, the diversity gain is 3dB and the Implementation Margin (IM) uses 2.5dB. In the following calculation, same assumptions are used for NR NTN satellite bands.

For band n255, 9dB noise figure is assumed which is aligned with band n24.

Both dedicate 30MHz duplexer and n65 duplexer are allowed to be used for band n256. Finally, 9dB noise figure is assumed for band n256 as well.

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2 and A.3.2 from TS 38.101-1 [17] (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1 from TS 38.101-1 [17])  with parameters specified in Table 7.4.3.2.2-1 and Table 7.4.3.2.2-2.

Table 7.4.3.2.2-1: Two antenna port reference sensitivity QPSK REFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NTN satellite band # | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 | FDD |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |
|  | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |
| n256 | 30 |  | -96.6 | -94.6 | -93.5 | FDD |
|  | 60 |  | -97 | -94.9 | -93.7 |  |

Table 7.4.3.2.2-2: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NTN satellite band # | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
|  | 15 | 25 | 50 | 75 | 100 |  |
| n255 | 30 |  | 24 | 36 | 50 | FDD |
|  | 60 |  | 10 | 18 | 24 |  |
|  | 15 | 25 | 50 | 75 | 100 |  |
| n256 | 30 |  | 24 | 36 | 50 | FDD |
|  | 60 |  | 10 | 18 | 24 |  |