3GPP RAN WG4 Meeting #116		R4-2509566

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.1.2.1.3

Source:	Apple

Title:	PC1 for inter-band UL CA ever

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

Though PC1.5 inter-band UL CA has been introduced in Rel-18, it was limited to 2-band with 3Tx configuration only. In last RAN4 meeting, PC1.5 inter-band UL CA with 2Tx was first introduced [1] but without any specific band combination added since the corresponding Rel-19 non-spectrum WID [2] does not include any example UL combination. The TS 38.101-1 draft CR [1] revised the uplink inter-band CA power class table (Table 6.2A.1.3-1) by adding Class 1.5 columns and removing the Class 5 columns which have existed since Rel-15 but never been specified. The Class 1 columns which also have existed since Rel-15 and never been specified, were somehow preserved despite it was uncertain whether PC1 for inter-band UL CA would be introduced in 5G time frame. In this contribution, we share our view on whether it would be meaningful to introduce PC1 for inter-band UL CA.

# Discussion

The demand for better UL coverage has prompted the introduction of high-power UE (HPUE) since LTE [3] and the first release of NR specifications [4]. Power classes with maximum output power higher than that of power class 3 (PC3) at 23 dBm are all considered as HPUE which is applicable for both single carrier and UL combinations. To date, the following HPUE power classes have been specified for NR.

| Power Class | Power (dBm) | Single CC | UL Combination |
| --- | --- | --- | --- |
| PC2 | 26 | Yes | Yes |
| PC1.5 | 29 | Yes | Yes |
| PC1 | 31 | Yes | No |

Table 2-1 NR HPUE power classes

For single CC, except for PC1, the maximum output power has been defined as the sum of the power at all antenna connectors with the assumption that each Tx path is transmitting at the same power, that is, PC2 can be 26dBm with 1Tx or (23dBm + 23dBm) with 2Tx, and PC1.5 can be (26dBm + 26dBm) with 2Tx or (23dBm + 23dBm + 23dBm + 23dBm) with 4Tx. In recent Rel-20 5G-A UE RF objectives discussions, there has been proposal to introduce single CC PC1 with 4Tx for FWA based on (26dBm + 26dBm + 26dBm + 26dBm) [5]. However, such configuration can technically achieve maximum output power at 32dBm which is 1dB higher than PC1. If RAN4 would stick with PC1 without introducing a new power class, the PCMAX capability of the intended 4Tx HPUE would be underutilized.

Observation 1: For PC1 with 4Tx based on 4 x 26dBm, the PCMAX capability would be underutilized by 1 dB.

Similarly, for inter-band UL CA, if the intention would be to combine 29dBm in one band and 29dBm in another band, the maximum total output power would also be 1dB higher than PC1. Though inter-band UL CA can also be composed by (29dBm + 26dBm) = 30.8dBm which is close to PC1 but not exact, we double this was the intention to preserve PC1 for inter-band UL CA in the power class table in [1]. On the other hand, for inter-band UL CA with (29dBm + 26dBm) configuration, based on the current RAN4 definition, it is still considered as PC1.5 where the 30.8dB maximum total output power can be achieved by indicating higherPowerLimit-r17 capability.

Observation 2: For PC1 inter-band UL CA based on (29dBm + 29dBm), the PCMAX capability would also be underutilized by 1 dB.

Based on the above assessment, we think it is very unlikely PC1 for inter-band UL CA would ever be introduced in 5G. To avoid the confusion in the technical specifications, RAN4 may consider removing the PC1 columns from the uplink inter-band CA power class table via the Rel-19 draft CR for NR_ENDC_RF_Ph4-Core work item.

Observation 3: It is very unlikely PC1 for inter-band UL CA would ever be introduced in 5G.

Proposal: RAN4 to consider removing the PC1 columns from the uplink inter-band CA power class table via the Rel-19 draft CR for NR_ENDC_RF_Ph4-Core work item.

# 3	Conclusion

In this contribution, we share our view on whether it would be meaningful to introduce PC1 for inter-band UL CA.

Observation 1: For PC1 with 4Tx based on 4 x 26dBm, the PCMAX capability would be underutilized by 1 dB.

Observation 2: For PC1 inter-band UL CA based on (29dBm + 29dBm), the PCMAX capability would also be underutilized by 1 dB.

Observation 3: It is very unlikely PC1 for inter-band UL CA would ever be introduced in 5G.

Proposal: RAN4 to consider removing the PC1 columns from the uplink inter-band CA power class table via the Rel-19 draft CR for NR_ENDC_RF_Ph4-Core work item.

# 4	References

R4-2507915 “Draft CR for 38.101-1: PC1.5 for 2Tx and 3Tx inter-band uplink CA”, T-Mobile USA, Qualcomm, 3GPP TSG-RAN WG4 Meeting #115, Malta, May 19th – 25th, 2025

RP-241656 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, HiSilicon, 3GPP TSG RAN meeting #104, June 17th – 20th, 2024

3GPP TS 36.101 V19.2.0 (2025-06)

3GPP TS 38.101-1 V15.28.0 (2024-12)

RP-250811 “Moderator's summary for RAN4 led REL-20”, RAN4 Chair (Huawei), 3GPP TSG-RAN Meeting #107, Incheon, Korea, March 12th – 14th, 2025