3GPP TSG-RAN WG4 Meeting #116                	                R4-2509592

Bengaluru, IN, 25th - 29th Aug 2025

Title:	RRM Core requirements on event triggered L1 measurement reporting

Source:	China Telecom

Agenda item:			7.25.3.1

Document for:	Discussion

- Introduction

In the RAN4 #115 meeting, the measurements related enhancements were discussed and WF [1] has been approved. In this paper, we will continue to discuss the RRM Core requirements on event triggered L1 measurement reporting for LTM.

- Discussion

Issue 2-3: whether to introduce/update requirements for reporting criteria per measurement category

Candidate options: - Option 1: Define a separate capability for supported number of events triggered L1 measurement reporting. (CTC, Nokia, Xiaomi, CATT, vivo, MTK, ZTE, CMCC, E///) - Details: - Option 1a:(vivo) - RAN4 to specify the reporting capabilities for SSB-based L1 measurements for LTM by considering intra-frequency measurement and inter-frequency measurement. For CSI-RS based L1 measurements for LTM, consider 1 layer of measurement for the reporting capability. - Reporting criteria for LTM event triggered reporting is 4 per frequency layer. - The total number of reporting criteria for LTM event triggered reporting is 4*n, where n is the total number frequency layers for L1-RSRP measurement, including intra-frequency layer(s), inter-frequency layer(s) for SSB-based L1-RSRP measurement and 1 frequency layer for CSI-RS based L1-RSRP measurement if configured. - Option 1b: E/// - RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. - RAN4 to agree that Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS - Option 2: update the existing requirements to cover L1 event triggered reporting criteria per measurement category. (OPPO) - Option 3: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM. (Apple)

In legacy, requirements for reporting criteria per measurement category for L3 measurement and reporting had been specified in section 9.1.4. Similarly, for event triggered L1 measurement reporting, the requirements for reporting criteria per measurement category shall be defined too.

Both L3 measurement reporting and L1 measurement reporting could trigger LTM, no need for UE to be able to support different events of L3 and L1 in parallel per category. So we think a separate capability is better.

Proposal 1: Define a separate capability for supported number of events triggered L1 measurement reporting.

- Summary

In this paper, we provide our views on measurements related enhancements for LTM. From this discussion we have derived the following proposals:

Proposal 1: Define a separate capability for supported number of events triggered L1 measurement reporting.

- References

[1] R4-2508282, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple Inc.