3GPP TSG-RAN4 Meeting # 116	R4-2509633

# Bengaluru, India, 25th – 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft CR | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on active TCI state switch delay requirements for Low band CA via switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LBCA_Sw-Core |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The requirements for active TCI state switch delay for Low band CA via switching was not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The definition for Tfirst-SSB in active TCI state switch delay requirements were added below - time to first available SSB transmission on the corresponding cell as configured via LBCA-SwitchingPattern |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The active TCI state switch delay requirements for Low band CA via switching will be missed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10.3, 8.10.5, 8.15.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------- Start of Change <1> --------------

8.10.3	MAC-CE based TCI state switch delay

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE for indication of UE-specific PDCCH TCI state as defined in clause 6.1.3.15 of TS 38.321 [7]  in slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. The UE shall be able to receive PDCCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ .Where THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3];

-	Tfirst-SSB is

-	if the UE supports [supportedLowBandSwitching], time to first available SSB transmission on the corresponding cell as configured via LBCA-SwitchingPattern after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

-	otherwise, time to first SSB transmission after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

-	TSSB-proc = 2 ms;

-	TOk = 1 if target TCI state is not in the active TCI state list for PDSCH, 0 otherwise.

If the target TCI state is unknown, upon receiving PDSCH carrying MAC-CE for indication of UE-specific PDCCH TCI state as defined in clause 6.1.3.15 of TS 38.321 [7] in slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ +3Nslotsubframe,µ + TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc) / NR slot length. The UE shall be able to receive PDCCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ .

Where

-	T L1-RSRP = 0 in FR1 or when the TCI state switching not involving QCL-TypeD in FR2. Otherwise,

-	T L1-RSRP is the time for Rx beam refinement in FR2, defined as

-	TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1,

-	with the assumption of M=1

-	with TReport = 0

-	TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in clause 9.5.4.2

-	configured with higher layer parameter repetition set to ON

-	with the assumption of M=1 for periodic CSI-RS

-	for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

-	with TReport = 0

-	TOuk = 1 for CSI-RS based L1-RSRP measurement, and 0 for SSB based L1-RSRP measurement when TCI state switching involves QCL-TypeD

-	TOuk = 1 when TCI state switching involves other QCL types only

-	Tfirst-SSB is time to first SSB transmission after L1-RSRP measurement when TCI state switching involves QCL-TypeD;

-	Tfirst-SSB is

-	if the UE supports [supportedLowBandSwitching], time to first available SSB transmission on the corresponding cell as configured via LBCA-SwitchingPattern after MAC CE command is decoded by the UE for other QCL types; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

-	otherwise, time to first SSB transmission after MAC CE command is decoded by the UE for other QCL types;

The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

-------------- End of Change <1> --------------

-------------- Start of Change <2> --------------

8.10.5	RRC based TCI state switch delay

If the target TCI state is known, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ (TRRC_processing +TOk*(Tfirst-SSB + TSSB-proc)) / NR slot length, The UE is not required to receive PDCCH/PDSCH/CSI-RS or transmit PUCCH/PUSCH until the end of switching period.

Where

-	Slot n is the last slot overlapping with the PDSCH carrying RRC activation command.

-	TRRC_processing is the RRC processing delay defined in clause 11.2 of TS 36.331 [16] if the corresponding RRC message is embedded in E-UTRA RRC message, otherwise it is the RRC processing delay defined in clause 12 of TS 38.331 [2].

-	Tfirst-SSB is

-	if the UE supports [supportedLowBandSwitching], time to first available SSB transmission on the corresponding cell as configured via LBCA-SwitchingPattern after RRC processing by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

-	otherwise, time to first SSB transmission after RRC processing by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state.

-	TSSB-proc and TOk are defined in clause 8.10.3.

If the target TCI state is unknown, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ (TRRC_processing  +TL1-RSRP +TOuk*(Tfirst-SSB + TSSB-proc)) / NR slot length, The UE is not required to receive PDCCH/PDSCH/CSI-RS or transmit PUCCH/PUSCH until the end of switching period.

Where

-	Slot n is the last slot overlapping with the PDSCH carrying RRC activation command.

-	TRRC_processing is the RRC processing delay defined in clause 11.2 of TS 36.331 [16] if the corresponding RRC message is embedded in E-UTRA RRC message, otherwise it is the RRC processing delay defined in clause 12 of TS 38.331 [2].

-	Tfirst-SSB is time to first SSB transmission after L1-RSRP measurement when TCI state switching involves QCL-TypeD;

-	Tfirst-SSB is

-	if the UE supports [supportedLowBandSwitching], time to first available SSB transmission on the corresponding cell as configured via LBCA-SwitchingPattern after RRC processing time at the UE for other QCL types; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

-	otherwise, time to first SSB transmission after RRC processing time at the UE for other QCL types;

The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

-	TL1-RSRP, TOuk and TSSB-proc are defined in clause 8.10.3.

The requirements for RRC based TCI state switch delay apply when only 1 TCI state is configured in RRC TCI state list. When  THARQ> TRRC_processing a longer switching delay is allowed. Where THARQ is the time between DL data transmission and acknowledgement as specified in TS 38.213 [3].

-------------- End of Change <2> --------------

-------------- Start of Change <3> --------------

8.15.3	MAC-CE based downlink TCI state switch delay

The requirements in this clause shall apply for DL TCI state switch using separate DL TCI state or joint TCI state of unified TCI state switch framework.

In case that source RS in DL TCI state or joint TCI state is associated with a PCI different from that of the serving cell, the requirements in this clause shall apply if the cell with different PCI satisfies the known cell condition defined in clause 8.15.1. If the known cell condition is not met, longer delay may be expected.

In case of joint TCI state switch, if the target PL-RS is not maintained, UE is not expected to receive on DL based on the target TCI state before UE completes the DL and UL TCI state switch.

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. The UE shall be able to receive UE-dedicated PDCCH/PDSCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ where THARQ (in slot) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3];

-	Tfirst-SSB is

-	if the UE supports [supportedLowBandSwitching], time to first available SSB transmission on the corresponding cell as configured via LBCA-SwitchingPattern after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

-	otherwise, time to first SSB transmission after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

-	TSSB-proc = 2 ms;

-	TOk = 1 if target TCI state is not in the active TCI state list for PDSCH/PDCCH, 0 otherwise.

If the target TCI state is unknown, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ +3Nslotsubframe,µ + (TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc)) / NR slot length. The UE shall be able to receive UE-dedicated PDCCH/PDSCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ .

Where

-	T L1-RSRP = 0 in FR1 or when the TCI state switching not involving QCL-TypeD in FR2. Otherwise,

-	T L1-RSRP is the time for Rx beam refinement in FR2, defined as

-	TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1,

-	with the assumption of M=1

-	with TReport = 0

-	TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in clause 9.5.4.2

-	CSI-RS based L1-RSRP measurement only apply for TCI state switch when source RS is associated with serving cell

-	configured with higher layer parameter repetition set to ON

-	with the assumption of M=1 for periodic CSI-RS

-	for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

-	with TReport = 0

-	TOuk = 1 for CSI-RS based L1-RSRP measurement, and 0 for SSB based L1-RSRP measurement when TCI state switching involves QCL-TypeD

-	TOuk = 1 when TCI state switching involves other QCL types only

-	Tfirst-SSB is time to first SSB transmission after L1-RSRP measurement when TCI state switching involves QCL-TypeD;

-	Tfirst-SSB is

-	if the UE supports [supportedLowBandSwitching], time to first available SSB transmission on the corresponding cell as configured via LBCA-SwitchingPattern after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

-	otherwise, time to first SSB transmission after MAC CE command is decoded by the UE for other QCL types;

The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

-------------- End of Change <3> --------------