3GPP TSG-RAN WG4 Meeting #116	R4-2509638

# Bengaluru, India, 25th – 29th August, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for downlink coverage enhancement in NTN

Agenda item:	7.27.5.2

Document for:	Discussion

Introduction

In this contritubtion, we provide our views on downlink coverage enhancement for NR NTN RRM requirements based on the approved WF [1] and RAN2 agreements.

Discussion

Downlink Coverage Enhancement

RAN4 agreed that the configuration of multiple SMTCs with different periodicity is feasible and FFS whether and how to capture/chage RAN4 RRM requirements.

Agreement: From RAN4 perspective, it is feasible to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs. - In the reply LS (to be prepared by Xiaomi), the following information highlighted in yellow for both intra- and inter-frequency will be added for reference: - FFS on the exact RRM requirement change. - FFS whether to limit the number of SMTCs to no more than 4 is desirable in an earth-moving cell deployment Agreement: From RAN4 perspective, it is feasible to support SMTCs with up to 2 periodicities and different offsets but requires some clarifications/modifications to the existing RRM requirements. - RAN4 assumes that the configured SMTC periodicity is equal to or greater than the largest SSB periodicity among the cells associated with the relevant SMTC on the frequency layer. Agreement: ## The progress of RRM requirement definition for multiple SMTCs with up to two periodicities does not block the completion of RAN4 RRM core part of this WI.

Agreement in RAN2#130 - The maximum configured SMTCs per frequency for idle/inactive UEs is 6 (can come back if we find and issue) - To support SMTC enhancements for DL CE UE, introduce a new SMTC list instead of extending legacy smtc4list (can consider signaling optimizations for the new list to refer to the content of smtc4list to avoid signaling duplications) - We introduce a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. FFS if also an SSB-index based SMTC selection is supported - No enhancement for cell selection/reselection for DL CE is pursued (can re-discuss this if we will finally introduce a new barring scheme) - Working Assumption: We introduce a mechanism to assist the NW to configure the SMTCs in connected mode, according to one of the following 2 options: - Alt 1: UE provide the closest N reference locations/neighbor cells to network. - Alt 2: UE reports the selected SMTCs from configured SMTC set to network. - We continue in the next meeting on whether to confirm the WA (and go for either Alt1 or Alt2) or whether we don’t introduce any enhancement (i.e. not confirm the WA)

In the last RAN2 meeting, RAN2 agreed to introduce location-based SMTC selection procedure and considered SSB index based SMTC selection, and the detailed procedure will be discussed in RAN2. Based on the procedure, UE can select one or more SMTCs from configured SMTCs. Therefore, SMTC periodicity in RRM requirements should be based on the SMTC selected through the location-based SMTC selection procedure.

- Proposal 1: The definition of SMTC periodicity in RRM requirements for UE supporting DL CE should be based on the SMTC selected through the location-based SMTC selection procedure

RAN4 agreed that the configured SMTC periodicity is equal to or greater than the largest SSB periodicity among the cells associated with the relevant SMTC on the frequency layer, and the selection procedure of SMTC used for measurement is settled in RAN2. Therefore, existing RRM requiremetns could be reused with update to the SMTC periodicity definition.

- Proposal 2: Existing RRM requirements could be reused with revising the SMTC periodicity definition for UE supporting DL CE.

Conclusion

In this contribution, we provide views on downlink coverage enhancement for NR NTN, and we propose

- Proposal 1: The definition of SMTC periodicity in RRM requirements for UE supporting DL CE should be based on the SMTC selected through the location-based SMTC selection procedure

- Proposal 2: Existing RRM requirements could be reused with revising the SMTC periodicity definition for UE supporting DL CE.

Reference

[1] R4-2508288, “WF on RRM requirements for NR_NTN_Ph3_Part2,” Moderator (Qualcomm Incorporated)