3GPP TSG-RAN WG4 Meeting #116	R4-2509657

Bengaluru, India, 25th August 2025 – 29th August 2025

Agenda item:	7.20.6

Source:	MediaTek Inc.

Title:	Discussion on NR MIMO Phase 5 demodulation and CSI requirements

Document for:	Discussion

# Introduction

In the previous RAN4#115 meeting NR MIMO Phase 5 was discussed and WF with several open issues was introduced for further work [1]. In this document we discuss remaining open issues with several observations and proposals. We also have separate simulation results document [2].

# Discussion

In work item description of NR MIMO Phase 5 there is the following RAN1 objective for CSI support for up to 128 CSI-RS antenna ports.

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

In work item description of NR MIMO Phase 5 there is the following RAN4 performance objective.

- Specify necessary performance requirements for the enhancements listed above.

In updated RAN1 UE features list for Rel-19 NR [3] there are several UE capabilities for CSI support for up to 128 CSI RS antenna ports. Note, support for each number of antenna ports has own dedicated capability.

UE capabilities of extended codebooks for CSI support for up to 128 CSI RS antenna ports - Enhanced Type-I SP codebook for [48, 64, 128] ports – Scheme-A - Enhanced Type-I SP codebook for [48, 64, 128] ports – Scheme-B - Enhanced Type-I MP codebook for [48, 64, 128] ports - Extended Rel-16 eType-II codebook for [48, 64, 128] Tx ports - Extended Rel-17 FeType-II codebook with [48, 64] Tx ports - Extended Rel-18 eType-II Doppler codebook for [48, 64, 128] Tx ports

In previous releases RAN4 has specified requirements for Type-I SP codebook, Rel-16 eType-II codebook and Rel-18 eType-II Doppler codebook. From these codebooks Type-I SP codebook is the most deployed codebook in live networks by far. Correspondingly, in previous releases RAN4 has not specified requirements for Type-I MP codebook and Rel-17 FeType-II codebook. Enhanced Scheme-A Type-I SP codebook, for Ranks 1 to 4, is straightforward extension from legacy Rel-15 Type-I SP codebook. Therefore, we consider this as the most potential use case. We see that defining only PMI requirements for new Rel-19 CSI codebooks is sufficient.

Observation #1: In previous releases RAN4 has specified requirements for Type-I SP codebook, Rel-16 eType-II codebook and Rel-18 eType-II Doppler codebook.

Observation #2: Type-I SP codebook is the most deployed codebook in live networks.

Observation #3: In previous releases RAN4 has not specified requirements for Type-I MP codebook and Rel-17 FeType-II codebook.

Observation #4: Enhanced Scheme-A Type-I SP codebook, for Ranks 1 to 4, is straightforward extension from legacy Rel-15 Type-I SP codebook.

New enhanced and extended codebooks introduce variants with different number of antenna ports. Available options are 48, 64, and 128 antenna ports, except FeType-II codebook does not support 128 antenna ports.

Table 1: RAN1 UE feature list of Enhanced Type-I SP codebook Scheme A

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-2-1-1 | Enhanced Type-I SP codebook for 64 ports – Scheme-A | Support of enhanced Type-I SP codebook for Scheme-A with 64 Tx ports by aggregating multiple NZP CSI-RS resources | 2-35 |
| 59. NR_MIMO_Ph5 | 59-2-1-1a | Enhanced Type-I SP codebook for 48 ports – Scheme-A | Support of enhanced Type-I SP codebook for Scheme-A with 48 Tx ports by aggregating multiple NZP CSI-RS resources | 59-2-1-1 |
| 59. NR_MIMO_Ph5 | 59-2-1-1b | Enhanced Type-I SP codebook for 128 ports – Scheme-A | Support of enhanced Type-I SP codebook for Scheme-A with 128 Tx ports by aggregating multiple NZP CSI-RS resources | 59-2-1-1 |

Table 1 lists the main parts of RAN1 UE feature list of Enhanced Type-I SP codebook Scheme A. Here we can see that the baseline capability for Enhanced Type-I SP codebook is with 64 antenna ports. In addition, Enhanced Type-I SP codebook can support 48 and/or 128 antenna ports. This similar principle applies also to other Rel-19 Enhanced/Extended codebooks with 48 or more antenna ports.

Observation #5: The baseline capability for Rel-19 Enhanced/Extended codebooks is with 64 antenna ports.

## 2.1 General scope

In this Section we discuss open issues of general scope.

Issue 1-1-7: Clarify if PMI reporting requirements are needed for eTypeII-r19 Way forward: - Option 1: RAN4 introduce PMI reporting requirements for eTypeII-r19 - Option 2: RAN4 do not introduce PMI requirements for eTypeII-r19

We observe that the market is dominated by Type-I SP codebook solutions, and the potential of new eTypeII solutions is limited. Therefore, we see little benefit in introducing new requirements for the eTypeII-r19 codebook.

Proposal #1: We propose to not consider eTypeII-r19 codebook for PMI requirements.

Issue 1-1-8: Clarify if PMI reporting requirements are needed for typeII-Doppler-r19 Way forward: - Option 1: RAN4 introduce PMI reporting requirements for typeII-Doppler-r19 - Option 2: RAN4 do not introduce PMI requirements for typeII-Doppler-r19

We observe that the market is dominated by Type-I SP codebook solutions, and the potential of new eTypeII-based codebook solutions is limited. Therefore, we see little benefit in introducing new requirements for the typeII-Doppler-r19 codebook.

Proposal #2: We propose to not consider typeII-Doppler-r19 codebook for PMI requirements.

Issue 1-1-9: Clarify if UE demodulation/CSI reporting requirements are needed for SRS port grouping Way forward: - Option 1: RAN4 introduce PDSCH demodulation requirements for SRS port grouping - Option 1A: RAN4 to discuss reuse of existing demod requirements for SRS port grouping scheme. - Option 1B: RAN4 to discuss the new performance requirements for R19 8Rx UE supporting SRS port grouping, FFS the test setup. - Option 2: RAN4 do not introduce UE demodulation/CSI reporting requirements for SRS port grouping

In the previous meeting, it was noted that there is no precedent in RAN4 for specifying UE demodulation requirements for SRS-based precoders. Furthermore, it was proposed to potentially reuse existing UE demodulation requirements for the new SRS port grouping scheme to avoid additional simulation work.

Observation #6: There is no precedent in RAN4 to have UE demodulation requirements for SRS based precoders.

We observe that if RAN4 were to introduce new requirements for SRS-based precoders, it should first define the behaviour of test equipment. Additionally, with agreed-upon precoding processing, real performance simulations should be conducted. Typically, RAN4’s efforts to define test equipment precoder processing, such as in the Rel-17 FeType-II codebook, have faced challenges in reaching consensus. Therefore, we have a pessimistic view regarding the introduction of UE demodulation requirements for SRS-based precoders.

Proposal #3: We propose to not consider UE demodulation/CSI reporting requirements for SRS port grouping.

Issue 1-1-10: Clarify if UE demodulation requirements are needed for UE reporting enhancement for CJT Way forward: - Option 1: RAN4 do not introduce demodulation requirements for reporting enhancements for CJT deployments - Option 2: RAN4 further discussion on whether to introduce new PDSCH demodulation performance requirements for PDSCH transmission Scheme C/Scheme D/Scheme E. Issue 1-1-11: Clarify if CSI reporting requirements are needed for UE reporting enhancements for CJT Way forward: - Option 1: RAN4 do not introduce CSI reporting requirements for UE reporting enhancements for CJT - Option 2: RAN4 introduce new requirements for CJT calibration offset reporting for the time offset (TO), the carrier frequency offset (CFO) and the phase offset (PO) scenarios. Further study exact implementation of the new requirements. - RAN4 do not introduce new CQI/RI and dedicated PMI reporting requirements but consider PMI calculation as part of the CJT calibration reporting requirements. - Introduce a new subsection in TS 138 101-4 to cover introduction of CJT calibration offset reporting requirements.

In the previous meeting, it was proposed to further study a test scenario with two TRPs using a non-ideal or non-synchronized backhaul for Coherent Joint Transmission (CJT). In this scenario, the UE would report various channel properties, such as time offset, frequency offset, and phase offset. Furthermore, these reported offsets would be pre-compensated at the transmitter, and the corresponding performance could then be measured and compared to non-compensated transmission.

We see that implementing such a test scenario would be unnecessarily complex. We consider it sufficient to directly measure the reporting accuracy of the listed channel properties, as is done in the RRM requirements. Therefore, we propose not to introduce UE demodulation or CSI reporting requirements for UE reporting enhancement for CJT.

Proposal #4: We propose to not consider UE demodulation requirements for UE reporting enhancement for CJT.

Proposal #5: We propose to not consider CSI reporting requirements for UE reporting enhancements for CJT.

## 2.2 Test assumptions for typeI-SinglePanel-r19

In this Section we discuss open issues of test assumptions for typeI-SinglePanel-r19.

Issue 1-2-7: Number of CSI-RS ports Agreement: - Agree on to define requirement for 64 ports. Not to define requirements for 48 ports. FFS on 128 ports.

We see it is sufficient to define requirements with only a minimum capability of 64 ports.

Proposal #6: We propose to not consider 128 ports for typeI-SinglePanel-r19 requirements.

Issue 1-2-8: N1 and N2 configuration for 64 CSI-RS ports Way forward: - Select single one from below options - Option 1: (16,2) - Option 2: (8,4)

In our simulations, we observe lower and more feasible gamma values with the Option 2 (8, 4) configuration. Therefore, we believe that this option would result in a more robust test configuration.

Proposal #7: We propose use N1 and N2 configuration for 64 CSI-RS ports as (8,4) for typeI-SinglePanel-r19 requirements.

Issue 1-2-13: portMappingMethod Agreement: - Select one for requirements. FFS on which one next meeting.

We believe there is no performance difference between method1 and method2. We must select one, and we propose to choose method1, although we do not have a strong preference for either.

Proposal #8: We propose to use portMappingMethod as method1 for typeI-SinglePanel-r19 requirements.

Issue 1-2-14: Number of slots for CSI-RS transmission Agreement: - One slot for 64 ports. FFS for 128 ports.

We see that using more than one slot for CSI-RS transmission overhead would be impractical. Therefore, we propose to use only one slot for 128 ports in the typeI-SinglePanel-r19 requirements if such requirements were to be introduced.

Proposal #9: We propose to use one slot also for 128 ports for typeI-SinglePanel-r19 requirements, if introduced.

Issue 1-2-15: CQI/RI/PMI delay Way forward: - Option 1: RAN4 set the CQI/RI/PMI delay according to the number of CSI-RS ports and CSI processing capabilities. - Option 2: Re-discuss the CSI-RS scheduling and CSI reporting pattern to match the capability 2 of timeline to support up to 128 CSI-RS ports

Proposal #10: We propose to define CSI-RS scheduling and CSI reporting pattern to match the capability 2 of timeline to support up to 128 CSI-RS ports for typeI-SinglePanel-r19 requirements.

Issue 1-2-16: MCS level Agreement: - MCS20 (Table 1) as the starting point

In our simulations, we observe lower and more feasible gamma values with the MCS22 configuration. Additionally, the SNR operating point would be more feasible with MCS22. Therefore, we believe that MCS22 would result in a more robust test configuration.

Proposal #11: We propose to use MCS22 (Table 1) for typeI-SinglePanel-r19 requirements with 64 ports.

## 2.3 Test assumptions for eTypeII-r19 (if introduced)

In this Section we discuss open issues of test assumptions eTypeII-r19. All proposals are conditional to agreement if any eTypeII-r19 requirements will be introduced.

Issue 1-3-1: Test metric Way forward: - Option 1: reuse the test metric for eType-II PMI reporting requirements, throughput ratio between the following PMI and random Rel-19 refined Type-I PMI Mode A. - Other options are not precluded

Proposal #12: We propose to reuse the usual test metric for eType-II reporting requirements for eTypeII-r19, if introduced.

Issue 1-3-2: Duplex mode Way forward: - Option 1: Both FDD and TDD - Other options are not precluded

Proposal #13: We propose to have both FDD and TDD duplex modes for eType-II reporting requirements for eTypeII-r19, if introduced.

Issue 1-3-3: Number of Rx antennas Way forward: - Option 1: Both 2Rx and 4Rx - Other options are not precluded

Proposal #14: We propose to have both 2Rx and 4Rx for eType-II reporting requirements for eTypeII-r19, if introduced.

Issue 1-3-4: Propagation channel Way forward: - Option 1: TDLA30-5 - Other options are not precluded

Proposal #15: We propose to use TDLA30-5 propagation channel for eType-II reporting requirements for eTypeII-r19, if introduced.

Issue 1-3-5: Correlation configuration Way forward: - Option 1: XP-Medium - Other options are not precluded

Proposal #16: We propose to use XP-Medium correlation for eType-II reporting requirements for eTypeII-r19, if introduced.

Issue 1-3-6: typeII-ri-restriction-r19 Way forward: - Option 1: Rank2 - Other options are not precluded

Proposal #17: We propose to use Rank2 for eType-II reporting requirements for eTypeII-r19, if introduced.

Issue 1-3-7: Number of CSI-RS ports Way forward: - Option 1: 64 ports - Other options are not precluded

Proposal #18: We propose to use 64 ports for eType-II reporting requirements for eTypeII-r19, if introduced.

Issue 1-3-8: N1 and N2 configuration for 64 CSI-RS ports Way forward: - Option 1: (8,4) - Other options are not precluded

Proposal #19: We propose to use N1 and N2 configuration of (8,4) for eType-II reporting requirements for eTypeII-r19 for 64 ports, if introduced.

Issue 1-3-9: (Number of aggregated NZP CSI-RS resources) K for 64 CSI-RS ports Way forward: - Option 1: K=2 - Other options are not precluded

Proposal #20: We propose to K=2 for eType-II reporting requirements for eTypeII-r19 for 64 ports, if introduced.

Issue 1-3-10: MCS level Way forward: - Option 1: MCS20, and RAN4 may consider higher MCS (code rate) for PDSCH FRC, if the required SNR value is too low to achieve 90% of the peak rate with the follow PMI. - Other options are not precluded

Proposal #21: We propose to use MCS22 for eType-II reporting requirements for eTypeII-r19 for 64 ports, if introduced.

Issue 1-3-11: numberOfPMI-SubbandsPerCQI-subband-r19 Way forward: - Option 1: set as 1 - Other options are not precluded

We see it is sufficient to define requirements with only a minimum capability of numberOfPMI-SubbandsPerCQI-subband-r19 set as 1.

Proposal #22: We propose to use numberOfPMI-SubbandsPerCQI-subband-r19 as 1 for eType-II reporting requirements for eTypeII-r19, if introduced.

Issue 1-3-12: paramCombination-r19 Way forward: - Option 1: set paramCombination-r19 as 6 (L =4, pν =1/2, β=1/2) as the starting point. - Other options are not precluded

Proposal #23: We propose to use paramCombination-r19 as 6 for eType-II reporting requirements for eTypeII-r19, if introduced.

Issue 1-3-13: Other parameters Way forward: - Option 1: Reuse the test parameters of the existing Rel-16 eType-II PMI reporting requirements with 16 CSI-RS ports as baseline. - Other options are not precluded

Proposal #24: We propose to reuse the test parameters of the existing Rel-16 eType-II PMI reporting requirements with 16 CSI-RS ports as baseline for eType-II reporting requirements for eTypeII-r19, if introduced.

# Conclusion

In this contribution we provide the view on the NR MIMO Phase 5 demodulation and CSI requirements issues. The following observations and proposals are made:

Observation #1: In previous releases RAN4 has specified requirements for Type-I SP codebook, Rel-16 eType-II codebook and Rel-18 eType-II Doppler codebook.

Observation #2: Type-I SP codebook is the most deployed codebook in live networks.

Observation #3: In previous releases RAN4 has not specified requirements for Type-I MP codebook and Rel-17 FeType-II codebook.

Observation #4: Enhanced Scheme-A Type-I SP codebook, for Ranks 1 to 4, is straightforward extension from legacy Rel-15 Type-I SP codebook.

Observation #5: The baseline capability for Rel-19 Enhanced/Extended codebooks is with 64 antenna ports.

Proposal #1: We propose to not consider eTypeII-r19 codebook for PMI requirements.

Proposal #2: We propose to not consider typeII-Doppler-r19 codebook for PMI requirements.

Observation #6: There is no precedent in RAN4 to have UE demodulation requirements for SRS based precoders.

Proposal #3: We propose to not consider UE demodulation/CSI reporting requirements for SRS port grouping.

Proposal #4: We propose to not consider UE demodulation requirements for UE reporting enhancement for CJT.

Proposal #5: We propose to not consider CSI reporting requirements for UE reporting enhancements for CJT.

Proposal #6: We propose to not consider 128 ports for typeI-SinglePanel-r19 requirements.

Proposal #7: We propose use N1 and N2 configuration for 64 CSI-RS ports as (8,4) for typeI-SinglePanel-r19 requirements.

Proposal #8: We propose use portMappingMethod as method1 for typeI-SinglePanel-r19 requirements.

Proposal #9: We propose use one slot also for 128 ports for typeI-SinglePanel-r19 requirements, if introduced.

Proposal #10: We propose to define CSI-RS scheduling and CSI reporting pattern to match the capability 2 of timeline to support up to 128 CSI-RS ports for typeI-SinglePanel-r19 requirements.

Proposal #11: We propose to use MCS22 (Table 1) for typeI-SinglePanel-r19 requirements with 64 ports.

Proposal #12: We propose to reuse the usual test metric for eType-II reporting requirements for eTypeII-r19, if introduced.

Proposal #13: We propose to have both FDD and TDD duplex modes for eType-II reporting requirements for eTypeII-r19, if introduced.

Proposal #14: We propose to have both 2Rx and 4Rx for eType-II reporting requirements for eTypeII-r19, if introduced.

Proposal #15: We propose to use TDLA30-5 propagation channel for eType-II reporting requirements for eTypeII-r19, if introduced.

Proposal #16: We propose to use XP-Medium correlation for eType-II reporting requirements for eTypeII-r19, if introduced.

Proposal #17: We propose to use Rank2 for eType-II reporting requirements for eTypeII-r19, if introduced.

Proposal #18: We propose to use 64 ports for eType-II reporting requirements for eTypeII-r19, if introduced.

Proposal #19: We propose to use N1 and N2 configuration of (8,4) for eType-II reporting requirements for eTypeII-r19 for 64 ports, if introduced.

Proposal #20: We propose to K=2 for eType-II reporting requirements for eTypeII-r19 for 64 ports, if introduced.

Proposal #21: We propose to use MCS22 for eType-II reporting requirements for eTypeII-r19 for 64 ports, if introduced.

Proposal #22: We propose to use numberOfPMI-SubbandsPerCQI-subband-r19 as 1 for eType-II reporting requirements for eTypeII-r19, if introduced.

Proposal #23: We propose to use paramCombination-r19 as 6 for eType-II reporting requirements for eTypeII-r19, if introduced.

Proposal #24: We propose to reuse the test parameters of the existing Rel-16 eType-II PMI reporting requirements with 16 CSI-RS ports as baseline for eType-II reporting requirements for eTypeII-r19, if introduced.

# Reference

- R4-2508627, Way Forward for [115][329] NR_MIMO_demod, Samsung

- R4-2509658, Simulation results of NR MIMO Phase 5 demodulation and CSI requirements, MediaTek

- R1-2502977, Updated RAN1 UE features list for Rel-19 NR after RAN1 #120bis, AT&T, NTT DOCOMO, inc.