3GPP TSG-RAN WG4 Meeting #116	                              R4-2509680

Bengaluru, India, August 25th – 29th, 2025

Title: 	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Source: 	OPPO

Agenda item:	7.14.3.1

Document for:	Approval

Introduction

In last meeting, FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor was discussed and the agreements were captured in the approved WF[1].

In this contribution, we will provide our views on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor.

Discussion

- Issue 1-1-1: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” Agreement: For the quitting condition of FBS: - Introduce a new UAI indicating “overheating” issue: - RAN4 will not discuss the UE behaviour after reporting UAI - Introduce a timer-based solution: - UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition - Further discuss the candidate numbers for T1. Way forward: For re-evaluation mechanism for scenario 1/2/3/4: - Check whether the following mechanism is agreeable: - UE re-evaluates the activation condition after the timer expires, where the timer starts after UE quits from Multi-Rx mode.

In last meeting, it was agreed to introduce timer based solution for quit condition of FBS, i.e., UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition and the candidate values of T1 is FFS. According to the requirements in section 9, the FR2 measurement period based on FBS can be several seconds and its multiples. And in legacy, there is a timer for UE overheating assistance information reporting which can be used as baseline.

OverheatingAssistanceConfig ::= SEQUENCE { overheatingIndicationProhibitTimer ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30, s60, s90, s120, s300, s600, spare3, spare2, spare1} overheatingIndicationProhibitTimer Prohibit timer for overheating assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

Considering the measurement delay and the UE power saving, the candidate value of T1 can be several seconds to hundreds of seconds, e.g., select 5s-300s from the legacy timer overheatingIndicationProhibitTimer.

For the re-evaluation mechanism, we think the timer-based solution can also be used, i.e., the timer starts after UE quits from Multi-Rx mode and UE re-evaluates the activation condition after the timer expires.

Proposal 1: The candidate value of T1 can be several seconds to hundreds of seconds e.g., select 5s-300s from the legacy timer overheatingIndicationProhibitTimer.

Proposal 2: Introduce another timer for re-evaluation mechanism, i.e., UE re-evaluates the activation condition after the timer expires where the timer starts when UE quits from FBS mode.

- Issue 1-1-3: UE capability for Rx BSF optimization Agreement: For FBS capability design: - UE capability is per-band and optional - TDD only - FR2-1 only - FFS: in the same FG, whether to define separate UE capability for (intra-frequency) and (intra-frequency + inter-frequency)

Regarding the capability for BSF, it was agreed in last meeting that the capability is per band and optional. The feature group was also discussed and the main controversial part is whether to define separate capability for inter-frequency measurement. So far, we don’t see the necessity to differentiate the intra-frequency and inter-frequency for a specific band. We think when UE reports support of BSF on a certain band, it should support BSF for all the carriers on this band since the implementation is similar. One issue for inter-frequency BSF provided in last meeting is the buffer. But we understand when UE supports Multi-Rx, the baseband capability is also improved which is matched with the corresponding RF.

Proposal 3: For FBS capability design, do not define separate capability for (intra-frequency) and (intra-frequency + inter-frequency).

Conclusions

In this paper, we provided our views on FR2-1 SSB based L3 measurement delay reduction for optimizing BSF. And the following proposals are provided.

Proposal 1: The candidate value of T1 can be several seconds to hundreds of seconds e.g., select 5s-300s from the legacy timer overheatingIndicationProhibitTimer.

Proposal 2: Introduce another timer for re-evaluation mechanism, i.e., UE re-evaluates the activation condition after the timer expires where the timer starts when UE quits from FBS mode.

Proposal 3: For FBS capability design, do not define separate capability for (intra-frequency) and (intra-frequency + inter-frequency).

Reference

- R4-2508266 WF on RRM requirements for NR_RRM_Ph5_Part2, CATT

- R4-2505562 Topic summary for [115][213] NR_RRM_Ph5_Part2, CATT