3GPP TSG-RAN WG4 Meeting #116	                              R4-2509685

Bengaluru, India, August 25th – 29th, 2025

Title: 	Discussion on RRM requirements for low band carrier aggregation via switching

Source: 	OPPO

Agenda item:	7.3.4

Document for:	Approval

Introduction

In RAN#106 meeting, a new WID [1] on low band carrier aggregation via switching was agreed with the following objectives:

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

In last meeting, RAN4 discussed the related RRM requirements and the conclusions were captured in the approved WF [2]. In this document, we will further discuss the RRM requirements based on the open issues in [3].

Discussion

Issue 1-3-1: Necessity of activated SDL SCell L3 measurement and corresponding interruption requirement Online Agreement on Tuesday: - RAN4 to define the requirement for activated SDL SCell L3 measurement. Issue 1-3-2: Requirement design of activated SDL SCell L3 measurement Online Agreement on Tuesday: - For activated SDL SCell measurement, UE performs the SDL SCell measurement on the overlapped occasions of SMTC window and SDL SCell reception duration indicated by switching pattern. FFS: - If no SSB occasion of the serving cell is contained within the LB-CA switching pattern, the requirements do not apply (from Nokia proposal) - FFS on details of requirement design.

For activated SDL SCell measurement, it was agreed in last meeting that the UE performs the SDL SCell measurement on the overlapped occasions of SMTC window and SDL SCell reception duration indicated by switching pattern. But the L3 measurement on PCell was not clarified and we think the same conclusion can be used.

Proposal 1: The PCell measurement is performed on the overlapped occasions of SMTC window and PCell reception duration indicated by switching pattern.

Agreement: - Not to change definition of intra-frequency/inter-frequency measurement with/without gap measurement - FFS on the use case for MG application.

In previous agreement, it was also agreed that the definition of intra-frequency/inter-frequency measurement with/without gap is not changed. Based on above agreements, we think some additional clarifications are needed for intra-frequency and inter-frequency measurement.

Firstly, we think the measurement restriction in proposal 1 also applies to the neighbor cells which are on the same frequency of the PCell or SDL SCell i.e., intra-frequency measurement, but only apply to intra-frequency measurements without gap. If gap is configured for the measurement on SDL SCell, such limitation is not needed. Otherwise, the gap will be wasted and serving and neighbor cell measurements on the same frequency will be limited.

Proposal 2: The measurement restriction (i.e., measure on the overlapped occasions of SMTC window and PCell/SDL SCell reception duration indicated by switching pattern) only apply to intra-frequency measurement without gap.

The second one is gap application. According to the definition of intra-frequency/inter-frequency measurement, both carriers of PCell and SDL SCell are intra-frequency but UE is indicated working on one cell, then according to the agreement above, the measurement on the other cell will need switching or interruption or gaps. In this work item, we think switching is used and no interruption or gaps are needed.

But there is one special case that no SSB occasion of the serving cell or neighbor cell is overlapped with the cell reception duration according to the LB-CA switching pattern. In such case, there are two options for UE behavior:

- Option 1: UE is not required to perform the measurement and no requirements apply.

- Option 2: UE to perform the measurement based on gaps and intra-frequency measurement requirements with gap apply.

But considering this is not common case, we think option 1 can be used in this release.

Proposal 3: When no SSB occasion of PCell/SDL SCell/neighbor cell is overlapped with reception duration indicated by switching pattern, UE is not required to perform the measurement and no requirements apply.

On the other hand, the detailed requirements design for intra-frequency measurement is not clear, e.g., how to handle the skipped SMTC occasion due to switching.

RAN1 has defined the semi-static switching pattern as a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2.

RAN1#120bis meeting: Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the time unit (resolution) of the semi-static switching pattern configuration is slot (1ms for 15kHz SCS). Agreement - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2. - The pattern starts from the beginning of SFN 0 of PCell. - Restriction of maximum X switch(es) within Y slot(s). - FFS: detailed value and relation of X and Y. - FFS: other restriction. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms.

Based on the RAN1 design on the switching pattern, the cell configuration is flexible and it is possible that the SMTC windows overlapped with SDL SCell reception duration are not uniform. So comparing to the legacy, the SDL SCell measurement requirements cannot be defined by simply multiplying a scaling factor. And we think there are two approaches can be considered:

- Approach 1: Clarify in the requirements that only SMTC occasions overlapped with switching pattern need to be considered and do not define the extension of measurement period, e.g.,

- Table 9.2.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1) DRX cycle; TSSB_measurement_period_intra. No DRX; max(200 ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320 ms; max(200 ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320 ms; ceil( 5 x Kp ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2：When UE supports [LB CA via switching], only the SMTC occasions overlapped with the corresponding cell (PCell or SDL SCell) reception duration indicated by switching pattern are considered..

- Approach 2: The approach in CCA related requirements can be reused, i.e., the requirements can be defined by adding the number of SMTC occasions which are skipped. E.g.,

- Table 9.2.5.2-14: Measurement period for intra-frequency measurements without gaps (FR1) for UE supporting [LB CA via switching] DRX cycle; TSSB_measurement_period_intra. No DRX; max(200 ms, ceil((5+ Lmeas,LBCA) x Kp) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320 ms; max(200 ms, ceil(1.5x (5+ Lmeas,LBCA) x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320 ms; ceil((5+ Lmeas,LBCA) x Kp ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When DRX is not configured, Lmeas,LBCA is the number of SMTC occasions not available at the UE during TSSB_measurement_period_intra for measurement, where Lmeas,LBCA <Lmeas,max,LBCA. When DRX is configured, Lmeas,LBCA is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TSSB_measurement_period_intra for measurement, where Lmeas,LBCA < Lmeas,max,LBCA. When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. FFS: The UE is not required to determine the availability of SMTC occasions more frequent than what is required by CSSFintra. NOTE 3: Lmeas,max,LBCA is FFS. NOTE 4: Upon exceeding Lmeas,max,LBCA over the period of time TSSB_measurement_period_intra, the UE has to restart the measurement procedure..

- Approach 3: Extend the measurement period requirements by a scaling factor KLBCA which is similar as Kp, e.g.,

When UE supports [LB CA via switching], KLBCA is the scaling factor for an SSB frequency layer to be measured without GAP. KLBCA = Ntotal_LBCA / Navailable_LBCA, where Navailable_LBCA and Ntotal_LBCA are calculated as follows: - For a window W of duration max(SMTC period, xRP_max, P), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion, and P is the switching periodicity: - Ntotal_LBCA is the total number of SMTC occasions within the window, including those overlapped with GAP and MUSIM gap occasions within the window, and - Navailable_LBCA is the number of SMTC occasions that are overlapped with the corresponding cell reception duration indicated by switching pattern and not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions within the window W, after accounting for measurement GAP and MUSIM gap collisions by applying the collision rules for GAP and MUSIM gap in clauses 9.1.8.3, 9.1.10.4, 9.1.10.5, 9.1.12.3, and 9.1.13.3, respectively. - xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap. Requirements in this clause do not apply when Navailable_LBCA = 0 due to fully overlapping between SMTC occasions and MUSIM gap occasions within the window W or fully non-overlapping between SMTC occasions and cell reception duration indicated by switching pattern.

The DraftCR in our companion paper is based on approach 1.

For inter-frequency and inter-RAT measurement, we understand the existing measurement requirements with and without gap can be reused. For SFTD and measurement with NCSG, we can further discuss whether to apply to UE supporting low band CA via switching.

Proposal 4: Consider the following three approaches to define activated SDL SCell and intra-frequency measurement requirements:

- Approach 1: Clarify in the requirements that only SMTC occasions overlapped with switching pattern need to be considered and do not define the extension of measurement period.

- Approach 2: The approach in CCA related requirements can be reused, i.e., the requirements can be defined by adding the number of SMTC occasions which are skipped.

- Approach 3: Extend the measurement period requirements by a scaling factor KLBCA which is similar as Kp.

Proposal 5: Reuse the existing measurement requirements with/without gap for inter-frequency measurement and inter-RAT measurement.

Issue 1-3-4: Requirement design of deactivated SDL SCell measurement and corresponding interruption Agreement on Thursday: - The legacy deactivated SCell measurement delay requirements are applied for UE to perform deactivated SDL SCell measurement. Agreement on Thursday: - For deactivated SDL SCell measurement, interruptions on active serving cell(s) are allowed with up to X% probability of missed ACK/NACK. FFS: X, and X can be same or different for different SDL SCC measurement cycle (i.e., configured measCycleSCell).

For the interruption due to deactivated SDL SCell measurement, considering the switching between PCell and SDL SCell, each measurement will cause interruption which may be not same as legacy. We think the interruption can be defined based on the duration of SMTC and the RF retuning time, i.e., interruptions on active serving cell(s) are allowed with up to X% probability of missed ACK/NACK where X is interruption length/measurement cycle.

Proposal 6: The interruption during measurement on deactivated SDL SCell is defined as up to X% probability of missed ACK/NACK, where X is interruption length/measurement cycle.

- Interruption length includes SMTC duration and RF retuning time.

Conclusions

In this paper, we give our views on RRM requirements for low band carrier aggregation via switching. And the following proposals are provided.

Proposal 1: The PCell measurement is performed on the overlapped occasions of SMTC window and PCell reception duration indicated by switching pattern.

Proposal 2: The measurement restriction (i.e., measure on the overlapped occasions of SMTC window and PCell/SDL SCell reception duration indicated by switching pattern) only apply to intra-frequency measurement without gap.

Proposal 3: When no SSB occasion of PCell/SDL SCell/neighbor cell is overlapped with reception duration indicated by switching pattern, UE is not required to perform the measurement and no requirements apply.

Proposal 4: Consider the following three approaches to define activated SDL SCell and intra-frequency measurement requirements:

- Approach 1: Clarify in the requirements that only SMTC occasions overlapped with switching pattern need to be considered and do not define the extension of measurement period.

- Approach 2: The approach in CCA related requirements can be reused, i.e., the requirements can be defined by adding the number of SMTC occasions which are skipped.

- Approach 3: Extend the measurement period requirements by a scaling factor KLBCA which is similar as Kp.

Proposal 5: Reuse the existing measurement requirements with/without gap for inter-frequency measurement and inter-RAT measurement.

Proposal 6: The interruption during measurement on deactivated SDL SCell is defined as up to X% probability of missed ACK/NACK, where X is interruption length/measurement cycle.

- Interruption length includes SMTC duration and RF retuning time.

Reference

- RP-243317 New WID on low band carrier aggregation via switching

- R4-2508259 WF on RRM requirements for NR_LBCA_Sw, Apple

- R4-2505556 Topic summary for [115][207] NR_LBCA_Sw, Apple