# 3GPP TSG-RAN WG4 Meeting #116	R4-2509693

# Bengaluru, India, August 25th – 29th, 2025

Title: 	On RRM requirements of on-demand SSB SCell operation

Source: 	OPPO

Agenda item:	7.23.3.1

Document for:	Discussion

Introduction

In last meeting, a WF was approved for OD-SSB based requirements [1].

In this contribution, we will provide discussion on remaining issues of RRM requirements for On-demand SSB SCell operation.

Discussion

RAN4 has agreed to define requirements for OD-SSB based deactivated Scell measurement and Scell activation. In this paper, we mainly focus on the discussion on OD-SSB based deactivated SCell L3 measurement (Case 1). Most proposals about the fast mode windows below can also apply for case 2.

Open issues for Case 1

Sub-issue 2: OD-SSB in FMW collided with MG - Option 1: UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion. - Option 2: UE to prioritize the OD-SSB carrier/layer within FWM

RAN4 already agreed in last meeting that when NW configures single OD-SSB carrier with multiple legacy SSB carriers, CSSF =1 for OD-SSB layer, OD-SSB carrier will be prioritized among all legacy SSB carriers outside gap. For OD-SSB within gap, we are ok to follow the legacy principle that UE is allowed to skip the OD-SSB within gap if FMW collides with MG occasion.

Proposal 1: UE is allowed to skip the OD-SSB(s) within gap if FMW collides with MG occasion(s).

Issue 1-1-2: OD-SSB based deactivated SCell measurement – Procedure Sub-issue 1: The start point of T2 Agreement: - The time point of OD-SSB deactivation. Sub-issue 2: The minimum interval(T2) between two FMWs Agreement: - For FR1, 5*max (measCycleSCell, DRX cycles) - For FR2, 4s for power class 1/5 and 3s for power class 2/3/4 Sub-issue 3 –UE’s behaviour when OD-SSB further activation/reconfiguration within T2 Agreement: - The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement within T2. Sub-issue 4 – UE’s behaviour after T2 when OD-SSB further activation/reconfiguration occurs within T2 between two FMWs The issue can be further discussed based on contribution driven, and the discussion will not impact the WI completion. - Option 1: When OD-SSB further activation/reconfiguration occurs within T2, the UE should perform ‘fast measurement’ immediately after T2. - Option 2: when OD-SSB further activation/reconfiguration occurs within T2, UE should follow legacy deactivated SCell measurement after T2. - Other option is not precluded.

In last meeting, RAN4 agreed the time point of OD-SSB deactivation as the start point of T2, which is the minimum interval between two FMWs.

Since UE is not required to perform “fast measurement” more frequently than the minimum interval T2, when NW re-activates the OD-SSB within the T2, UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement within T2.

However, if the network indication of OD-SSB was totally ignored by UE within 1st T2, UE could miss one change to fulfil the network’s expectation. And even one 2nd T2 will be restarted at the time instance of 2nd OD-SSB deactivation, which would cause OD-SSB activation/reconfiguration continuously being missed by UE if another shot of 3rd OD-SSB activation came within 2nd T2. In this case, UE would always follow legacy deactivated SCell measurement after 1st shot of fast measurement.

Observation 1: OD-SSB activation/reconfiguration may be continuously missed by UE if another shot of OD-SSB activation came within 2nd T2 triggered by 2nd OD-SSB de-activation.

In our view, we prefer option 1 that the UE should perform ‘fast measurement’ immediately after T2 when OD-SSB further activation/reconfiguration occurs within T2. The benefit of reactivation in such a short interval could be that network expect UE can quickly complete fast measurement and report again assuming the SCell is still a detected cell to UE. And UE can perform “fast measurement” without cell search immediately after T2, assuming skipping the PSS/SSS detection and/or SSB index acquisition.

Proposal 2: When OD-SSB further activation/reconfiguration occurs within T2, the UE should perform ‘fast measurement’ immediately after T2.

Open issues for Case 2

Issue 1-2-2: Requirement for RAN1 Alt Time-C2 Agreement - Option 1: RAN4 to define the delay requirement only based on the OD-SSB periodicity. - Option 2: RAN4 to define the delay requirement only based on the OD-SSB periodicity only if - The periodicity of AO-SSB and OD-SSB are same - It doesn’t impact the WI’s completion.

For scenario 2 with non-periodic SSB (i.e., RAN1 Alt Time-C2), since UE will follow single DL timing for measuring SSBs on the same frequency, non-periodic SSBs will bring extra efforts for UE to detect and measure. As for intra-frequency measurement on SSBs outside SMTC, no RRM requirements are defined, and additional SFTD measurement may be needed for timing alignment.

Similarly, we prefer to not to define any enhanced requirements for this scenario under NES operation. The legacy requirements for measurements on AO-SSBs within SMTC can be applied.

Proposal 3: For requirement for RAN1 Alt Time-C2, RAN4 not to define any new enhanced requirements. The legacy requirements for measurements on AO-SSBs within SMTC can be applied.

For scenario 3 (OD-SSB and always-on SSB are within different frequencies), since RAN2 agreed on a new MO for OD-SSB frequency layer which is different from AO-SSB frequency layer, network should enable one MO with reporting from OD-SSB MO and AO-SSB MO for the SCC, but not in parallel. Because usually one servingcellMO is configured or selected for one carrier, and it also impacts on the definition of intra-frequency layer . Even for Redcap UE, NCD-SSB and CD-SSB in servingcellMOs are not measured simultaneously. Otherwise, it is up to UE implementation for this scenario.

The requirements of scenario 3 will be very complicated if RAN4 was going to support it. From our side, we prefer not to define any requirements for scenarios 3 in R19.

Proposal 4: For scenario 3 (OD-SSB and always-on SSB are within different frequencies), RAN4 not to define requirements for different frequencies.

Other open issues

I Issue 1-4-3: Additional Processing time for OD-SSB activation Agreement - RAN4 should introduce UE capability with values of 2 ms and 5 ms. Issue 1-4-4: Additional Processing time for OD-SSB parameter update Agreement - Option 1: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates. - The additional processing time is 5ms or 2ms based on UE capability reporting. - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated]. - Option 2: When NW updates the OD-SSB parameters for an active Scell by MAC-CE, additional 5ms processing time is applied on top of THARQ+3ms after UE receiving the MAC-CE. - The additional processing time is 5ms - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated]. - Note 2: No capability is needed Issue 1-4-5: RAN1 LS - OD-SSB deactivation timeline Agreement - RAN4 to confirm RAN1’s working assumption about OD-SSB deactivation processing time. Further discuss the wording in the reply LS. - Further discuss in RAN4 the UE additional processing time for OD-SSB deactivation. - Option 1: No UE additional processing time for OD-SSB deactivation - Option 2: Follow the same values as UE additional processing time for OD-SSB activation

Proposal 5: For Additional Processing time for OD-SSB parameter update, we support option 2 without UE capability.

Proposal 6: For the UE additional processing time for OD-SSB deactivation, we support to follow the conclusion for OD-SSB activation.

RAN2 LS

For the case where AO-SSB and OD-SSB have the same center frequency, RAN2 had made the following agreement: - The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured. Please inform RAN2 if any concern regarding the above agreement.

About RAN2 LS on OD-SSB specific SMTC, when the OD-SSB is activated and Scell is activated, two kinds of SMTC can be used for L3 measurement on SCell (i.e., MO for servingcell) and intra-frequency neighbor cell of Scell (i.e., MO for intra-f neighbor cell). For the legacy, SMTC is shared for both serving cell and neighbour cell on the same SSB center frequency in the same MO. Now for OD-SSB of SCell, specific SMTC is introduced. Whether the OD-SSB specific SMTC or the legacy SMTC configured in the MO should be used for L3 measurement on intra-frequency neighbour cell on the same center frequency, needs to be discussed in RAN4. Two candidate options are proposed as below:

- Option 1: OD-SSB specific SMTC for both Scell and intra-frequency neighbor cell with SSB on the same frequency.

- Option 2:OD-SSB specific SMTC for Scell, and the legacy SMTC configured in MO on the SSB same frequency

From our side, we prefer option 1 because SMTC can be kept to be used as the L3 measurement timing configuration for all related cells to be measured including both serving cell and neighbour cell with the same SSB center frequency. Anyway, OD-SSB specific SMTC can be configured with RRC reconfiguration, and network can coordinate the L3 measurements on all cells based on SMTC.

RAN2 is to be informed if any clarification or concern are made regarding the above agreement. RAN4 will send LS to RAN2 based on the conclusion.

Proposal 7: RAN4 to clarify that OD-SSB specific SMTC can be used for L3 measurements on both Scell and intra-frequency neighbor cell with on the same SSB center frequency.

Proposal 8: Send LS to RAN2 based on Proposal 7.

Conclusions

In this paper, we discuss the open issues for OD-SSB based deactivated SCell L3 measurement in Case 1 and Case 2.

Proposal 1: UE is allowed to skip the OD-SSB(s) within gap if FMW collides with MG occasion(s).

Observation 1: OD-SSB activation/reconfiguration may be continuously missed by UE if another shot of OD-SSB activation came within 2nd T2 triggered by 2nd OD-SSB de-activation.

Proposal 2: When OD-SSB further activation/reconfiguration occurs within T2, the UE should perform ‘fast measurement’ immediately after T2.

Proposal 3: For requirement for RAN1 Alt Time-C2, RAN4 not to define any new enhanced requirements. The legacy requirements for measurements on AO-SSBs within SMTC can be applied.

Proposal 4: For scenario 3 (OD-SSB and always-on SSB are within different frequencies), RAN4 not to define requirements for different frequencies.

Proposal 5: For Additional Processing time for OD-SSB parameter update, we support option 2 without UE capability.

Proposal 6: For the UE additional processing time for OD-SSB deactivation, we support to follow the conclusion for OD-SSB activation.

Proposal 7: RAN4 to clarify that OD-SSB specific SMTC can be used for L3 measurements on both Scell and intra-frequency neighbor cell with on the same SSB center frequency.

Proposal 8: Send LS to RAN2 based on Proposal 7.

Reference

- R4-2508277, WF on RRM requirements for Netw_Energy_NR_enh_Part1

- R2-2504981, LS on L3 measurement of OD-SSB