# 3GPP TSG-RAN WG4 Meeting #116	R4-2509714			R4-250xxxx

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.22.3.1

Source: 	CMCC

Title: 	Discussion on A-IoT BS RF requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #106 meeting, a new WI [1] of A-IoT is approved with following RAN4 lead RF part objectives:

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

In this contribution, we discuss the RF requirements for Type 1-C Ambient-IOT BS.

- Discussion

2.1 A-IoT BS Tx requirement

- Tx signal quality related requirement

In theory, The baseband signal for OOK-4 is much like that of the ASK signal. But now RAN1 has introduced the CP for OOK-4 to achieve orthogonal with NR sub-carrier which will impact the modulation signal quality, resulting worse envelope ripple performance. Besides, we should consider the tradeoff between ripple and ACLR at reader side. The relaxaed ripple requirements, the better ACLR. Different from RFID without ACLR requirements which using specific spectrum without in-band spectrum sharing with other service, A-IoT BS should meet certain ACLR requirements to make sure co-existence with NR/LTE. So certain envelope ripple is required for better ACLR requirements.

At device side, typically, device will use 0.5 volt voltage difference on top of reference voltage to judge value 1 or value 0. So in theory the 0.5 volt voltage difference would cover the 0.15 ripple without impacting final digital judgement. In addition to, reference voltage is derived by filtering the signal using low-band pass filter with lower cutoff frequency after diode rectification, which means the transient envelop ripple will be ignored and will not impact reference voltage. So at device side, certain envelope ripple requirement relaxation is acceptable.

In last meeting, option 2 that no need to define RF envelop parameter is proposed. As discussed above, RAN4 needs to consider the trade off between this requirement and the ACLR requirement. If no such requirements, it’s risky that reader may sacrifice envelop ripple to reserve more space for ACLR which is not allowed. Besides, certain ripple relaxation is acceptable at device side. So it’s suggested to define envelope ripple requirements with some relaxation, e.g. 0.15 (A-B).

Proposal 1: it’s suggested to define RF envelope Ripple requirements with certain relaxation, e.g. 0.15*(A-B).

Compared with ripple requirements, RF pulsewidth and rise/fall time are much important since device demode value 1 and 0 in time domain using edge detection. If the RF pulsewidth is much shorter, the device may ignore signal jump, making it’s hard to identify the chip edge or demode signal.

Proposal 2: it’s suggested to reuse same RF envelope rise time, fall time and RF pulsewidth requirements as RFID requirements.

- ACLR

For unwanted emission requirements, legacy NB-IOT requirements are used for co-existence evaluation in SI. The conclusion of evaluation shows that legacy requirements can fulfil the co-existence when NR macro BS is outdoor and A-IOT reader is indoor. One scenario that causes severe interference is when legacy NR UE is indoor, which require some existing scheduling mechanism to avoid adjacent interference or enhanced mechanism to mitigate interference (as captured in TR 38.769 clause 6.6.5.10). Consider enhanced mechanism to mitigate interference is not within Rel-19 WI scope, then legacy NB-IOT requirements are suggested for A-IOT reader.

For NB-IoT, 100kHz offset is defined. OOK4 modulation scheme has worse out of band spectrum characteristics. In theory, reader will require larger offset for the same requirements. In last meeting, some companies propose to allow relaxation based on NB requirements with the same 100kHz offset. For future, reader FDM operation may be supported for other devices, e.g. 2a, 2b or 2c. for forward compatible, it’s better to use the same NB-IoT requriements. Our preference is the option 2 in last meeting.

Proposal 3: it’s suggested to define ACLR1/ACLR2 as 40dB/50dB but with wider offset if it’s hard with 100kHz offset.

2.2 A-IoT BS Rx requirement

- Reference Sensitivity

Based on RAN1 link budget analysis as below, R2D link is bottleneck for D1T1-B.

Table D1T1-96bit-10%BLER-6kbps-BPSK-6dBi BS antenna gain

| Scenario | CW case | R2D pathloss (dB) | D2R pathloss (dB) | min pathloss (dB) | coverage(m) |
| --- | --- | --- | --- | --- | --- |
| Device 1 |  |  |  |  |  |
| D1T1-B | 1-4 | 67.1 | 84.54 | 67.10 | 36.59 |

But it’s noted, RAN1 use 33dBm as reader max output power, but based on RAN4 latest agreements, reader output power could be increased to 38dBm per antenna connector. If the assume 2Tx of reader, then the coverage margin between R2D and D2R link could be shrink (from 17.44 to 9.44dB). If CW is distant from device, then the D2R link and R2D link could be comparable. Therefore, REFSENSE is significant.

Following list our simulation results and related parameters

| Parameters | Assumptions |
| --- | --- |
| Carrier frequency | 900 MHz |
| SCS | 15 kHz as baseline |
| Block structure | Depend on RAN1 |
| Channel model | AWGN |
| Number of Tx/Rx chains for Ambient IoT device | 1 |
| Number of BS antenna elements | 2 |
| Reference data rate | 1 kbps (M) |
| Message size | 96 bits |
| BLER target | 10% |
| Sampling frequency（SFO） | Randomly select a value from the range of [104 ppm~105 ppm] for device 1 |
| [OOK/BPSK chip rate] | 7.5 kcps for 2SB 15 kHz Transmission bandwidth |
| Receiver bandwidth | the same as transmission bandwidth. |
| Waveform (CW) | unmodulated single tone |
| Modulation | QPSK |
| Line code | Manchester encoding |
| FEC | 1/3 Convolutional Code |
| ADC bit width | Ideal |
| D2R receiver | coherent receiver |
| Repetition | None |
| Preamble configuration | 31bit; |
| Midamble configuration | 31bit; |

Table 1: LLS results

|  | BLER |  |
| --- | --- | --- |
| SNR | Variable SFO | Fixed SFO |
| -7.8 | 10% | 19% |
| -7.6 | 10% | 17% |
| -6.4 | 9% | 9% |
| -6.2 | 7% | 9% |
| -6 | 6% | 6% |

Observation 1: the SNR that meet 10% BLER is -6.4dB with corresponding assumptions as in above table.

As for the desens target, following list last meeting agreements.

- Issue 2-1-1: Reference sensitivity level - Agreement: - Define REFSENS for both OOK and BPSK. Depending on the simulations, they may or may not be the same. - Use [X dB] as the desens target, with CW single tone input through a signal generator, CW input level to BS Ant is assumed as [TBD] dBm;

In last meeting, it is approved to use signal generator to generate CW single tone. Compared with real CW node, the signal generator will produce much ideal CW signal which means the phase noise level is much lower than that of CW node.

Observation 2: the phase noise of signal generator is much lower than that of CW node.

Based on previous assumption, at BS side, CW cancellation mechanism could achieve about 100 dB using anolog and digital algorithm. But it’s hard for CW node to achieve 100dBc integral phase noise. This means CW’s phase noise is the key factor that dominate final REFSENSE performance if no additional phase noise cancellation algorithm is implemented.

Observation 3: the phase noise of CW is the key factor that will decrease REFSENSE performance.

However, current testing using the ideal CW with much better phase noise this means if use signal generator for final testing, we relax the RF requirements. In other words, the REFSENSE derived from testing using signal generator is the REFSENSE without considering phase noise impact.

Observation 4: the REFSENSE derived from testing using signal generator is the REFSENSE without considering phase noise impact, which equals to the testing criteria is relaxed.

Our suggestion is to update the CW testing signal and using real CW node for REFSENSE testing. If so, the testing CW node phase noise performance will impact reader’s REFSENSE testing.

Proposal 4: it’s suggested to use real CW node for reader REFSENSE testing instead of signal generator.

If companies insist on using CW generated by signal generator, residual phase noise during REFSENSE definition should be much stricter and the same as or even less than the phase noise performance of signal generator to align the RF requirements definition and the testing. In other words, the residual phase noise is not the key factor that determine final desense, instead, the residual CW signal is.

Observation 5: if companies insist on no update of current agreements, we need to define much stricter residual phase noise performance for REFSENSE definition, i.e. lower desense.

For the target desense, following lists our analysis. It’s suggested to use 6dB desense which is same for NR ACS/blocking related requirements. For the minimum 200kHz D2R, the wanted signal is -109dBm which corresponds to 74dB PL between device to reader and 20m distant between CW and device. Following assume -10dB backscatter loss, -6dB target SNR, 2dB implementation margin and 6dB antenna gain at reader side when calculating distance between CW and reader..

| D2R CBW | desense | wanted signal | noise floor | phase noise interference | integral phase noise/dBc | received CW signal | CW power | MCL between CW and BS | distance |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 20 | -106 | -112 | -92 | -50 | -42 | 33 | 75 | 309 |
|  | 20 | -106 | -112 | -92 | -60 | -32 | 33 | 65 | 98 |
|  | 20 | -106 | -112 | -92 | -70 | -22 | 33 | 55 | 31 |
|  | 20 | -106 | -112 | -92 | -80 | -12 | 33 | 45 | 10 |
|  | 20 | -106 | -112 | -92 | -90 | -2 | 33 | 35 | 3 |
|  | 20 | -106 | -112 | -92 | -100 | 8 | 33 | 25 | 1 |
| 200 | 6 | -109 | -111 | -106 | -50 | -56 | 33 | 89 | 1542 |
|  | 6 | -109 | -111 | -106 | -60 | -46 | 33 | 79 | 488 |
|  | 6 | -109 | -111 | -106 | -70 | -36 | 33 | 69 | 154 |
|  | 6 | -109 | -111 | -106 | -80 | -26 | 33 | 59 | 49 |
|  | 6 | -109 | -111 | -106 | -90 | -16 | 33 | 49 | 15 |
|  | 6 | -109 | -111 | -106 | -100 | -6 | 33 | 39 | 5 |
| 3520 | 6 | -97 | -99 | -94 | -50 | -44 | 33 | 77 | 368 |
|  | 6 | -97 | -99 | -94 | -60 | -34 | 33 | 67 | 116 |
|  | 6 | -97 | -99 | -94 | -70 | -24 | 33 | 57 | 37 |
|  | 6 | -97 | -99 | -94 | -80 | -14 | 33 | 47 | 12 |
|  | 6 | -97 | -99 | -94 | -90 | -4 | 33 | 37 | 4 |
|  | 6 | -97 | -99 | -94 | -100 | 6 | 33 | 27 | 1 |

| CW power | PL:CW to device/20m | backscatter loss | PL: device to reader | REFSENSE |
| --- | --- | --- | --- | --- |
| 33 | 58 | 10 | 72 | -107 |

Proposal 5: OOK REFSENSE for 15kHz CBW is -106dB with 10dB desense which lead to 70dB residual integral phase noise when distance between CW and BS is 30m.

- the narrowband IMD

- Currently, the NB-IoT has been widely used indoors, e.g. the Water meter, gas meter, electricity meter. In the large factory, the NB-IoT will also be used in automatic parking system. Therefore, the narrowband IMD requirements should be defined for A-IoT BS. Besides, n8 900MHz is the main deployment spectrum for NB-IoT due to better propergation performance.

- The narrowband IMD requirement is defined for NB-IoT. And for the in-band mode, NB-IoT almost reuse the same requirements as the same class BS without NB except for adding new notes to emphase “The frequency offset shall be adjusted to accommodate the IMD product to fall in the NB-IoT RB for NB-IoT in-band operation”. for the interfering level, it reuse the same level as ACS requirements wich is 9dB less than the blocking requirement derived from simulation considering co-location case.

- For A-IoT, usually, the NB-IoT device will be relatively far away from the A-IoT BS from deployment point of view. In theory, the interference level could be much less than -47dBm for medium range BS.

- If we assume 10m distance, the interference level could be -67dBm (additional 20dB passloss) assuming BS- BS minimum 1m distance.

- If we assume 5m distance, the interference level could be -61dBm (additional 14dB passloss) assuming BS- BS minimum 1m distance.

- If we assume 2m distance, the interference level could be -53dBm (additional 6dB passloss) assuming BS- BS minimum 1m distance.

- For indoor case, to not limit the NB BS and A-IoT deployment, we suggest to use 2m distance assumption, which means the interference level is -53dBm.

- For other narrow band IMD requirements, it’s suggested to reuse the same value as for medium range NB-IoT system.

- Proposal 6: for narrow band IMD requirements, the interference level is suggested as -53dBm. Other parameters are suggested to reuse the same value as NB IoT system.

# 3. Conclusions

In this contribution, RF requirements for gNB like reader are discussed with following observations and proposals.

Proposal 1: it’s suggested to define RF envelope Ripple requirements with certain relaxation, e.g. 0.15*(A-B).

Proposal 2: it’s suggested to reuse same RF envelope rise time, fall time and RF pulsewidth requirements as RFID requirements.

Proposal 3: it’s suggested to define ACLR1/ACLR2 as 40dB/50dB but with wider offset if it’s hard with 100kHz offset.

Observation 1: the SNR that meet 10% BLER is -6.4dB with corresponding assumptions as in above table.

Observation 2: the phase noise of signal generator is much lower than that of CW node.

Observation 3: the phase noise of CW is the key factor that will decrease REFSENSE performance.

Observation 4: the REFSENSE derived from testing using signal generator is the REFSENSE without considering phase noise impact, which equals to the testing criteria is relaxed.

Proposal 4: it’s suggested to use real CW node for reader REFSENSE testing instead of signal generator.

Observation 5: if companies insist on no update of current agreements, we need to define much stricter residual phase noise performance for REFSENSE definition, i.e. lower desense.

Proposal 5: OOK REFSENSE for 15kHz CBW is -106dB with 10dB desense which lead to 70dB residual integral phase noise when distance between CW and BS is 30m.

- Proposal 6: for narrow band IMD requirements, the interference level is suggested as -53dBm. Other parameters are suggested to reuse the same value as NB IoT system.

# 4. Reference

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.

- R4-77AH-IoT-0018, On BS emission requirement in standalone operation for NB-IoT, Huawei

- R4-160610, On BS emission mask in stand-alone operation for NB-IoT, Huawei

- R4-161289, Way forward on BS unwanted emission requirement for NB-IoT

- R4-2420470, WF on RF requirements for A-IOT, Moderator (Huawei).

- R4-2417088, WF on RF requirement impact of A-IoT, Huawei.

- R4-2414305, WF on RF requirements for A-IoT, Huawei

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID, Specification for RFID Air Interface Protocol for Communications at 860 MHz - 960 MHz, Version 2.0.0 Ratified