# 3GPP TSG-RAN WG4 Meeting #116			                           R4-2509773

# Bengaluru, India, August 25th – 29th, 2025

Source: 	Xiaomi

Title: 	Discussion on Study of testability and interoperability for AI/ML mobility

Agenda Item:	7.19.4

Document for:	Discussion

# 1Introduction

Ddiscussion on testability and interoperability for AI/ML mobility was still going on. Here below we attach the RRM agreements on testability and interoperability in previous RAN4 meeting:

Issue 1-1-1: Prediction consistency in time domain <Way forward >: - Consider the propagation condition for the validation of the RRM measurement prediction requirements before modeling different time-varying characteristics per cell/site in FR1. - Further discuss the following options on how to model different time-varying characteristics per cell/site in FR1. - Option 1: Change Tx power. - FFS: whether it is possible to change Tx power every symbol at TE using conductive test - Option 2: Change channel condition with fixed Tx power. - Option 3: Computer simulations alike test setup can be adopted. TE can transmit the signals of cell 1, 2, 3 with time-variance following a certain channel model. - Option 4: Select one reference UE trajectory model (e.g., from TR 38.843) to study prediction consistency impact Issue 1-2-1: Testing Setup in FR2 <Way forward >: - For the testing setup for AI/ML mobility in FR1 and FR2, discuss the following configurations in the next meeting: - Number of SSBs per cell - Channel model - FFS the same or different for temporal/frequency domain prediction - Impact of UE trajectory - FFS the same or different for temporal/frequency domain prediction - Dynamic range of received power (consistency of channel) - Measurement prediction specific parameters - Emulation of noise and EPRE - L3 filter coefficient

Based on the achieved agreements, we would like to share our further consideration on the testability and interoperability issues for AI/ML mobility and provide our proposals.

- Discussion

RAN4 has discussed several meetings on how to test UE mobility performance under measurement prediction use case.

For FR1, based on last meeting progress, we would like to provide our consideration on the prediction consistency in time domain firstly.

To verify the accuracy of the predicted L3 filtered cell-level measurement result, from our view, sufficient randomness and variation in time has to be configured in the test and emulated by the test equipment. Basically, there are two propagation assumptions: AWGN channel and Fading channel. The AWGN channel is a deterministic channel, where signal power will be constant for both spatial and time domain. While, the Fading channel is a random channel, where variation in spatial and/or time domain can be included. It’s hard to verify the prediction functionality introduced by AI using AWGN, since there is no any uncertainty in both spatial and time domain. Just as the progress achieved in Channel model assumption for AI based BM, we think the fading channel should be assumed in the test. For both the scenarios of FR1 to FR1 inter-frequency intra-cell temporal domain case B and FR1 to FR1 inter-frequency inter-cell frequency domain, we understand the variation in time domain would be enough for FR1 scenario, so we think the TDL channel model is more reasonable.

Proposal 1: For testing in FR1 RRM measurement prediction, RAN4 to consider TDL channel model.

In last meeting, RAN4 agree to discuss how to model different time-varying characteristics per cell/site in FR1, which is mainly considered to simulate the impact due to moving UE trajectories. AI model is expected to be trained with real-world data, which is collected under dynamic UE mobility conditions. However, traditional test methods are executed under certain SNR level with velocity-induced doppler shifts. We understand the UE trajectory would introduce large-scaled propagation parameters impact for test setup. RAN4 has agreed to consider conducted testing as a baseline option for testing in FR1. In conducted testing, the DUT is directly connected to test equipment using physical cables. The TE simulate signal through controlled, precise signal generation. To reflect the large-scaled impact due to moving UE trajectories, we think it is reasonable to change Tx power, which is the transmit power generating by TE. In TR 38.843 section 6.3.1, 3 options of UE trajectory model are considered. We think RAN4 can take these three models as starting point.

Proposal 2: For testing in FR1 RRM measurement prediction, RAN4 to simulate the UE moving trajectories impact by changing Tx power.

The issue of how to obtain the ground truth in FR1 during test has been discussed for several meetings. During RRM requirements discussion for FR1 in SI stage, two alternatives are captured in the TR for further study. Alt1 is derived based on the transmitted or reception power, and Alt2 is derived based on the reported measurement value under certain conditions. For Alt1, as RAN4 has concluded that conductive testing method could be used for test in FR1, the TE can adjust the transmission power to mimic the cell to be measured and the cell to be predicted similar to legacy approach. So, we agree that the actual ground truth is available at the TE. However, we see some potential issues. As we support to use fading channel, the channel conditional is random in time domain. UE and TE need to align the time instance during the test when UE conducting the prediction to get the corresponding ground truth as UE implementation are diverse, e.g. sliding and non-sliding. We believe this issue need to be figured out in L3 RSRP prediction accuracy discussion. For Alt2, from our side, the main point is to solve the potential UE cheat issue, while the test procedure can reuse the legacy as baseline.

Proposal 3: For testing in FR1 RRM measurement prediction, the way the ground truth is derived has impact on the testing procedure:

- For Alt2 that the ground truth of L3-RSRP is reported by UE, the legacy test procedure can be taken as baseline;

- For Alt1 that the ground truth of L3-RSRP based on the transmitted or reception power, additional test procedure on how TE determine the ideal L3-RSRP need to be further studied.

Specifically, for the FR1 to FR1 inter-frequency inter-cell frequency domain scenario, the multi-cell configuration need to be pay more attention. To verify the accuracy of the predicted L3 filtered cell-level measurement result in this scenario, multi-cell configuration in the test need to be further studied. We think the configuration in this case depends on the number of cells to be predicted, which needs to wait for RAN2 decision. If we only verify that case one cell to be measured and one cell to be predicted, we understand that the legacy test setup can be used as baseline. If we need to verify the case one cell to be measured and multiple cells to be predicted, we think the test setup need to be further studied.

Proposal 4: For the scenario of FR1 to FR1 inter-frequency inter-cell frequency domain scenario, the legacy test procedure can be taken as baseline for single cell prediction. FFS for multi-cells prediction.

Furthermore, for the testing setup in FR1, we support that RAN4 to reuse the legacy testing setup as much as possible. As agreed in section A.3.6.1 of TS 38.133, the basic antenna configuration for legacy FR1 RRM test case is antenna configuration 1x2, which, from our understanding, represents the configuration of 1 Tx port and 2 Rx ports. We support to consider the 1 Tx configuration for FR1 RRM measurement prediction test as well.

### A.3.6.1 Antenna configurations for FR1 Unless otherwise specified, NR FDD or NR TDD cells in all RRM Test cases in AWGN propagation condition are configured with Antenna Configuration 1x2.

Considering the testing setup on the number of SSBs per cell, we support to take 1 SSB per SS-burst as baseline. Going through the legacy FR1 mobility related test cases, basically all the SSB configuration parameter considers 1 SSB per SS-burst. For example, for the legacy SA event triggered reporting tests without gap under non-DRX, we capture the general test parameter as below:

Table A.6.6.1.1.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for FR1 Parameter; Unit; Test configuration; Value; Comment. Active cell; 1, 2, 3; Cell 1. Neighbour cell; 1, 2, 3; Cell 2; Cell to be identified.. RF Channel Number; 1, 2, 3; 1: Cell 1 and Cell 2. SSB configuration; 1; SSB.1 FR1. 2; SSB.1 FR1. 3; SSB.2 FR1. SMTC configuration; 1; SMTC.2. 2; SMTC.1. 3; SMTC.1. A3-Offset; dB; 1, 2, 3; -4.5. CP length; 1, 2, 3; Normal. Hysteresis; dB; 1, 2, 3; 0. Time To Trigger; s; 1, 2, 3; 0. Filter coefficient; 1, 2, 3; 0; L3 filtering is not used. DRX; 1, 2, 3; OFF. Time offset between serving and neighbour cells; 1; 3 ms; Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1.. 2; 3 s; Synchronous cells. 3; 3 s; Synchronous cells. T1; s; 1, 2, 3; 5. T2; s; 1, 2, 3; 5.

The SSB pattern of SSB.1 FR1 and SSB.2 FR1 are configured with 1 SSBs per SS-burst. As we support to reuse the legacy testing setup as much as possible, we prefer to follow the same SSB configuration as baseline.

Proposal 5: For testing setup in FR1 RRM measurement prediction, RAN4 to take antenna configuration of 1 Tx and SSB configuration of 1 SSB per SS-burst as baseline.

For FR2, we understand that we face the similar testability issues as in NR_AIML_Air WI. We think the conclusions on testability achieved in NR_AIML_Air WI can be taken as baseline.

Proposal 6: For FR2 measurement prediction, conclusions on testability that RAN4 made for AI/ML BM can be taken as a baseline.

- Conclusion

Proposal 1: For testing in FR1 RRM measurement prediction, RAN4 to consider TDL channel model.

Proposal 2: For testing in FR1 RRM measurement prediction, RAN4 to simulate the UE moving trajectories impact by changing Tx power.

Proposal 3: For testing in FR1 RRM measurement prediction, the way the ground truth is derived has impact on the testing procedure:

- For Alt2 that the ground truth of L3-RSRP is reported by UE, the legacy test procedure can be taken as baseline;

- For Alt1 that the ground truth of L3-RSRP based on the transmitted or reception power, additional test procedure on how TE determine the ideal L3-RSRP need to be further studied.

Proposal 4: For the scenario of FR1 to FR1 inter-frequency inter-cell frequency domain scenario, the legacy test procedure can be taken as baseline for single cell prediction. FFS for multi-cells prediction.

Proposal 5: For testing setup in FR1 RRM measurement prediction, RAN4 to take antenna configuration of 1 Tx and SSB configuration of 1 SSB per SS-burst as baseline.

Proposal 6: For FR2 measurement prediction, conclusions on testability that RAN4 made for AI/ML BM can be taken as a baseline.

- Reference

- R4-2508269, WF on RRM requirements for FS_NR_AIML_Mob, Nokia