3GPP TSG-RAN WG4 Meeting #116				                               R4-2509800

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.23.4

Title: 	Discussion on the performance part for NES

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In the last RAN4 meeting, there were some initial inputs on the test cases for Rel19 NES but without agreements on the common testing configuration and part of test cases [2]. In this paper we provide further consideration on which test cases needed and how to define the corresponding testing configurations.

- Discussion

  - OD-SSB test cases

According to the discussion on the core part, the using scenario for OD-SSB can be:

- Case #1: No always-on SSB on the cell and

- Case #2: Always-on SSB is periodically transmitted on the cell

And the following agreement was achieved in RAN4#112 regarding to the Scenario 2 and 2A

- Agreement: - RAN4 to discuss OD-SSB based SCell activation requirements based on following RAN1 agreed scenarios. - Case #1 and Scenario #2 - Case #1 and Scenario #2A - Case #2 and Scenario #2 - Case #2 and Scenario #2A

Usually in RAN4 in order to save the testing efforts, we can define test cases for part of them above.

Proposal 1: RAN4 can define the OD-SSB based test case for

    - Case #1 and Scenario #2

    - Case #2 and Scenario #2A

For the type of OD-SSB, since RAN4 agreed to take NCD-SSB for the OD-SSB requirements, we need only to define the test case for NCD-SSB.

R4#112bis agreements: RAN4 to first take NCD-SSB as a start point for OD-SSB requirements.

Proposal 2: Define the NCD OD-SSB test cases only.

However, for the test cases configuration themselves, in our views, in order to simplify the test case setting, we prefer the following configurations:

- Both FR1 and FR2

- Only test OD-SSB periodicities with 5ms, and 20ms

- Only test non-DRX mode

- without SSB time index detection

Proposal 3: RAN4 to define the OD-SSB test cases with the following general principles:

- Both FR1 and FR2

- Only test OD-SSB periodicities with 5ms, 10ms and 20ms

- Only test non-DRX mode

- Without SSB time index detection

On the other hand, in comparison with the legacy always-on SSB, the key new aspects for OD-SSB introduced in Rel19 include:

- Case 1 OD-SSB based deactivated SCell measurement

- Case 1 & 2 SCell activation time

Therefore, we suggest to define the test cases for the above two functionalities at least.

Proposal 4: RAN4 can define the following test cases with the specific configurations as:

| No. | Test category | Test configuration |
| --- | --- | --- |
| 1 | OD-SSB based deactivated SCell measurement | OD-SSB periodicity 10ms NCD No-DRX FR1 &FR2 |
| 2 | SCell Activation delay requirements | OD-SSB periodicity 20ms Unknown SCell NCD No-DRX FR1 &FR2 |

  - Adaptation SSB test cases

For using scenario, the SSB adaptation is for RRC_CONNECT mode only. Thus we need to define the test cases for adaptation SSB in RRC_CONNECT mode.

R4#113 discussion Agreement: - No RRM impacts for SSB adaptation requirement for IDLE mode.

Proposal 5: RAN4 shall define the test cases for adaptation SSB in RRC_CONNECT mode only.

Based on the core part discussion, the adaptation SSB can be used for:

- L1 measurement

  - L1-RSRP, L1-SINR, CBD

- L3 measurement

- SCell activation

R4#112bis discussion Agreement: - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement Note: Above list could be further updated based on RAN4 discussion.

Therefore the proposed test case list for the adaptation SSB can be:

Proposal 6:RAN4 can define the following SSB adaptation test case

- L1-RSRP

- L1-SINR

- CBD/BFD

- L3 measurement

- Deactivated SCell measurement

- normal SCell activation

- Summary

This paper proposes the main configurations parameters and test cases list to verify the RRM requirements related to NES in Rel19.

Proposal 1: RAN4 can define the OD-SSB based test case for

    - Case #1 and Scenario #2

    - Case #2 and Scenario #2A

Proposal 2: Define the NCD OD-SSB test cases only.

Proposal 3: RAN4 to define the OD-SSB test cases with the following general principles:

- Both FR1 and FR2

- Only test OD-SSB periodicities with 5ms, 10ms and 20ms

- Only test non-DRX mode

- Without SSB time index detection

- Proposal 4: RAN4 can define the following test cases with the specific configurations as:

| No. | Test category | Test configuration |
| --- | --- | --- |
| 1 | OD-SSB based deactivated SCell measurement | OD-SSB periodicity 10ms NCD No-DRX FR1 &FR2 |
| 2 | SCell Activation delay requirements | OD-SSB periodicity 20ms Unknown SCell NCD No-DRX FR1 &FR2 |

Proposal 5: RAN4 shall define the test cases for adaptation SSB in RRC_CONNECT mode only.

Proposal 6:RAN4 can define the following SSB adaptation test case

- L1-RSRP

- L1-SINR

- CBD/BFD

- L3 measurement

- Deactivated SCell measurement

- normal SCell activation

- References

- RP-240857

- R4-2505570

- 3GPP TS 38.133 v19.1.0