3GPP TSG-RAN WG4 Meeting #116		R4-2510026

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.23.3.1

Source: 	CMCC

Title: 			  Discussion on RRM requirement of OD-SSB for NES enhancement

Document for:	Discussion

- Introduction

In last meeting, RAN4 mainly discussed about T2 design and SCell activation related issues, the WF has been approved in [1]. In this paper, we further discuss OD-SSB related open issues.

- OD-SSB based deactivated SCell L3 measurement procedure (Case 1)

Issue 1-1-1: OD-SSB based deactivated SCell measurement – requirement in FMW

Sub-issue 2: OD-SSB in FMW collided with MG

- Option 1: UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion.

- Option 2: UE to prioritize the OD-SSB carrier/layer within FWM

In legacy, the MG based measurement always have the highest priority. The SMTC for intra-frequency measurement will be skipped if it collides with MG occasion, which is reflected in ‘Kp’ factor.

To avoid the impact on MG based measurement, UE shall be allowed to skip the OD-SSB measurement within FMW if the OD-SSB occasion in FMW collides with MG occasion.

Currently, the length of FMW is Tuncertain + min(Treport , Tidentify), and Tidentify equals the minimum RAN4 OD-SSB measurement period requirement as follow.

- A detected cell: OD-SSB based measurement period

- A newly detectable cell: OD-SSB based cell identification period

We think the above OD-SSB skipping behavior shall also be captured in the definition of Tidentify, in which using ‘Kp’ to scale the measurement period with the consideration of measurement occasion collides with MG. The definition of ‘Kp’ shall be updated as:

When UE supports concurrent GAPs, i.e., supports the following capability or capabilities’ combination: - concurrentMeasGap-r17, or - concurrentMeasGapsPreMG-r18, or - concurrentMeasGapsNCSG-r18, Or when UE supports musim-GapPreference-r17 or both concurrent measurement GAPs and musim-GapPreference-r17 and UE concurrent GAPs or periodic MUSIM gaps or both concurrent gaps and periodic MUSIM gaps are configured Kp is the scaling factor for an SSB frequency layer to be measured without GAP. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows: - For a window W of duration max(OD-SSB period, xRP_max), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the SSB frequency layer, and starting from the beginning of any OD-SSB occasion: - Ntotal is the total number of OD-SSB occasions within the window, including those overlapped with GAP and MUSIM gap occasions within the window, and - Navailable is the number of OD-SSB occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions within the window W, after accounting for measurement GAP and MUSIM gap collisions by applying the collision rules for GAP and MUSIM gap in clauses 9.1.8.3, 9.1.10.4, 9.1.10.5, 9.1.12.3, and 9.1.13.3, respectively. Kp = 1 when Navailable = 0. - xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap. Requirements in this clause do not apply when Navailable = 0 due to fully overlapping between SMTC occasions and MUSIMmeasurement gap occasions within the window W. When UE supports [MUSIM-GapConfig-17] and the SMTC occasion of the target frequency layer is overlapping with the configured aperiodic MUSIM gap, longer cell identification period for the target frequency layer is expected. - Otherwise, when the UE is not configured with or UE does not support concurrent GAPs and the UE is not configured with periodic MUSIM gaps or UE does not support MUSIM gaps: When OD-SSB is fully non overlapping with measurement gaps or NCSG, or intra-frequency SMTC is fully overlapping with MGs or NCSG, Kp=1 When OD-SSB is partially overlapping with measurement gaps, Kp = 1/(1- (OD-SSB period /MGRP)), where OD-SSB period < MGRP. When intra-frequency SMTC is partially overlapping with NCSG, Kp = 1/(1- (OD-SSB period /VIRP)), where OD-SSB period < VIRP. For calculation of Kp, if the high layer signalling (TS 38.331 [2]) of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, the SMTC periodicity corresponds to the value of higher layer parameter smtc2; for the other cells, the SMTC periodicity corresponds to the value of higher layer parameter smtc1. If the higher layer signaling in TS 38.331 [2] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index or Tidentify_intra_with_index Requirements in this clause do not apply when OD-SSB is fully overlapping with measurement gaps or NCSG.

Proposal 1: UE is allowed to skip the OD-SSB measurement within FMW if the OD-SSB occasion in FMW collides with MG occasion, and this shall be considered in the ‘Kp’ factor of Tidentify when define the FMW length. ‘Kp’ shall be updated as above.

Issue 1-1-2: OD-SSB based deactivated SCell measurement – Procedure

Sub-issue 4 – UE’s behaviour after T2 when OD-SSB further activation/reconfiguration occurs within T2 between two FMWs

- The issue can be further discussed based on contribution driven, and the discussion will not impact the WI completion.

- Option 1: When OD-SSB further activation/reconfiguration occurs within T2, the UE should perform ‘fast measurement’ immediately after T2.

- Option 2: when OD-SSB further activation/reconfiguration occurs within T2, UE should follow legacy deactivated SCell measurement after T2.

- Other option is not precluded.

In our understanding, when to activate the OD-SSB is up to network, then this scenario may be happened, but not typical.

We see the pros and cons of Option 1 as follows:

Pros: If network expect one valid measurement report after this OD-SSB activation, then UE with Option 1 can derive one valid measurement result with less OD-SSB occasion, furthermore, help network with energy saving.

Cons: For every OD-SSB occasion, UE shall determine whether it occurs within or after T2, then decide the measurement behaviour. Besides, the starting time and length of FMW defined before can not be reused.

With above analysis, although we do see some benefits of Option1, we are also ok to go with Option 2 for the sake of progress.

Proposal 2: Both ok with Option 1 and Option 2. If Option 1 is adopted, then the starting time and length of FMW should be re-defined.

- OD-SSB based deactivated SCell L3 measurement (Case 2)

Issue 1-2-2: Requirement for RAN1 Alt Time-C2

Agreement

- Option 1: RAN4 to define the delay requirement only based on the OD-SSB periodicity.

- Option 2: RAN4 to define the delay requirement only based on the OD-SSB periodicity only if

  - The periodicity of AO-SSB and OD-SSB are same

- It doesn’t impact the WI’s completion.

The definition of Alt Time-C2 is:

Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported.

Firstly, we think it is rather complicated to define the requirement based on the uneven distributed union of AO-SSB and OD-SSB. Then, considering the OD-SSB periodicity is not larger than the AO-SSB periodicity, we think the requirement shall be defined based on OD-SSB periodicity to ensure the network power saving gain. The requirement we mentioned here including the measurement requirement in FMW, the SCell activation delay requirement and the  L1 measurement in the active SCell after SCell activation.

Proposal 3: For Alt Time-C2 case, the measurement requirement in FMW, the SCell activation delay requirement and the  L1 measurement in the active SCell after SCell activation shall be defined based on OD-SSB periodicity.

Issue 1-2-4: Requirement for scenario 3 (OD-SSB and always-on SSB are within different frequencies)

Sub issue 2: Requirement for scenario 3

Agreement:

- RAN4 will further discuss whether and how to define the requirements for scenario 3, and meanwhile the discussion will not impact the WI completion.

- Agreement from RAN2#130

- When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

- On one hand, Scenario 3 that AO-SSB and OD-SSB have different center frequency has been identified and confirmed by RAN1. RAN2 also introduced a new servingCellMO-OD to indicate the MO of OD-SSB. It is feasible for RAN4 to define the requirement for this scenario 3. On the other hand, in our understanding, the use case of Scenario 3 is to make sure the Cell can be also used as PCell, since there are AO-SSBs on sync raster, but the NES gain will decrease or disappear at this case. Based on above consideration, we don’t have strong view on whether to define the requirements for Scenario 3, but we still would like to share our view on how to define the requirements for Scenario 3.

- Based on the agreement from RAN2, that when OD-SSB is activated, UE use servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell. Base on that, about when to measure the OD-SSB with fast mode, the following agreement of Case2 can be reused:

- UE receive an OD-SSB activation indication, and

- UE has not sent a valid measurement report before OD-SSB activation indication within the time window as follow, and

  - max(5*measCycleSCell,  5*DRX cycles) for FR1, or

  - 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 in FR2.

- The time duration between OD-SSB activation indication and last OD-SSB de-activation indication is longer than T2, and

- After OD-SSB activation indication, within the FMW

Proposal 4: For Scenario 3 that OD-SSB and AO-SSB are within different frequencies, when OD-SSB is activated, UE use servingCellMO-OD to measure serving cell. Reuse the agreement of Case 2 about when to measure the OD-SSB with fast mode.

- As for the neighbour cell measurement, when OD-SSB is activated, the neighbour cell measurement on OD-SSB carrier shall follow the requirement of intra-frequency measurement, and the neighbour cell measurement on AO-SSB carrier shall follow the inter-frequency measurement without gap requirement. When OD-SSB is deactivated, we think there is no need to perform the neighbour cell measurement on OD-SSB carrier, since the OD-SSB is mainly for serving cell measurement and synchronization.

Proposal 5: When OD-SSB is deactivated, UE don’t perform the neighbour cell measurement on OD-SSB carrier.

Proposal 6: When OD-SSB is activated, UE perform neighbour cell measurement on OD-SSB carrier with intra-frequency measurement requirement, and perform neighbour cell measurement on AO-SSB carrier with inter-frequency measurement without gap requirement.

- OD-SSB based SCell activation

Issue 1-3-1: The known cell condition in Case 2

Agreement

- The current known SCell activation delay methodology can be reused in Case 2 at least for Alt Time-C1 with the following combination.

  - When the SSB measured for known cell condition is AO-SSB or OD-SSB or the combine of AO-SSB and OD-SSB, and the SSB used in SCell activation procedure is AO-SSB, OD-SSB or the combine of AO-SSB and OD-SSB.

- FFS whether apply to Alt Time-C2, if the requirement is defined.

- FFS whether apply to scenario 3, if the requirement is defined.

- FFS whether additional AGC time should be included in the known SCell activation delay requirement for scenario 3, if defined.

- In last meeting, the known cell condition for Case 2 Alt Time-C1 has been agreed. The FFS part is how to define the known condition for Case 2 Alt Time-C2 and scenario 3.

- Regarding Alt Time-C2, all AO-SSB and OD-SSB settings are same with Alt Time-C1 except for the union of AO-SSB and OD-SSB has a non-periodic time domain pattern, the combination measurement of AO-SSB and OD-SSB doesn’t exist. Therefore, the agreement of Alt-Time C1 can be applied except for AO-SSB and OD-SSB combination part.

Proposal 7: The current known SCell activation delay methodology can be reused in Case 2 Alt Time-C2 with the following combination.

- When the SSB measured for known cell condition is AO-SSB or OD-SSB, and the SSB used in SCell activation procedure is AO-SSB or OD-SSB.

- Regarding Scenario 3, firstly, the time-domain synchronization information can be shared between AO-SSB and OD-SSB due to following relationship:

- SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters.

- When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications

- Secondly, although the frequency location of AO-SSB and OD-SSB is different, due to AO-SSB and OD-SSB are in the same BWP, the AGC information can be also shared between AO-SSB and OD-SSB.

- Lastly, UE can not combine the measurement of AO-SSB and OD-SSB due to different frequency.

- In summary, the agreement of Alt-Time C1 can be applied except for AO-SSB and OD-SSB combination part.

Proposal 8: The current known SCell activation delay methodology can be reused in Scenario 3 with the following combination.

- When the SSB measured for known cell condition is AO-SSB or OD-SSB, and the SSB used in SCell activation procedure is AO-SSB or OD-SSB.

- OD-SSB general issues

Issue 1-4-4: Additional Processing time for OD-SSB parameter update

Agreement

- Option 1: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates.

  - The additional processing time is 5ms or 2ms based on UE capability reporting.

  - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated].

- Option 2: When NW updates the OD-SSB parameters for an active SCell by MAC-CE, additional 5ms processing time is applied on top of THARQ+3ms after UE receiving the MAC-CE.

  - The additional processing time is 5ms

  - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated].

  - Note 2: No capability is needed

- From UE processing point of view, we think the processing procedure for ‘OD-SSB activation’ and ‘OD-SSB parameter update’ are similar, all are talking about when UE will realize that a OD-SSB burst which is should have been transmitted is no longer being transmitted, or else versa.

- Therefore, we propose to reuse the ‘OD-SSB activation’ timeline for ‘MAC CE based OD-SSB transmission parameter updates’, which depends on UE capability. Furthermore, we think the UE processing capability for this two features shall be consistent, which means if UE have the capability to process OD-SSB activation with 2ms processing time, UE shall also have the capability to process OD-SSB parameter update with 2ms processing time. From this point of view, to reduce the UE capability overhead, we think one UE capability can be shared by two features, further extend the UE capability of ‘Additional processing time for OD-SSB activation’ to ‘Additional processing time for OD-SSB activation and MAC-CE based OD-SSB transmission parameter updates’.

Proposal 9: Apply the ‘OD-SSB activation’ timeline and the UE capability for ‘MAC CE based OD-SSB transmission parameter updates’, that the additional processing time is 5ms or 2ms based on UE capability reporting.

Proposal 10: Extend the range of the UE capability ‘Additional processing time for OD-SSB activation’ to ‘Additional processing time for OD-SSB activation and MAC-CE based OD-SSB transmission parameter updates’.

Issue 1-4-5: RAN1 LS - OD-SSB deactivation timeline

Agreement

- RAN4 to confirm RAN1’s working assumption about OD-SSB deactivation processing time. Further discuss the wording in the reply LS.

- Further discuss in RAN4 the UE additional processing time for OD-SSB deactivation.

  - Option 1: No UE additional processing time for OD-SSB deactivation

  - Option 2: Follow the same values as UE additional processing time for OD-SSB activation

- In last meeting, some companies think that if no deactivation processing time, there may exist scheduling issue. For example, UE doesn’t realize that a OD-SSB burst which is should have been transmitted is no longer being transmitted, then the data on this resource will be missed.

- However, after the double check, we found that the OD-SSB deactivation will only happen before SCell activation, therefore, no data scheduling at all. From this perspective, we think no additional processing time is needed for OD-SSB deactivation.

Proposal 11: No UE additional processing time for OD-SSB deactivation.

Issue 1-4-10: OD-SSB interruption requirements

Background: Agreement from RAN4#114bis

- New interruption is allowed for scenario 2 due to OD-SSB activation/deactivation.

- New interruption is also allowed for scenario 2 at the end of the FMW if UE does not receive SCell activation command.

- The interruption can be applied in both Case 1 and Case 2.

- The detail condition, duration and position of the interruption is FFS.

- The interruption is allowed for scenario 2a due to OD-SSB activation/deactivation.

- No new interruption requirement is defined.

- The interruption requirement follows the same principle of legacy SCell activation.

- For Case 1 Scenario 2 and Scenario 3, when OD-SSB is activated or deactivated, the UE RF chain on OD-SSB carrier will be ON or OFF correspondingly, which will cause interruption on other RF chains. The starting time of interruption is only allowed within the UE processing delay T_processing (including HARQ, MAC-CE processing and additional processing time). The interruption duration shall be 1 slot for FR1.

Proposal 12: For Case 1 Scenario 2 and Scenario 3, new interruption is allowed due to OD-SSB activation/deactivation. The starting time of interruption is only allowed within the UE processing delay T_processing (including HARQ, MAC-CE processing and additional processing time). The interruption duration shall be 1 slot for FR1.

- For Case 2 Scenario 2, the UE RF chain state keeps unchanged by OD-SSB activation/deactivation due to AO-SSB are transmitted on this carrier, so no interruption will be caused.

- Proposal 13: For Case 1 Scenario 2, new interruption is not allowed due to OD-SSB activation/deactivation.

- For both Scenario 2 and Scenario 3, if UE does not receive SCell activation command at the end of FMW, the same interruption of deactivated SCell measurement interruption can be applied, provided that the measCycleSCell equal or longer than 640ms.

- Proposal 14: For both Scenario 2 and Scenario 3, if UE does not receive SCell activation command at the end of FMW, the same interruption of deactivated SCell measurement interruption can be applied, provided that the measCycleSCell equal or longer than 640ms.

- Conclusion

Based on the discussion above, the following proposals and observations are concluded.

Proposal 1: UE is allowed to skip the OD-SSB measurement within FMW if the OD-SSB occasion in FMW collides with MG occasion, and this shall be considered in the ‘Kp’ factor of Tidentify when define the FMW length. ‘Kp’ shall be updated as above.

Proposal 2: Both ok with Option 1 and Option 2. If Option 1 is adopted, then the starting time and length of FMW should be re-defined.

Proposal 3: For Alt Time-C2 case, the measurement requirement in FMW, the SCell activation delay requirement and the  L1 measurement in the active SCell after SCell activation shall be defined based on OD-SSB periodicity.

Proposal 4: For Scenario 3 that OD-SSB and AO-SSB are within different frequencies, when OD-SSB is activated, UE use servingCellMO-OD to measure serving cell. Reuse the agreement of Case 2 about when to measure the OD-SSB with fast mode.

Proposal 5: When OD-SSB is deactivated, UE don’t perform the neighbour cell measurement on OD-SSB carrier.

Proposal 6: When OD-SSB is activated, UE perform neighbour cell measurement on OD-SSB carrier with intra-frequency measurement requirement, and perform neighbour cell measurement on AO-SSB carrier with inter-frequency measurement without gap requirement.

Proposal 7: The current known SCell activation delay methodology can be reused in Case 2 Alt Time-C2 with the following combination.

- When the SSB measured for known cell condition is AO-SSB or OD-SSB, and the SSB used in SCell activation procedure is AO-SSB or OD-SSB.

Proposal 8: The current known SCell activation delay methodology can be reused in Scenario 3 with the following combination.

- When the SSB measured for known cell condition is AO-SSB or OD-SSB, and the SSB used in SCell activation procedure is AO-SSB or OD-SSB.

Proposal 9: Apply the ‘OD-SSB activation’ timeline and the UE capability for ‘MAC CE based OD-SSB transmission parameter updates’, that the additional processing time is 5ms or 2ms based on UE capability reporting.

Proposal 10: Extend the range of the UE capability ‘Additional processing time for OD-SSB activation’ to ‘Additional processing time for OD-SSB activation and MAC-CE based OD-SSB transmission parameter updates’.

Proposal 11: No UE additional processing time for OD-SSB deactivation.

Proposal 12: For Case 1 Scenario 2 and Scenario 3, new interruption is allowed due to OD-SSB activation/deactivation. The starting time of interruption is only allowed within the UE processing delay T_processing (including HARQ, MAC-CE processing and additional processing time). The interruption duration shall be 1 slot for FR1.

- Proposal 13: For Case 1 Scenario 2, new interruption is not allowed due to OD-SSB activation/deactivation.

- Proposal 14: For both Scenario 2 and Scenario 3, if UE does not receive SCell activation command at the end of FMW, the same interruption of deactivated SCell measurement interruption can be applied, provided that the measCycleSCell equal or longer than 640ms.

- Reference

- R4-2508277, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson